3GPP TSG-SA5 Meeting #129-e	S5-201416

- February 24 – 28, 2020																				     s5-2abcde

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0257 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add IOCs to support SON functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SON_5G |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The NRM to support SON functions as defined in TS 28.313 is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following changes: - SON functions NRM fragment - IOC MobilityRobustnessOptimization - IOC RachOptimization - IOC PCIConfiguration - SON attributes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5G SON functions are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2, 4.3, 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	Void.

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	Void.

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	Void.

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GR NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV) Release 2; Management and Orchestration; Report on NFV Information Model".

[41]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[42]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[43]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire".

[44]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[45]	IETF RFC 8528: "YANG Schema Mount".

[46]	IETF RFC 8340: "YANG Tree Diagrams".

[47]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[48]	3GPP TS 38.463: "NG-RAN; E1 application protocol (E1AP)".

[x]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

Next Modified Section

## 4.2	Class diagram

### 4.2.1	Class diagram for gNB and en-gNB

#### 4.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this gNB and en-gNB. For the UML semantics, see 3GPP TS 32.156 [43]. Subsequent clauses provide more detailed specification of various aspects of these classes.

The model fragments are for management representation of gNB and en-gNB for all NG-RAN deployment scenario as listed below.

-	Non-split NG-RAN deployment senario, represents the gNB defined in TS 38.401[4].

-	2-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU and gNB-DU defined in TS 38.401[4] clause 6.1.1.

-	3-split NG-RAN deployment scenario, represents the gNB consist of gNB-CU-CP, gNB-CU-UP and gNB-DU defined in TS 38.401[4] clause 6.1.2.

Figure 4.2.1.1-1: NRM for all deployment scenarios

Figure 4.2.1.1-2: NRM for EPs for all deployment scenarios

Figure 4.2.1.1-3: NRM for <<IOC>>NRSectorCarrier and <<IOC>>BWP for all deployment scenarios

Figure 4.2.1.1-4: Cell Relation view for all deployment scenarios

NOTE 1:	The above NRM fragment uses SubNetwork to hold both NR and LTE external entities and frequencies.

Figure 4.2.1.1-5: Cell Relation view for all deployment scenarios

NOTE 2:	The above NRM fragment uses NRNetwork to hold NR external entities and frequency and using EUtraNetwork to hold LTE external entities and frequency. The NRNetwork and EUtraNetwork are subclasses of SubNetwork (defined in TS 28.622 [30]) with no additional attributes. The reason using NRNetwork and EUtraNetwork is for a clean separation of NR external entities and frequency and LTE external entities and frequency.

Figure 4.2.1.1-x: SON functions NRM fragment

NOTE: in the case where an IOC (e.g. RachOptimizationFunction) contained under multiple level of containment tree, IOC at the lowest level (e.g. NRCellCU) overrides the IOCs at higher level(s) of the same containment tree.

#### 4.2.1.2	Inheritance

Figure 4.2.1.2-1: Inheritance Hierarchy

Next Modified Section

## 4.3	Class definitions

### 4.3.x	RachOptimizationFunction

#### 4.3.x.1	Definition

This IOC contains attributes to support the SON function of RACH optimization (See clause 7.1.1 in TS 28.313 [x]).

#### 4.3.x.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ueAccProbilityDistPerSSB | CM | M | M | - | M |
| ueAccProbilityDistPerCSI | CM | M | M | - | M |
| ueAccDelayProbilityDistPerSSB | CM | M | M | - | M |
| ueAccDelayProbilityDistPerCSI | CM | M | M | - | M |
| contentiousRACHAttemptsPerSSB | CM | M | M | - | M |
| contentiousRACHAttemptsPerCSI | CM | M | M | - | M |
| rachOptimizationControl | CM | M | M | - | M |

#### 4.3.x.3	Attribute constraints

| Name | Definition |
| --- | --- |
| ueAccProbilityDistPerSSB CM Support Qualifier | RACH Optimization is supported |
| ueAccProbilityDistPerCSI CM Support Qualifier | RACH Optimization is supported |
| ueAccDelayProbilityDistPerSSB CM Support Qualifier | RACH Optimization is supported |
| ueAccDelayProbilityDistPerCSI CM Support Qualifier | RACH Optimization is supported |
| contentiousRACHAttemptsPerSSB CM Support Qualifier | RACH Optimization is supported |
| contentiousRACHAttemptsPerCSI CM Support Qualifier | RACH Optimization is supported |
| rachOptimizationControl CM Support Qualifier | RACH Optimization is supported. |

#### 4.3.x.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.y	MobilityRobustnessOptimizationFunction

#### 4.3.y.1	Definition

This IOC contains attributes to support the SON function of MRO (See clause 7.1.2 in TS 28.313 [x]).

#### 4.3.y.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| totalHoFailureRate | CM | M | M | - | M |
| intraRATHoFailureRate | CM | M | M | - | M |
| intraRATTooEarlyHoFailureRate | CM | M | M | - | M |
| intraRATTooLateHoFailureRate | CM | M | M | - | M |
| intraRATHoToWrongCellFailureRate | CM | M | M | - | M |
| interRATHoFailureRate | CM | M | M | - | M |
| interRATTooEarlyHoFailureRate | CM | M | M | - | M |
| interRATTooLateHoFailureRate | CM | M | M | - | M |
| interRATUnnecessaryHoFailureRate | CM | M | M | - | M |
| interRATPingPongHoFailureRate | CM | M | M | - | M |
| mroControl | CM | M | M | - | M |

#### 4.3.y.3	Attribute constraints

| Name | Definition |
| --- | --- |
| totalHoFailureRate CM Support Qualifier | MRO is supported |
| intraRATHoFailureRate CM Support Qualifier | MRO is supported |
| intraRATTooEarlyHoFailureRate CM Support Qualifier | MRO is supported |
| intraRATTooLateHoFailureRate CM Support Qualifier | MRO is supported |
| intraRATHoToWrongCellFailureRate CM Support Qualifier | MRO is supported |
| interRATHoFailureRate CM Support Qualifier | MRO is supported |
| interRATTooEarlyHoFailureRate CM Support Qualifier | MRO is supported |
| interRATTooLateHoFailureRate CM Support Qualifier | MRO is supported |
| interRATUnnecessaryHoFailureRate CM Support Qualifier | MRO is supported |
| interRATPingPongHoFailureRate CM Support Qualifier | MRO is supported |
| mroControl CM Support Qualifier | MRO is supported |

#### 4.3.y.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions

4.3.z	DSONPciCIConfigurationFunction

#### 4.3.z.1	Definition

This IOC contains attributes to support the D-SON function of PCI configuration (See clause 7.1.3 in TS 28.313 [x]).

#### 4.3.z.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dSONPciConfigurationControl | CM | M | M | - | M |
| pciList | CM | M | M | - | M |

#### 4.3.z.3	Attribute constraints

| Name | Definition |
| --- | --- |
| dSONPpciConfigurationControl CM Support Qualifier | D-SON PCI configuration is supported |
| pciList CM Support Qualifier | D-SON PCI configuration is supported |

#### 4.3.z.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.w	CSONPciConfigurationFunction

#### 4.3.w.1	Definition

This IOC contains attributes to support the C-SON function of PCI configuration (See clause 7.1.3 in TS 28.313 [x]).

#### 4.3.w.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cSONPciConfigurationControl | CM | M | M | - | M |
| nrCellpciValue | CM | M | M | - | M |
| csonPciList | CM | M | M | - | M |

#### 4.3.w.3	Attribute constraints

| Name | Definition |
| --- | --- |
| cSONPciConfigurationControl CM Support Qualifier | C-SON PCI configuration is supported |
| nrCellpciValue CM Support Qualifier | C-SON PCI configuration is supported |
| csonPciList CM Support Qualifier | C-SON PCI configuration is supported |

#### 4.3.w.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.v	DSONPciConfigControlFunction

#### 4.3.v.1	Definition

This <<IOC>> DSONPciConfigControlFunction represents the control information of D-SON PCI configuration.

#### 4.3.v.2	Attributes

The DSONPciConfigControlFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dsonPciConfigurationSwitch | CM | M | M | - | M |

#### 4.3.v.3	Attribute constraints

| Name | Definition |
| --- | --- |
| dsonPciConfigurationSwitch CM Support Qualifier | D-SON PCI configuration is supported |

.

#### 4.3.v.4	Notifications

The common notifications defined in subclause X.1.5 are valid for this IOC, without exceptions or additions.

Next Modified Section

## 5.4	Attribute definitions

### 5.4.1	Attribute properties

The following table defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

| Attribute Name |  | Documentation and Allowed Values |  | Properties |
| --- | --- | --- | --- | --- |
| aMFIdentifier |  | The AMFI is constructed from an AMF Region ID, an AMF Set ID and an AMF Pointer. The AMF Region ID identifies the region, the AMF Set ID uniquely identifies the AMF Set within the AMF Region, and the AMF Pointer uniquely identifies the AMF within the AMF Set. (Ref. 3GPP TS 23.003 [13]) |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| aMFSetId |  | It represents the AMF Set ID, which is uniquely identifies the AMF Set within the AMF Region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| aMFSetMemberList |  | It is the list of DNs of AMFFunction instances of the AMFSet. allowedValues: N/A |  | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| aMFRegionId |  | It represents the AMF Region ID, which identifies the region. allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 [13]. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| localAddress | This parameter specifies the localAddress including IP address and VLAN ID used for initialization of the underlying transport. First string is IP address, IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). Second string is VLAN Id (See IEEE 802.1Q [39]). |  | type: String multiplicity: 2 isOrdered: True isUnique: N/A defaultValue: None isNullable: False |  |
| remoteAddress | Remote address including IP address used for initialization of the underlying transport. IP address can be an IPv4 address (See RFC 791 [37]) or an IPv6 address (See RFC 2373 [38]). |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| nfProfileList |  | It is a set of NFProfile(s) to be registered in the NRF instance. NFProfile is defined in 3GPP TS 29.510 [23]. |  | type: <<dataType>> multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nSIIdList |  | It is a set of NSI Id. The NSI ID is defined in subclause 6.1.6.2.8 of 3GPP TS 29.531 [24]. |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sNSSAIList |  | See subclause 4.4.1. |  |  |
| sBIFQDN |  | It is used to indicate the FQDN of the registered NF instance in service-based interface, for example, NF instance FQDN structure is: nftype<nfnum>.slicetype<sliceid>.mnc<MNC>.mcc<MCC>.3gppnetwork.org |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| sBIServiceList |  | It is used to indicate the all supported NF services registered on service-based interface. |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nRTACList |  | It is the list of Tracking Area Codes (either legacy TAC or extended TAC). allowedValues: Legacy TAC and Extended TAC are defined in clause 9.3.3.10 of TS 38.413 [5]. |  | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| supportedBMOList |  | It is used to indicate the list of supported BMOs (Bridge Managed Objects) required for integration with TSN system. |  | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| managedNFProfile |  | This parameter defines profile for managed NF (See TS 23.501 [22]). allowedValues: N/A |  | type: ManagedNFProfile multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nfInstanceID |  | This parameter defines unique identity of the NF Instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [44] allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| nfType |  | This parameter defines type of Network Function allowedValues: See TS 23.501[22] for NF types |  | type: ENUM multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fqdn |  | This parameter defines FQDN of the Network Function (See TS 23.003 [5]) allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| ipAddress |  | This parameter defines IP Address of the Network Function. It can be IPv4 address (See RFC 791 [24]) or IPv6 address (See RFC 2373 [25]). allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| authzInfo |  | This parameter defines NF Specific Service authorization information. It shall include the NF type (s) and NF realms/origins allowed to consume NF Service(s) of NF Service Producer (See TS 23.501[22]). allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: True |
| locality |  | The parameter defines information about the location of the NF instance (e.g. geographic location, data center) defined by operator (See TS 29.510[23]). allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: True |
| capacity |  | This parameter defines static capacity information in the range of 0-65535, expressed as a weight relative to other NF instances of the same type; if capacity is also present in the nfServiceList parameters, those will have precedence over this value (See TS 29.510[23]) allowedValues: 0-65535 |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| nFInfo |  | This parameter includes NF specific data in Managed NF profile allowedValues: N/A |  | type: NFInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| hostAddr |  | This parameter defines host address of a NF allowedValues: N/A |  | type: HostAddr multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| priority |  | This parameter defines Priority (relative to other NFs of the same type) in the range of 0-65535, to be used for NF selection; lower values indicate a higher priority. If priority is also present in the nfServiceList parameters, those will have precedence over this value (See TS 29.510[23]). allowedValues: 0-65535 |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| supportedDataSetIds |  | This parameter defines list of supported data sets in the UDR instance (See TS 29.510[23]). allowedValues: "SUBSCRIPTION", "POLICY", EXPOSURE", "APPLICATION" |  | type: ENUM multiplicity: 1..* isOrdered: N/A isUnique: False defaultValue: None isNullable: False |
| nFSrvGroupId |  | This parameter defines identity of the group that is served by the NF instance (See TS 29.510[23]). allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| smfServingAreas |  | This parameter defines the SMF service area(s) the UPF can serve (See TS 29.510[23]). allowedValues: N/A |  | type: String multiplicity: 1..* isOrdered: F isUnique: True defaultValue: None isNullable: False |
| isRemoveAllowed |  | This indicates if the subject NRCellRelation can be removed (deleted) or not. If YES, the subject NRCellRelation instance can be removed (deleted). If NO, the subject NRCellRelation instance shall not be removed (deleted) by any entity but an MnS consumer. allowedValues: YES, NO |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isHOAllowed |  | This indicates if HO is allowed or prohibited. If YES, handover is allowed from source cell to target cell. The source cell is identified by the name-containing NRCellCU of the NRCellRelation that contains the isHOAllowed. The target cell is referenced by the NRCellRelation that contains this isHOAllowed. If NO, handover shall not be allowed. allowedValues: YES, NO |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| x2BlackList |  | This is a list of DNs of NRCellCU and ExternalNRCellCU. If the target node DN is a member of the source node’s NRCellCU.x2BlackList, the source node is: 1) Prohibited from sending X2 connection request to target node; 2) Forced to tear down established X2 connection to target node 3) Not allowed to accept incoming X2 connection request from target node. The same DN may appear here and in NRCellCU.x2WhiteList. In such case, the DN in x2WhiteList shall be treated as if it is absent. |  | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| xnBlackList |  | This is a list of DNs of NRCellCU and ExternalNRCellCU. If the target node DN is a member of the source node’s NRCellCU.xnBlackList, the source node is: 1) Prohibited from sending Xn connection request to target node; 2) Forced to tear down established Xn connection to target node 3) Not allowed to accept incoming Xn connection request from target node. The same DN may appear here and in NRCellCU.xnWhiteList. In such case, the DN in xnWhiteList shall be treated as if it is absent. |  | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| x2WhiteList |  | This is a list of DNs of NRCellCU and ExternalNRCellCU. If the target node DN is a member of the source node’s NRCellCU.x2WhiteList, the source node: - is allowed to request the establishment of X2 connection with the target node; - is not allowed to initiate the tear down of established X2 connection to target node The same DN may appear here and in NRCellCU.x2BlackList. In such case, the DN here shall be treated as if it is absent. |  | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| xnWhiteList |  | This is a list of DNs of NRCellCU and ExternalNRCellCU. If the target node DN is a member of the source node’s NRCellCU.xnWhiteList, the source node: - is allowed to request the establishment of Xn connection with the target node; - is not allowed to initiate the tear down of established Xn connection to target node The same DN may appear here and in NRCellCU.xnBlackList. In such case, the DN here shall be treated as if it is absent. |  | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| x2XnHOBlackList |  | This is a list of DNs of any number and combination of cells represented by the following IoCs: NRCellCU ExternalNRCellCU. ExternalEUtranCellTDD ExternalEUtranCellFDD EUtranCellTDD EUtranCellFDD For all the entries in NRCellCU.x2XnHOBlackList, the subject NRCellCU is prohibited to use the X2 or Xn interface for HOs even if an X2 or Xn interface exists to the target cell. |  | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| groupId |  | This parameter identiies a list of target NF services on which the same communication model is applied to. allowedValues: N/A |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |
| commModelType |  | This parameter defines communication model used by a NF to interact with NF service(s) (See TS 23.501 [2]). allowedValues:”DIRECT_COMMUNICATION_WO_NRF”, “DIRECT_COMMUNICATION_WITH_NRF”, “INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY”, “INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY” |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| targetNFServiceList |  | This parameter lists target NF services sharing same communication model and configuration. allowedValues: N/A |  | type: DN multiplicity: 1..* isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| commModelConfiguration |  | This parameter defines configuration parameters for specific communication model for a group of NF Services. allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| supportedFuncList |  | This parameter lists functionalities supported by a SCP. Refer to TS 23.501 [2]. |  | type: SupportedFunction multiplicity: 1..* isOrdered: N/A isUnique: False defaultValue: None isNullable: False |
| address |  | This parameter defines address of a SCP instance, it can be IP address (either IPv4 address (See RFC 791 [24]) or IPv6 address (See RFC 2373 [25])) or FQDN (See TS 23.003 [5]). |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| function |  | This parameter defines name of a functionality supported by a SCP. |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| policy |  | This parameter defines configuration policies of a functionality supported by a SCP. |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| capabilityList |  | This parameter lists capabilities supported by a NEF. Refer to TS 23.501 [2]. allowedValues: N/A |  | type: String multiplicity: 1..* isOrdered: N/A isUnique: False defaultValue: None isNullable: False |
| isINEF |  | This parameter defines if the NEF is an Intermediate NEF. allowedValues: TRUE, FALSE |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| isCAPIFSup |  | This parameter defines if the NEF support Common API Framework. allowedValues: TRUE, FALSE |  | type: Boolean multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| sEPPType |  | This parameter defines the type of a SEPP entity. Refer to TS 33.501 [52]. allowedValues: “CSEPP”, “PSEPP” |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: False defaultValue: None isNullable: False |
| sEPPId |  | This parameter is identifier of a SEPP, it is unique inside a PLMN. allowedValues: N/A |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| remotePlmnId |  | This parameter defines PLMNId of the remote SEPP. allowedValues: N/A |  | Type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| remoteSeppAddress |  | This parameter defines address of the remote SEPP. It can be IP address (either IPv4 address (See RFC 791 [24]) or IPv6 address (See RFC 2373 [25])) or FQDN(See TS 23.003 [5]). allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| remoteSeppId |  | This parameter defines identifier of the remote SEPP. it is unique inside a PLMN. allowedValues: N/A |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| n32cParas |  | This attribute is used to configure parameters to establish security link between two SEPPs. allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| n32fPolicy |  | This attribute is used to configure policies to protect the messages exchanged between SEPPs. allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: F isUnique: N/A defaultValue: None isNullable: False |
| withIPX |  | This attribute defines if there’s an IPX interconnected between two SEPPs. allowedValues: TRUE, FALSE |  | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| totalHoFailureRate |  | This indicates the target of the number of handover failure events divided by the total number of handover events, together with its targetWeight. This target is suitable for MRO or LBO. allowedValues: hoFailureRate: Integer 1..100 (percentage). targetWeight: Integer 1..N. The higher the number the higher the weight. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intraRATHoFailureRate |  | This indicates the assigned target of the number of handover failure events divided by the total number of intra-RAT handover events, together with its targetWeight. This target is suitable for MRO or LBO. allowedValues: hoFailureRate: Integer 1..100 (percentage). targetWeight: Integer 1..N. The higher the number the higher the weight. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intraRATTooEarlyHoFailureRate |  | This indicates the assigned target of the number of too early handover failure events divided by the total number of intra-RAT handover events, together with its targetWeight. This target is suitable for MRO or LBO. allowedValues: hoFailureRate: Integer 1..100 (percentage). targetWeight: Integer 1..N. The higher the number the higher the weight. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intraRATTooLateHoFailureRate |  | This indicates the assigned target of the number of too late handover failure events divided by the total number of intra-RAT handover events, together with its targetWeight. This target is suitable for MRO or LBO. allowedValues: hoFailureRate: Integer 1..100 (percentage). targetWeight: Integer 1..N. The higher the number the higher the weight. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| intraRATHoToWrongCellFailureRate |  | This indicates the assigned target of the number of handover to wrong cell failure events divided by the total number of intra-RAT handover events, together with its targetWeight. This target is suitable for MRO or LBO. allowedValues: hoFailureRate: Integer 1..100 (percentage). targetWeight: Integer 1..N. The higher the number the higher the weight. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| interRATHoFailureRate |  | This indicates the assigned target of the number of handover failure events divided by the total number of inter-RAT handover events, together with its targetWeight. This target is suitable for MRO or LBO. allowedValues: hoFailureRate: Integer 1..100 (percentage). targetWeight: Integer 1..N. The higher the number the higher the weight. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| interRATTooEarlyHoFailureRate |  | This indicates the assigned target of the number of too early handover failure events divided by the total number of inter-RAT handover events, together with its targetWeight. This target is suitable for MRO or LBO. allowedValues: hoFailureRate: Integer 1..100 (percentage). targetWeight: Integer 1..N. The higher the number the higher the weight. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| interRATTooLateHoFailureRate |  | This indicates the assigned target of the number of too late handover failure events divided by the total number of inter-RAT handover events, together with its targetWeight. This target is suitable for MRO or LBO. allowedValues: hoFailureRate: Integer 1..100 (percentage). targetWeight: Integer 1..N. The higher the number the higher the weight. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| interRATUnnecessaryHoFailureRate |  | This indicates the assigned target of the number of unnecessary handover failure events divided by the total number of inter-RAT handover events, together with its targetWeight. This target is suitable for MRO or LBO. allowedValues: hoFailureRate: Integer 1..100 (percentage). targetWeight: Integer 1..N. The higher the number the higher the weight. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| interRATPingPongHoFailureRate |  | This indicates the assigned target of the number of ping-pong handover failure events divided by the total number of inter-RAT handover events, together with its targetWeight. This target is suitable for MRO or LBO. allowedValues: hoFailureRate: Integer 1..100 (percentage). targetWeight: Integer 1..N. The higher the number the higher the weight. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| mroControl |  | This attribute determines whether the MRO function is enabled or disabled. allowedValues: On, Off |  | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| pciList |  | This holds a list of physical cell identities that can be assigned to the NR cells. This attribute shall be supported if D-SON PCI configuration function is supported. See subclause 8.2.3, 8.3.1 in TS 28.313 [x]. allowedValues: See TS 38.211 [32] subclause 7.4.2 for legal values of pci. The number of pci in the list is 0 to 1007. |  | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dSONPciConfigurationControl |  | This attribute determines whether the D-SON PCI configuration function at NR cell DU is enabled or disabled. allowedValues: On, Off |  | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| nrCellpciValue |  | This holds the physical cell identities that can be assigned to a NR cell. This attribute shall be supported if C-SON PCI configuration function is supported. See subclause 8.2.3, 8.3.1 in TS 28.313 [x]. allowedValues: See TS 38.211 [32] subclause 7.4.2 for legal values of pci. The number of pci in the list is 0 to 1007. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cSONPciConfigurationControl |  | This attribute determines whether the C-SON PCI configuration function is enabled or disabled. allowedValues: On, Off |  | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dsonPciConfigurationSwitch |  | This attribute determines whether the D-SON PCI configuration Function at NR cell CU is activated or deactivated. allowedValues: On, Off |  | type: boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| csonPciList |  | This holds a list of physical cell identities that can be assigned to the pci attribute by gNB. The assignment algorithm is not specified. This attribute shall be supported if and only if the Domain-Centralized is supported. See TS 28.313, ref [X] subclause 7.1.3. allowedValues: See TS 38.211 [32] subclause 7.4.2.1 for legal values of pci. The number of pci in the list is 1 to 100X. |  | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ueAccProbilityDistPerSSB |  | This is a list of target Access Probability (APn) for the RACH optimization function. Each instance APn of the list is the probability that the UE gets access on the RACH channel per SSB within n number of preambles sent over an unspecified sampling period. This target is suitable for RACH optimization. allowedValues: Each element of the list, APn, is a pair (a, n) where a is the targetProbability (in %) and n is the number of preambles sent. The legal values for a are 25, 50, 75, 90. The legal values for n are 1 to 200. The number of elements specified is 4. The number of elements supported is vendor specific. The choice of supported values for a and n is vendor-specific. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| ueAccProbilityDistPerCSI |  | This is a list of target Access Probability (APn) for the RACH optimization function. Each instance APn of the list is the probability that the UE gets access on the RACH channel per CSI within n number of preambles sent over an unspecified sampling period. This target is suitable for RACH optimization. allowedValues: Each element of the list, APn, is a pair (a, n) where a is the targetProbability (in %) and n is the number of preambles sent. The legal values for a are 25, 50, 75, 90. The legal values for n are 1 to 200. The number of elements specified is 4. The number of elements supported is vendor specific. The choice of supported values for a and n is vendor-specific. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| ueAccDelayProbilityDistPerSSB |  | This is a list of target Access Delay probability (ADP) for the RACH optimization function. Each instance ADP of the list is the target time before the UE gets access on the RACH channel per SSB, for the P percent of the successful RACH Access attempts with lowest access delay, over an unspecified sampling period. This target is suitable for RACH optimization. allowedValues: Each element of the list, ADp, is a pair (p, d) where p is the targetProbability (in %) and d is the access delay (in milliseconds). The legal values for p are 25, 50, 75, 90. The legal values for d are 10 to 560. The number of elements specified is 4. The number of elements supported is vendor specific. The choice of supported values for a and b is vendor-specific. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| ueAccDelayProbilityDistPerCSI |  | This is a list of target Access Delay probability (ADP) for the RACH optimization function. Each instance ADP of the list is the target time before the UE gets access on the RACH channel per CSI, for the P percent of the successful RACH Access attempts with lowest access delay, over an unspecified sampling period. This target is suitable for RACH optimization. allowedValues: Each element of the list, ADp, is a pair (p, d) where p is the targetProbability (in %) and d is the access delay (in milliseconds). The legal values for p are 25, 50, 75, 90. The legal values for d are 10 to 560. The number of elements specified is 4. The number of elements supported is vendor specific. The choice of supported values for a and b is vendor-specific. |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| contentiousRACHAttemptsPerSSB |  | This indicates the assigned target of the number of RACH attempts with contention detected divided by the total number of RACH attempts for each SSB. This target is suitable for RACH optimization. allowedValues: Integer 1..100 |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| contentiousRACHAttemptsPerCSI |  | This indicates the assigned target of the number of RACH attempts with contention detected divided by the total number of RACH attempts for each CSI. This target is suitable for RACH optimization. allowedValues: Integer 1..100 |  | type: <<data type>> multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| rachOptimizationControl |  | This attribute determines whether the RACH Optimization function is enabled or disabled. allowedValues: On, Off |  | type: <<enumeration>> multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |

End of Modified Sections