3GPP TSG-SA5 Meeting #129e 	S5-201246

# e-meeting, 24 February – 4 March 2020

# Source:	Ericsson

# Title:	Adding charging scenarios principles

# Document for:	Approval

Agenda Item:	7.4.3

# 1	Decision/action requested

Adding the following changes to TS 28.201

# 2	References

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI).

[3]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[4]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

# 3	Rationale

Addition of basic principles for network slice performance and analytics charging scenarios.

# 4	Detailed proposal

First change

## 5.2 	Network slice performance and analytics charging scenarios

### 5.2.1	Basic principles

#### 5.2.1.1	General

Converged charging may be performed by the CSIF interacting with CHF using Nchf specified in TS 32.290 [2] and TS 32.291 [3]. In order to provide the data required for the management activities outlined in TS 32.240 [1] (Credit-Control, accounting, billing, statistics etc.), the CSIF shall be able to perform converged charging for charging data related to network slice performance and analytics.

The CSIF shall be able to perform convergent charging by interacting with CHF, for charging data related to network slice performance and analytics. The Charging Data Request and Charging Data Response are exchanged between the CSIF and the CHF, based on PEC scenarios specified in TS 32.290 [2]. The Charging Data Request is issued by the CSIF towards the CHF when certain conditions (chargeable events) are met.

The charging information collected per Single Network Slice Selection Assistance Information (S-NSSAI) the network slice instance belongs to. The unit usage reporting level can be either per rating group or per combination of the rating group and service id.

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following sub-clauses.

A detailed formal description of the converged charging parameters defined in the present document is found in TS 32.291 [3], and the CDR parameters defined in the present document is found in TS 32.298 [4].

End of changes