3GPP TSG-SA5 Meeting #133e 	S5-205148

# e-meeting 12th - 21st October 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0272 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add handover triggering measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2020/10/01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Obtaining the channel quality during a handover can help to optimize the handover success rate, improve the continuity of network coverage and ensure user experience. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add handover triggering measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement of handover-related indicators is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.X(new), 5.1.1.6.X.1(new), 5.1.1.6.X.2(new), 5.1.1.6.X.3(new), 5.1.1.6.X.4(new), A.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.1.1.6.X	Handover triggering measurements

5.1.1.6.X.1	RSRP quality distribution of the serving cell when HO is triggered

a)	This measurement provides the distribution RSRP quality of the serving cell reported in the UE measurement reports that triggered HO when measurement quantity RSRP is configured.

b)	DER (n=1)

c)	For each measurement sample, the bin corresponding to the value of the quality of the serving (source) cell RSRP in the UE measurement report causing HO on the NRCellRelation by the UE is incremented by one.

d)	A set of integers, each representing the (integer) number of samples with a RSRP quality of UE measurement report causing HO in the range represented by that bin.

e)	MM.HoServingCellRSRP.Bin

f)	NRCellRelation

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurement is for performance assurance.

5.1.1.6.X.2	RSRP quality distribution of the neighboring cell when HO is triggered

a)	This measurement provides the distribution RSRP quality of the neighbor cell reported in the UE measurement reports that triggered HO when measurement quantity RSRP is configured.

b)	DER (n=1)

c)	For each measurement sample, the bin corresponding to the value of the quality of the neighbor (target) cell RSRP in the UE measurement report causing HO on the NRCellRelation by the UE is incremented by one.

d)	A set of integers, each representing the (integer) number of samples with a RSRP quality of UE measurement report causing HO in the range represented by that bin.

e)	MM.HoNeighborCellRSRP.Bin

f)	NRCellRelation

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurement is for performance assurance.

5.1.1.6.X.3	RSRQ quality distribution of the serving cell when HO is triggered

a)	This measurement provides the distribution RSRQ quality of the serving cell reported in the UE measurement reports that triggered HO when measurement quantity RSRP is configured.

b)	DER (n=1)

c)	For each measurement sample, the bin corresponding to the value of the quality of the serving (source) cell RSRQ in the UE measurement report causing HO on the NRCellRelation by the UE is incremented by one.

d)	A set of integers, each representing the (integer) number of samples with a RSRP quality of UE measurement report causing HO in the range represented by that bin.

e)	MM.HoServingCellRSRQ.Bin

f)	NRCellRelation

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurement is for performance assurance.

5.1.1.6.X.4	RSRQ quality distribution of the neighboring cell when HO is triggered

a)	This measurement provides the distribution RSRQ quality of the neighbor cell reported in the UE measurement reports that triggered HO when measurement quantity RSRQ is configured.

b)	DER (n=1)

c)	For each measurement sample, the bin corresponding to the value of the quality of the neighbor (target) cell RSRQ in the UE measurement report causing HO on the NRCellRelation by the UE is incremented by one.

d)	A set of integers, each representing the (integer) number of samples with a RSRQ quality of UE measurement report causing HO in the range represented by that bin.

e)	MM.HoNeighborCellRSRQ.Bin

f)	NRCellRelation

g)	Valid for packet switched traffic.

h)	5GS.

i) 	One usage of this performance measurement is for performance assurance.

Next modified section

A.17	Monitoring of handovers

Mobility is one of the most significant feature of the mobile networks, and handover is one typical action of the mobility. The handover failure would cause service discontinuation, thus the performance of the handover has direct impact to the user experience.The handover procedure includes handover preparation, handover resource allocation and handover execution, and the performance related to handover needs to be monitored for each phase. The resources (e.g., PDU Session Resource) need to be prepared and allocated for a handover according to the QoS requirements for each  S-NSSAI.

The handover could occur intra-gNB and inter-gNB for 5G networks, and for inter-gNB case the handover could happen via NG or Xn interface.The handover could also occur between 5GS and EPS.

For the handover failures, the measurements with specific causes are required for trouble shooting.

The handover parameters setting could be specific for each NCR, and the handover performance could vary significantly for different NCRs, therefore the performance needs to be measured per NCR to support handover parameters optimization when necessary.

The information of the signal strength when handovers are triggered is useful for evaluating the customer experience (e.g. throughput) at the cell edge and during handover as well for network planning purposes (e.g., signal strength at the cell edge).

Signal strength of the serving and neighbouring cell reported by the UE in handover triggering measurement reports in NCRs which is also helpful for troubleshooting: for example, detect the "bad" configuration/neighbor in a particular direction by analyzing abnormal measurement report causing HO.

End of modifications