3GPP TSG-SA5 Meeting #133e 	S5-206052rev1

# e-meeting 12th – 21st October 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 007 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct Distributed PCI optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SON_5G |  |  |  |  |  | Date: |  |  | 2020-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Use Case and Management Services descriptions for D-SON PCI optimization do not confirm to RAN3 agreements. The PCI reconfiguration is controlled by the CU, as evident from TS 38.473 clause 9.2.1.10, Information Element NR PCI |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct Use Case and Management Services descriptions for D-SON PCI optimization |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with this specification and Stage 2 and Stage 3 in TS 28.541. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.4, 6.4.1.4.1, 6.4.1.4.2, 7.1.3.3.1, 8.2.31, 8.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | CR 0385, CR 0386 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-205141 |  |  |  |  |  |  |  |  |

First change

#### 6.1.1.4	PCI configuration and re-configuration

REQ-DPCI-CONFIG-FUN-1 producer of provisioning MnS should have a capability allowing an authorized consumer to set or update the list(s) of PCI value(s) for NR cell(s).

REQ-DPCI-CONFIG-FUN-2 producer of provisioning MnS should have a capability allowing an authorized consumer to enable or disable the PCI configuration function.

REQ-DPCI-CONFIG-FUN-3 producer of provisioning MnS should have a capability to notify the authorized consumer with the PCI value(s) being selected for NR cell(s).

REQ-DPCI-CONFIG-FUN-4 producer of provisioning MnS should have a capability to notify the authorized consumer about the detection or resolution of PCI collision or PCI confusion problems for NR cells.

REQ-DPCI-CONFIG-FUN-5 producer of provisioning MnS should have a capability allowing an authorized consumer to configure or re-configure the PCI list at the PCI configuration function.

Next change

##### 6.4.1.4.1	Initial PCI configuration

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically configure the initial PCI for a NR cell, from a list of PCIs. |  |
| Actors and Roles | D-SON management function to support initial PCI list configuration. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - The NR cells is not yet in operation. - No PCI list has been assigned to the NR cell. |  |
| Begins when | The D-SON management function decides to configure the PCI list for a NR cell. |  |
| Step 1 (M) | The D-SON management function requests the producer of provisioning MnS to configure the PCI list for a cell to the PCI configuration function. |  |
| Step 2 (M) | The D-SON management function requests the producer of provisioning MnS to enable the PCI configuration function at NR cell(s). |  |
| Step 3 (M) | When the cell is about to start operating, the PCI configuration function selects a PCI value from the list of PCI values and provides that to the cell. |  |
| Step 4 (M) | The producer of provisioning MnS notifies the consumer with the PCI value being assigned for the NR cell. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The PCI value of a NR cell has been configured. |  |
| Traceability | REQ-DPCI-CONFIG-FUN-1, REQ-DPCI-CONFIG-FUN-2, REQ-DPCI-CONFIG-FUN-3, REQ-DPCI-CONFIG-FUN-5 |  |

##### 6.4.1.4.2	PCI re-configuration

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically re-configure the PCI of an NR cell, due to PCI collision or PCI confusion problems. |  |
| Actors and Roles | D-SON management function to support PCI re-configuration. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS - The producer of fault supervision MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - A NR cell has been assigned a PCI value. - The PCI configuration function is in operation and enabled. |  |
| Begins when | The PCI configuration function has detected the PCI problem of a PCI collision or a PCI confusion for an NR cell. |  |
| Step 1 (M) | The D-SON PCI configuration function re-configures the PCI of that cell using one of the PCI values in the PCI list |  |
| Step 2 (M) | The PCI configuration function selects PCI value(s) from the PCI list. |  |
| Step 3 (M) | The producer of provisioning MnS notifies the consumer about the new PCI value of the NR cell. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The PCI collision or PCI confusion have been resolved. |  |
| Traceability | REQ-DPCI-CONFIG-FUN-3, REQ-DPCI-CONFIG-FUN-4, REQ-DPCI-CONFIG-FUN-5 |  |

Next change

#### 7.1.3.3	MnS Component Type C definition

##### 7.1.3.3.1	Notification information

The table below lists the notifications related to D-SON PCI configuration.

| Notifications | Description | Note |
| --- | --- | --- |
| PCI change notification | When the PCI configuration function changes the PCI of a cell, this change is notified using a notifyMOIAttributeValueChanges notification |  |

Next change

### 8.2.3	PCI configuration

#### 8.2.3.1	Initial PCI configuration

Figure 8.2.3.1-1 depicts a procedure that describes how D-SON management function can manage the PCI configuration (D-SON) function to assign the initial PCI values to an NR cell.

Figure 8.2.3.1-1: Initial PCI configuration procedure

1. The D-SON management function consumes the MnS of NF provisioning with modifyMOIAttributes operation to configure the PCI list for an NR cell.

1.a The producer of provisioning MnS sets the PCI list at the PCI configuration (D-SON) function. (NOTE)

2. The D-SON management function consumes the MnS of NF provisioning with modifyMOIAttributes operation to enable the PCI configuration function for NR cell(s) if it is not enabled.

2.a The MnS of provisioning enables the PCI configuration (D-SON) function (NOTE).

3. The PCI configuration (D-SON) function selects PCI value(s) from the PCI list.

4. The PCI configuration (D-SON) function reports the PCI value(s) being assigned to the MnS of NF provisioning.

5. The MnS of NF provisioning sends a notification notifyMOIAttributeValueChange to D-SON management function to indicate the PCI value(s) being assigned to NR cell(s).

NOTE: The interface between MnS of NF provisioning and PCI configuration (D-SON) function is not subject to standardization.

#### 8.2.3.2	PCI re-configuration

Figure 8.2.3.2-1 depicts a procedure that describes how the PCI configuration function, when detecting a PCI collision or confusion,  re-configures the PCI of the cell based on the PCI list, and notifies the D-SON management consumer..

D-SON management consumer

Producer of provisioning MnS

3. notifyMOIAttributeChange 
    to indicate new PCI value

PCI configuration function

2. Indicate that attribute is changed.

1. Detect and correct PCI collision or confusion

Figure 8.2.3.2-1: PCI re-configuration procedure

1. The PCI configuration (D-SON) function detects and corrects the PCI collision or PCI confusion problem for a NR cell (NOTE).

2. The PCI configuration (D-SON) function indicates the attribute change to the Producer of provisioning MnS..

3. The Producer of provisioning MnS sends a notification notifyMOIAttributeValueChange to the D-SON management function to indicate the new PCI value being assigned to NR cell.

NOTE: The interface between Producer of provisioning MnS and PCI configuration (D-SON) function is not subject to standardization.

End of changes