| 3GPP TS 28.100 V0.2.0 (2020-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Levels of autonomous network; (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Concepts and background	7

4.1	Autonomous network concept	7

4.2	Autonomous network level concept	7

4.3	Dimensions for evaluating autonomous network levels.	7

4.3.1 	Introduction	7

4.3.2	Scenarios	7

4.3.3	Management scope	8

4.3.4	Workflow	8

5	Framework approach for evaluating autonomous network levels	9

6	Use cases and requirements	10

6.1	Network and service planning scenarios	10

6.2	Network and service deployment scenarios	10

6.3	Network and service maintenance scenarios	10

6.4	Network and service optimization scenarios	11

6.4.1 	Autonomous network level for radio network coverage optimization	11

6.4.1.1	Introduction	11

Annex <A> (normative): <Normative annex for a Technical Specification>	11

Annex <X> (informative): Change history	12

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

Should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

Can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

Will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

Autonomous network is one of the important topics in 5G network. Complexity of 5G network increases with large number of devices and diversity of services. Different autonomous mechanisms are introduced by the industry to reduce the complexity of mobile network and service management. Moving from a manual operating network to a fully autonomous network requires a stepwise progression. For each step there are different capability and performance level of autonomy. Thus the concept of autonomous network levels and corresponding requirements are introduced to describe and evaluate each level in details.

1	Scope

The present document specifies the concepts, use cases and requirements for the levels of autonomous network.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Autonomous Network: telecommunication system (including management system and network) with autonomy capabilities which is able to be governed by itself with minimal to no human intervention.

Autonomous Network Level: describes the level of autonomy capabilities in the autonomous network.

Network and Service Planning: Editor's Note: network and service planning is FFS.

Network and Service Deployment: processes of allocation, installation, configuration, activation and verification of specific network and service.

Network and Service Maintenance: processes of monitoring, analysing and healing of the network and service issues.

Network and Service Optimization: processes of monitoring, analysing and optimizing the network and service performance.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<ABBREVIATION>	<Expansion>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AON	Autonomous Network

ANL	Autonomous Network Level

4	Concepts and background

Editor's note: this clause will contain the concept of autonomous network levels, dimensions for evaluating autonomous network levels.

4.1	Autonomous network concept

The network become complex due to large number of devices and diversity of services. Different autonomy mechanisms are introduced in the industry to reduce the complexity of mobile network management and control. The ultimate goal for autonomous network is to enable telecommunication system (including management system and network) within the autonomy mechanisms (e.g. AI/ML) to be governed by itself with minimal to no human intervention. Autonomous network can reduce the operating expenditure (OPEX) associated with the autonomous management and control of the complexity network and improve the service experience to enable various vertical industries (e.g. autonomous vehicle, smart city). Following are concepts related to autonomous network.

- Autonomous network level is used to describe the level of autonomy capabilities in the autonomous network.

- Self-Organization Network, Management data analytics, Intent driven management, closed loop SLS assurance are examples of enablers for autonomous network.

4.2	Autonomous network level concept

Different autonomy mechanisms in the telecommunication system may lead to different capabilities of autonomy and different operation efficiency on network management and control workflow, and indicates the level of autonomy of the network. The term Autonomous network level is used to describe the levels of autonomy capabilities in the autonomous network to improve the efficiency for network management and control. Participation of the human and telecommunication system in the network management and control workflow are different for each level and are important factors to evaluate the autonomous network levels. For each autonomous network level, some tasks can be performed by telecommunication system, some performed by human, and some performed by cooperation of human and telecommunication system. For example, in the highest autonomous network level, all tasks are performed by telecommunication system.

Editor's Note: the concept of autonomous network and autonomous network level need to be revised and coordinated with other SDOs.

4.3	Dimensions for evaluating autonomous network levels.

4.3.1 	Introduction

This clause describes the dimensions i.e. scenarios, management scope and workflow, which can be used for evaluating autonomous network level.

4.3.2	Scenarios

The autonomous network can be implemented for different scenarios, the complexity of autonomous network depends on the detailed scenarios where it is applied. Also it will be more challenging for the telecommunication system to achieve the autonomous network for full scenarios than for certain scenarios.  The autonomy capabilities of the scenarios will impact the autonomous network level for the whole autonomous network.

Following are scenario types categorized by network and service management process for autonomous network:

- Network and service planning

- Network and service deployment

- Network and service maintenance

- Network and service optimization

Editor's note: network and service planning is FFS.

4.3.3	Management scope

The autonomy can be implemented in different scopes, the complexity of autonomous network depends on its applicable scope. For example, it will be more challenging for the telecommunication system to achieve the autonomous network on cross domain network layer than domain network layer, because more autonomy mechanism needs to be introduced for the coordination between different domains. The autonomy capabilities of the management scope will impact the autonomous network level for the whole autonomous network.

Following are applicable scopes for autonomous network:

-	Autonomy in NE/NF layer, which means the autonomy mechanisms are executed in the NE/NF.

-	Autonomy in domain network layer, which means the autonomy mechanisms are executed in the MnF(s) in domain.

-	Autonomy in cross domain network layer, which means the autonomy mechanisms are executed in the MnF(s) in cross domain.

-	Autonomy in communication service layer, how to execute the autonomy mechanisms are executed in MnF(s) for communication service.

Editor's Note: autonomy in communication service layer is FFS.

Figure 4.Z.3-1: Autonomy for different management scope

4.3.4	Workflow

Workflow is used to describe the necessary steps to achieve certain management and control purposes. A workflow is composed of one or more management and control tasks. Each workflow task may be accomplished by human, or accomplished by telecommunication system with human assistance, or accomplished by telecommunication system without human intervention. The autonomy capabilities of the tasks in the workflow may impact the autonomous network level.

Following are the categorization of the tasks in a workflow:

-	Intent handling: The group of tasks which translate network or service intent from operator or customer into detailed operations and/or policies which may affect one or more of the following groups of tasks (i.e. awareness, analysis, decision, execution), also evaluate and feedback intent fulfilment information (e.g. the intent is satisfied or not) based on the detailed network and service information.

-	Awareness: The group of tasks which monitor network and service information (including network and service performance, network and service anomaly, network and service event, etc.).

-	Analysis: The group of tasks which analyse the obtained network and service information (e.g. network and service status, network and service issues and so on) or based on the historical network and service information to further predict the future change trend of the above network and service status, and make recommendation for decision.

-	Decision: The group of tasks which evaluate and decide the necessary operation for execution, e.g. network configuration or adjustment.

-	Execution: The group of tasks which execute the operations.

Editor's Note: the relationship between the above categorization of the tasks and the other standardized features including intent driven management and COSLA is FFS.

5	Framework approach for evaluating autonomous network levels

Editor's Note: the framework approach for evaluating autonomous network levels need to be revised and coordinated with other SDOs.

A framework approach for evaluating autonomous network levels is as following, which is used for evaluating the autonomy capability of telecom system.

Table X-1: Framework approach for evaluating autonomous network levels

| Network autonomy level |  | Task categories |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Execution | Awareness | Analysis | Decision | Intent handling |
| L0 | Manual operating network | Human | Human | Human | Human | Human |
| L1 | Assisted operating network | Human & Telecom system | Human & Telecom system | Human | Human | Human |
| L2 | Preliminary autonomous network | Telecom system | Human & Telecom system | Human & Telecom system | Human | Human |
| L3 | Intermediate autonomous network | Telecom system | Telecom system | Human & Telecom system | Human & Telecom system | Human |
| L4 | Advanced autonomous network | Telecom system | Telecom system | Telecom system | Telecom system | Human & Telecom system |
| L5 | Full autonomous network | Telecom system | Telecom system | Telecom system | Telecom system | Telecom system |
| Note 1: Human reviewed decision have the highest authority in each level if there is any confliction between human reviewed decision and telecom system generated decision. Note 2: The order of above five task categories does not reflect the workflow sequence. |  |  |  |  |  |  |

Level 0 manual operating network: No categorization of the tasks is accomplished by telecom system itself.

Level 1 assisted operating network: A part of the execution and awareness tasks are accomplished automatically by telecom system itself based on human defined rules. At this level, telecom system can assist human to improve the execution and awareness efficiency.

Level 2 preliminary autonomous network: All the execution tasks are accomplished automatically by telecom system itself. A part of the awareness and analysis tasks are accomplished automatically by telecom system itself based on human defined policies. At this level, telecom system can assist human to achieve the closed loop based on human defined policies.

Level 3 intermediate autonomous network: All the execution and awareness tasks are accomplished automatically by telecom system itself. A part of the analysis and decision tasks are accomplished automatically by telecom system itself based on human defined policies. At this level, the telecom system can achieve the closed loop automation based on the human defined closed loop automation policies.

Level 4 advanced autonomous network: All the execution, awareness, analysis and decision tasks are accomplished automatically by telecom system itself. And intent handling tasks can be partly accomplished automatically by telecom system itself based on human defined intent handling policies. At this level, telecom system can achieve the intent driven closed loop automation based on human defined intent handling policies, which means the telecom system can translate the intent to the detailed closed loop automation policies and translate the detailed network and service information to intent fulfilment information (e.g. the intent is satisfied or not) based on human defined intent handling policies.

Level 5 fully autonomous network: The entire network autonomy workflow is accomplished automatically by telecom system without human intervention. At this level, telecom system can achieve the whole network autonomy.

Note: 	Above framework approach for evaluating autonomous network levels is applicable for evaluating the autonomous network level from both management scope and scenario perspective. The overall autonomous network level of the whole telecom system is a comprehensive reflection of autonomous network level of the individual management scope and scenarios, which means in fully autonomous network level, the telecom system can achieve the whole network autonomy for all management scopes and scenarios.

6	Use cases and requirements

6.1	Network and service planning scenarios

Editor’s Note: this clause will contain the typical management use cases corresponding to telecom network and communication service planning scenarios and related management requirements for each autonomous levels. The management requirements include architecture requirements, enhancements of management and orchestration, important data and data collection policy and key management requirements on autonomous network related features.

6.2	Network and service deployment scenarios

Editor's Note: this clause will contain the typical management use cases corresponding to telecom network and communication service deployment scenarios and related management requirements for each autonomous levels. The management requirements include architecture requirements, enhancements of management and orchestration, important data and data collection policy and key management requirements on autonomous network related features.

6.3	Network and service maintenance scenarios

Editor's Note: this clause will contain the typical management use cases corresponding to telecom network and communication service maintenance scenarios and related management requirements for each autonomous levels. The management requirements include architecture requirements, enhancements of management and orchestration, important data and data collection policy and key management requirements on autonomous network related features.

6.4	Network and service optimization scenarios

Editor's Note: this clause will contain the typical management use cases corresponding to telecom network and communication service optimization scenarios and related management requirements for each autonomous levels. The management requirements include architecture requirements, enhancements of management and orchestration, important data and data collection policy and key management requirements on autonomous network related features.

6.4.1 Autonomous network level for radio network coverage optimization

Editor's Note: whether the following use case need to be generalized for common optimization scenarios is FFS.

6.4.1.1	Introduction

Radio networks are geographically distributed, and mobile user activity varies significantly in different places and at different times of day. To achieve the optimal coverage, a set of initial coverage configuration parameters (e.g. coverageShape, digitalTilt and digitalAzimuth) may not always meet the requirements. Therefore, the coverage configuration parameters need to be adjusted in a differentiated manner with the change of the radio network environment. It’s complex for the adjustment due to multiple factors needs to be considered, e.g. interference control, huge data and frequent traffic changes. So introducing the autonomous network level for radio network coverage optimization will benefit for operator to achieve the full autonomy goal step by step and have clear view on which typical issues can be addressed by utilizing autonomy mechanism in corresponding steps. The requirements for each autonomous level for radio network coverage optimization are different.

Editor's Note: The detailed workflow, classification of autonomous network level and corresponding requirements are FFS.

Annex <A> (normative):
<Normative annex for a Technical Specification>

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | SA5#132e | S5-204084 |  |  |  | Initial draft TS | 0.0.0 |
| 2020-08 | SA5#132e | S5-204555 S5-204086 S5-204087 |  |  |  | 1. pCR 28.100 Add skeleton 2. pCR 28.100 Add Scope 3. pCR 28.100 Add definitions and abbreviations | 0.1.0 |
| 2020-11 | SA5#134e | S5-206180 S5-206352 S5-206353 S5-206354 S5-206355 S5-206356 S5-206357 |  |  |  | 1. pCR 28.100 Update skeleton for clause 5 2. Add concept of autonomous network and autonomous network level 3. pCR TS 28.100 Add use case and requirements for coverage optimization 4. pCR 28.100 Add Introduction 5. pCR 28.100 Add definition for network and service management scenario types 6. pCR 28.100 Add dimensions for evaluating autonomous network levels 7. pCR 28.100 Add framework approach for evaluating autonomous network levels | 0.2.0 |