3GPP TSG SA WG5 Meeting 137-e		TDoc S5-213077rev1

- electronic meeting, online, 10 - 19 May 2021

- Source:	Huawei

- Title:	Potential solution for Key Issue #6

- Document for:	Approval

- Agenda Item:	6.5.1

# 1	Decision/action requested

The group is asked to discuss and approval the proposal.

# 2	References

[1]	3GPP TR 28.813: Study on new aspects of Energy Efficiency (EE) for 5G v0.5.0

# 3	Rationale

The ES cell activation/deactivation decision is typically based on the load information of the related cells and the energy saving policies (like the allowed ES time period, ES candidate cell relations) without special considering information such as service type of cell load, etc. Since the 5G system is expected to be able to provide support for a variety of different communication services such as eMBB services, URLLC services, MIoT services and V2X services, etc., the content of different communication services needs to be considered for cell activation/deactivation decision to reach a more efficient ES.

In this potential solution, 3GPP management system provides centralized ES for network functions according to service related information.

# 4	Detailed proposal

This document proposes the following changes in TR 28.813 [1].

1st Change

### 4.6.2	Potential solutions

#### 4.6.2.1	Potential solution #Y-1: Energy saving in 5G NR based on service related information

##### 4.6.2.1.1	Introduction

The ES cell activation/deactivation decision is typically based on the load information of the related cells and the energy saving policies (like the allowed ES time period, ES candidate cell relations) without considering information such as service type of cell traffic, or tenant information etc.

In this potential solution, 3GPP management system provides centralized ES for NG-RAN according to service related information.

##### 4.6.2.1.2	Description

This potential solution is based on the following information:

The service related information may include service characteristic information or tenant information of service.

The service characteristic information may include service type information, service name information, and service priority information.

- The service type information indicates service type, it can be decided by operator's policy, for example, one kind of service type may be eMBB, URLLC, mIoT, or V2X etc, or another kind of service type may be voice, video, industrial control, web browsing, or autonomous driving.

- The service name may be human-readable name according to operator's policy.

- The service priority information may be, for example, high priority, medium priority, or low priority.

The tenant information of service may include tenant type information, tenant name information, tenant priority information

- The tenant type may be, for example, Business to Consumer (B2C) tenant, Business to Business (B2B) tenant, Business to Household (B2H) tenant, Business to Business to Everything (B2B2X) tenant.

- The tenant name may be human-readable name according to operator's policy.

- The tenant priority information may be, for example, high priority, medium priority, or low priority.

The service related information can be obtained from UEs, 5GC NFs (such as UPFs or SMFs) or operators' information provisioned in 3GPP management system.

Based on the service related information of the analysis area, in which there may be one or more corresponding network functions such as NG-RAN gNBs, 5GC UPFs, SMFs, 3GPP management system decides ES actions for the corresponding cells.

End of change