3GPP TSG SA WG5 Meeting 137-e		S5-213393r01

- e-meeting, 10th - 19th May 2021					                     							revision for S5-21xxxx

- Source:	Alibaba group

- Title:	use case – Exposure of MnS for monitoring QoS of video application

- Document for:	Approval

Agenda Item:	6.5.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

# 3	Rationale

This pCR is to study the use case on exposure of MnS for monitoring QoS of video application.

# 4	Detailed proposal

Start of 1st Change

## 5.Y	Exposure of MnS for monitoring QoS of video application

### 5.Y.1	Description

A live concert with high-resolution video application service is provided with service provider A (i.e. associate to MnS exposure service consumer). A local hosting network (e.g. SNPN) is available and provides localized services for high-resolution video service of the concert. The local hosting network is managed by the service provider A. The high-resolution video streaming service of the live concert is available to be accessed from both the local hosting network and PLMN. Service provider A and the provider of PLMN (i.e. associate to MnS exposure service provider) are two different parties. The service provider A of the local hosting network can have a contract with the Operator of PLMN before discovering the proper MnSs that are needed through the MnS discovery service for exposure. According to the contract, the service provider A can have the permission to use the MnS for reading certain managed object related to QoS for the PLMN.

Once the contract is made, the corresponding MnF within the 3GPP management system is configured with permission rule which defines that the provider of the local hosting network can have the right to read certain managed object regarding QoS. The NRM fragments that are required to be read can be related to NR and 5GC. Through monitoring the managed object using exposure capability, the provider of the local hosting network can determine the situation when PLMN can not support the high-resolution video service with satisfied QoS. In this case, the service provider A can notify its customers to get access to the local hosting network for improving the QoS of the high-resolution video application.

### 5.Y.2	Issue and gaps

#### Gap:

The definition and the format of the permission rule for the network management capability exposure regarding NR and 5GC related managed object is not specified in existing 3GPP management system.

How to configure the permission rule for the network management capability exposure regarding NR and 5GC related managed object in 3GPP management system is not specified in current specifications.

How to publish MnS which can be exposed to externals to a suitable MnS discovery service producer for the exposure of MnS regarding NR and 5GC is not specified in existing 3GPP management system.

How to manage the published MnS data of a MnS discovery service producer for the exposure of MnS regarding NR and 5GC is not specified in existing 3GPP management system.

Whether a discovery permission for network management capability exposure is needed is not well studied in existing 3GPP management system.

End of Change

Start of 2nd Change

- 6	Potential requirement for network management capability exposure

- REQ-NSCE-x The 3GPP management system should have the capability to support operator to configure permission rules for exposing NR and 5GC related managed object.

- REQ-NSCE-x The 3GPP management system should have the capability to publish MnS which can be exposed to externals to a suitable MnS discovery service producer for the exposure of MnS regarding NR and 5GC.

- REQ-NSCE-x The 3GPP management system should have the capability to manage the published MnS data of a MnS discovery service producer for the exposure of MnS regarding NR and 5GC.

- REQ-NSCE-x The 3GPP management system should have the capability to support the configuration of discovery permission for network management capability exposure if needed.

End of Change