3GPP TSG SA WG5 Meeting 137-e		S5-213393r02

- e-meeting, 10th - 19th May 2021					                     							revision for S5-21xxxx

- Source:	Alibaba group

- Title:	use case – Exposure of MnS for monitoring QoS of video application

- Document for:	Approval

Agenda Item:	6.5.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

# 3	Rationale

This pCR is to study the use case on exposure of MnS for monitoring QoS of video application.

# 4	Detailed proposal

Start of 1st Change

## 5.Y	Exposure of MnS for monitoring QoS of video application

### 5.Y.1	Description

A use case of exposure of MnS for monitoring QoS of video application can be described as follows:

1. A live concert with high-resolution video application service is provided with service provider A (i.e. associate to eMnS consumer). A local hosting network (e.g. SNPN) is available and provides localized services for high-resolution video service of the concert. The local hosting network is managed by the service provider A. The high-resolution video streaming service of the live concert is available to be accessed from both the local hosting network and PLMN. Service provider A and the provider of PLMN (i.e. associate to eMnS provider) are two different parties.

2. The service provider A of the local hosting network can have a contract with the Operator of PLMN before identifying the proper MnS for exposure through BSS (e.g. by using Service Catalog). The BSS may obtain the information of MnS that is allowed to be exposed using a proper MnS service for exposure provided by OSS. The contract may contain the agreement on the exposure of MnSs for accessing certain management MnS related to QoS. According to the contract, the service provider A can have the permission to use the MnS for accessing certain performance MnS related to QoS for the PLMN. The performance MnS can be related to NR and 5GC, e.g. Average DL UE throughput in gNB.

3. Once the contract is made, the corresponding MnF within the 3GPP management system is configured with permission rule through the interface between BSS and OSS. The permission rule defines that the provider of the local hosting network can have the right to access certain management MnS regarding QoS.

4. Through obtaining the measurement MnS using exposure capability, the provider of the local hosting network can determine the situation when PLMN can not support the high-resolution video service with satisfied QoS. In this case, the service provider A can notify its customers to get access to the local hosting network for improving the QoS of the high-resolution video application.

### 5.Y.2	Issue and gaps

#### Gap:

The definition and the format of the permission rule for the network management capability exposure regarding NR and 5GC related performance MnS is not specified in existing 3GPP management system.

Whehter and how to publish MnS which can be exposed to externals to a suitable MnS service producer for the exposure of performance MnS regarding NR and 5GC is not specified in existing 3GPP management system.

End of Change