3GPP TSG SA WG5 Meeting 137-e		S5-213566d2

- e-meeting, 10th - 19th May 2021					                     						revision for S5-213392

- Source:	Alibaba group

- Title:	Add Description of Concept and Roles to TR 28.824

- Document for:	Approval

Agenda Item:	6.5.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

# 3	Rationale

This pCR specifies the concepts and roles related to network management capability exposure.

# 4	Detailed proposal

Start of 1st Change

# 4	Overview

## 4.X	General

### 4.X.1	Concepts related to network management capability exposure

### 4.X.1.1	Exposed Management Services

Exposed MnS (eMnS) represents the MnS that can be exposed by MnS producer to the external MnS consumer. eMnS may rely on a dedicated MnF (e.g. EGMF) that manages the exposure aspects.

Editor’s notes: Whether eMnS is exposed to external MnS consumer via BSS or a dedicated exposure platform is FFS.

### 4.X.1.2	Exposure of Management Services

Exposure of management services indicates the case that an external MnS consumer which is outside 3GPP management system can access management capability offered by MnS producer within 3GPP management system. In order to enable the exposure of MnS, an external MnS consumer has to sign a contract, which contains the agreement on what MnS optionally under what condition can be exposed, with an MNO. The signing of the contract may need the interaction with BSS. The condition can be certain constraint of MnS access based on the contract, e.g. the access quota of certain MnS, the access frequency of certain MnS, etc.

### 4.X.2	Roles related to network management capability exposure

### 4.X.2.1	Exposed MnS consumer

The logical entity accessing management capability offered by MnS producer, which is outside the 3GPP management system, is called exposed MnS consumer. An eMnS consumer is equivalent to MnS consumer which is outside the 3GPP management system. An eMnS consumer (e.g. verticals such as Industry, Internet Company, etc) can be associated to CSC or NSC. The eMnS consumers usually have specific service requirement on 5G network for their applications and wish to get access to certain management capability supported by 3GPP management system. In order to enable the exposure of MnS, the stakeholder (e.g. MNO, MVNO) usually makes contract with the vertical. The contract may contain the agreement on what MnS, optionally under what condition, can be exposed. Once the contract has been signed, the BSS may interact with OSS in order to complete the related configuration (e.g. what MnS, optionally under what condition, can be exposed) regarding the exposure of MnS within OSS. After that, the eMnS consumer can get access to management capability offered by MnS producer within 3GPP management system. The access may need the interaction with BSS (e.g. through Service Catalog).

### 4.X.2.2	Exposed MnS producer

The logical entity offering management capability that can be accessed by a MnS consumer (e.g. verticals and service provider) outside the 3GPP management system is called exposed MnS producer. An eMnS producer is equivalent to MnS producer having certain MnS that is allowed to be exposed to eMnS consumer. An eMnS producer can be associated to CSP or NSP. The vertical makes the contract with the stakeholder (e.g. MNO, MVNO) regarding the expose of MnSs.  The contract can include what kinds of MnS, optionally under certain condition, can be exposed to the eMnS consumer. The eMnS producer can offer management capability that is allowed to be accessed by the eMnS consumer upon the completion of configuration with respect to the exposure of MnS. The access may need the interaction with BSS (e.g. through Service Catalog). The contract includes what kinds of MnS, optionally under certain condition, can be exposed to the eMnS consumer.

End of Change