3GPP TSG SA WG5 Meeting 137-e		S5-213566d5

- e-meeting, 10th - 19th May 2021					                     						revision for S5-213392

- Source:	Alibaba group

- Title:	Add Description of Concept and Roles to TR 28.824

- Document for:	Approval

Agenda Item:	6.5.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

# 3	Rationale

This pCR specifies the concepts and roles related to network management capability exposure.

# 4	Detailed proposal

Start of 1st Change

# 4	Overview

## 4.X	General

### 4.X.1	Concepts related to network management capability exposure

### 4.X.1.1	Exposed Management Services

Exposed MnS (eMnS) represents the MnS that can be exposed by MnS producer to the external MnS consumer. eMnS may rely on a dedicated MnF (e.g. EGMF defined in 3GPP or function defined in other standard like TMF) that manages the exposure aspects.

Editor’s notes: Whether eMnS is exposed transparently to external MnS consumer via BSS or being processed through a dedicated exposure platform is FFS.

### 4.X.1.2	Exposure of Management Services

Exposure of management services indicates the case that an external MnS consumer which is outside 3GPP management system can indrectly access management capability offered by MnS producer within 3GPP management system. In order to enable the exposure of eMnS, an external customer, which has external MnS consumer, has to sign a contract, which contains the agreement on what eMnS optionally under what condition can be exposed, with an MNO. The signing of the contract may need the interaction with BSS. The condition can be certain constraint of eMnS access based on the contract, e.g. the access quota of certain eMnS, the access frequency of certain eMnS, etc.

### 4.X.2	Roles related to network management capability exposure

### 4.X.2.1	Exposed MnS consumer

The logical entity, external to the 3GPP management system, accessing management capability offered by MnS producer is called an eMnS consumer. An eMnS consumer is equivalent to MnS consumer which is outside the 3GPP management system. An eMnS consumer is owned by an external customer (e.g. vertical such as Industry, Internet Company, etc) which may take the role of a CSC or NSC. The external customer usually has specific service requirements on 5G network. In order to ensure access to eMnS, the stakeholder (e.g. MNO, MVNO) usually makes contract with the external customer. The contract may contain the agreement on what eMnS, optionally under what condition, can be exposed. Once the contract has been signed, the BSS may interact with OSS in order to complete the related configuration (e.g. what eMnS, optionally under what condition, can be exposed) regarding the exposure of eMnS within OSS. After that, the eMnS consumer can get access to management capability offered by MnS producer within 3GPP management system. The access may need the interaction with BSS (e.g. through Service Catalog).

### 4.X.2.2	Exposed MnS producer

The logical entity offering management capability that can be accessed by an eMnS consumer outside the 3GPP management system is called exposed MnS producer. An eMnS producer is owned by a service provider which may take the role of a CSP or NSP. The external customer makes the contract with the stakeholder (e.g. MNO, MVNO) regarding the expose of eMnSs.  The contract can include what kinds of eMnS, optionally under certain condition, can be exposed to the eMnS consumer. The eMnS producer can offer management capability that is allowed to be accessed by the eMnS consumer upon the completion of configuration with respect to the exposure of eMnS. The access may need the interaction with BSS (e.g. through Service Catalog). The contract includes what kinds of eMnS, optionally under certain condition, can be exposed to the eMnS consumer.

End of Change