3GPP TSG-SA5 Meeting #138-e 	S5-214120rev1

# e-meeting, 23 - 31 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.545 | CR | 0011 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case to management capability for tenant |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMEMTANE |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The NRM IOC may be introduced to support management capabilities for vertical enterprise use (e.g., tenant). It is necessary to describe the use case and requirement(s) of fault management of network slice instance as this management capability is newly added for tenant. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed changes in this CR are to support a management object for tenant that is capable to expose the alarm notification of network slice to tenant, that is represented the MOI provided by 3GPP management system. The statement to provide performance monitoring and alarm notification are to be desribed in the UC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case of tenant capability is not clear in this specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.x (new), 5.2.1, 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

5.1.x	MnS of alarm notification provided to tenant

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Enable MnS of alarm notification for vertical enterprise (tenant) to notify the particular alarm notifications of network slice. |  |
| Actors and Roles | MnS producer of fault management for network slice and vertical consumer. MnS Consumer of fault management for network slice, as tenant in this use case |  |
| Telecom resources | Fault management service producer Exposure governance that is capable to provide fault management service to tenant. |  |
| Assumptions | The vertical enterprise (tenant) as MnS consumer is authorized to consume fault management service of network slice. The alarm(s) are generated by 3GPP management system regarding particular S-NSSAI, or network slice instance ID. The MnS consumer is capable to consumer the FM MnS as the exposure governance is capable to provide the FM MnS for this tenant. |  |
| Pre-conditions | The exposure governance is capable to provide MnS for tenant, as the MOI of tenant is available. The MnS consumer, as tenant in this context, is authorized to consume fault management service. |  |
| Begins when | The MnS consumer wants to obtain the alarm event by subscribing alarm notifications regarding consumed network slice(s). |  |
| Step 1 (M) | The MnS consumer informs exposure governance that it wishes to receive alarm notifications for specific network slice(s). The exposure governance check credential of MnS consumer and authorization of the MnS consumer for FM MnS. |  |
| Step 2 (M) | The management function (e.g., the exposure governance) is capable to subscribe alarm notification of network slice via FM MnS to other management functions. |  |
| Step 3 (O) | The exposure governance subscribes alarm notification of network to other management function(s) to obtain alarm event(s) of network slice. |  |
| Step 4 (M) | FM MnS provides alarm notification for parituclar subscribed alarm event to MnS consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | The vertical MnS consumer is authroized has be notified the alarm event(s) via FM MnS. |  |
| Traceability | REQ-FSDR_NSI-FUN-1Y, REQ-FSDR_NSSI-FUN-1Y |  |

Next modification

5.2.1	Requirements for fault supervision data report service for NSI

REQ-FSDR_NSI-FUN-1	The fault supervision data report service for NSI shall have the capability to provide alarm notifications of an NSI to its authorized consumer.

REQ-FSDR_NSI-FUN-2	The fault supervision data report service for NSI shall have the capability to allow its authorized consumer to subscribe the alarm notifications of an NSI.

REQ-FSDR_NSI-FUN-3	The fault supervision data report service for NSI shall have the capability to allow its authorized consumer to unsubscribe the alarm notifications of an NSI.

REQ-FSDR_NSI-FUN-4	The fault supervision data report service for NSI shall have the capability to allow its authorized consumer to provide the filter for alarm notifications.

REQ-FSDR_NSI-FUN-5	The fault supervision data report service for NSI shall have the capability to provide the alarm list of an NSI.

REQ-FSDR_NSI-FUN-6	The fault supervision data report service for NSI shall have the capability to allow its authorized consumer to provide the filter for the alarms.

REQ-FSDR_NSI-FUN-7	The fault supervision data report service for NSI shall have the capability to provide changed alarm notifications of an NSI to its authorized consumer.

REQ-FSDR_NSI-FUN-8	The fault supervision data report service for NSI shall have the capability to provide cleared alarm notifications of an NSI to its authorized consumer.

REQ-FSDR_NSI-FUN-9	The fault supervision data report service for NSI shall have the capability to provide new generated alarm notifications of an NSI to its authorized consumer.

REQ-FSDR_NSI-FUN-10	The fault supervision data report service for NSI shall have the capability to provide alarm list rebuilt notifications to its authorized consumer whenever the alarm list is rebuilt.

REQ-FSDR_NSI-FUN-1Y	 The 3GPP management system should have the capability for the tenant MnS consumer to subscribe alarm notification of NSI.

Next modification

5.2.3	Requirements for fault supervision data report service for NSSI

REQ-FSDR_NSSI-FUN-1	The fault supervision data report service for NSSI shall have the capability to provide alarm notifications of a network slice subnet instance to its authorized consumer.

REQ-FSDR_NSSI-FUN-2	The fault supervision data report service for NSSI shall have the capability to allow its authorized consumer to subscribe the alarm notifications of an NSSI.

REQ-FSDR_NSSI-FUN-3	The fault supervision data report service for NSSI shall have the capability to allow its authorized consumer to unsubscribe the alarm notifications of an NSSI.

REQ-FSDR_NSSI-FUN-4	The fault supervision data report service for NSSI shall have the capability to allow its authorized consumer to provide the filter for alarm notifications.

REQ-FSDR_NSSI-FUN-5	The fault supervision data report service for NSSI shall have the capability to provide the alarm list of an NSSI.

REQ-FSDR_NSSI-FUN-6	The fault supervision data report service for NSSI shall have the capability to allow its authorized consumer to provide the filter for the alarms.

REQ-FSDR_NSSI-FUN-7	The fault supervision data report service for NSSI shall have the capability to provide changed alarm notifications of an NSSI to its authorized consumer.

REQ-FSDR_NSSI-FUN-8	The fault supervision data report service for NSSI shall have the capability to provide cleared alarm notifications of an NSSI to its authorized consumer.

REQ-FSDR_NSSI-FUN-9	The fault supervision data report service for NSSI shall have the capability to provide new generated alarm notifications of an NSSI to its authorized consumer.

REQ-FSDR_NSSI-FUN-10 The fault supervision data report service for NSSI shall have the capability to provide alarm list rebuilt notifications to its authorized consumer whenever the alarm list is rebuilt

REQ-FSDR_NSSI-FUN-1y The 3GPP management system should have the capability for the tenant MnS consumer to subscribe alarm notification of NSSI.

Next modification