3GPP TSG-SA5 Meeting #138-e 	S5-214368rev1

# e-meeting 23 - 31 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | - | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DAPS HO requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | E_HOO |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add requirements and use case for the development of management of DAPS HO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specification level requrements and use case for DAPS HO management added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.1.1.X, 6.4.1.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This is input to the Rel-17 28.313 draft CR for WI E_HOO. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ANR	Automatic Neighbour Relation

DAPS	Dual Active Protocol Stack

NCR	Neighbour Cell Relation

NG-RAN	Next Generation Radio Access Network

Next change

#### 6.1.1.X	DAPS handover management

REQ-DDAPSHO-FUN-1	The producer of NF provisioning MnS should have the capability allowing an authorized consumer to enable or disable DAPS handover from one cell to another cell.

REQ-DDAPSHO-FUN-2	The producer of NF provisioning MnS should have the capability allowing an authorized consumer to configure parameters for the DAPS handover function.

REQ-DDAPSHO-FUN-3	The producer of NF performance assurance MnS should have the capability to produce measurements related to DAPS handover.

Next change

#### 6.4.1.X	DAPS HO (Dual Active Protocol Stack Handover)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To configure DAPS HO parameters in cells in order to improve DAPS HO performance. |  |
| Actors and Roles | D-SON management function to support the DAPS HO function. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS. |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - DAPS HO is not in operation from the source cell to the target cell. |  |
| Begins when | The D-SON management function intends to enable DAPS HO from the source cell to the target cell. |  |
| Step 1 (M) | The D-SON management function requests the producer of NF provisioning MnS to configure parameters for the management of DAPS HO on the source cell. |  |
| Step 2 (M) | The D-SON management function requests the producer of provisioning MnS to enable DAPS HO from a source cell to a target cell. |  |
| Step 3 (M) | The DAPS HO function detects handover issues (e.g. too late DAPS HO, too early DAPS HO and DAPS HO to a wrong cell) by analysing reports from UEs and network side information, and acts to mitigate the DAPS HO issues by adjusting DAPS HO related RRC parameters sent from the gNB to the UE. |  |
| Step 4 (M) | The D-SON management function collects DAPS HO related measurements and analyses them to evaluate the DAPS HO performance. |  |
| Step 5 (M) | If the D-SON management function does not find the DAPS HO performance satisfactory, it updates the configuration parameters for the DAPS HO function. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | DAPS HO is in operation from the source cell to the target cell. |  |
| Traceability | REQ-DDAPSHO-FUN-1, REQ-DDAPSHO-FUN-2, REQ-DDAPSHO-FUN-3 |  |

End of changes