3GPP TSG-SA5 Meeting #140-e 	S5-216082

- e-meeting, 15 - 24 November 2021

- Source:	Ericsson

- Title:	Adding reference point architecture to clause 7.1.4

- Document for:	Approval

Agenda Item:	7.5.7

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: " Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Adding reference point architecture to clause 7.1.4.

# 4	Detailed proposal

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501:"System Architecture for the 5G System".

[3]	GSMA TD.201: "Common Billing and Charging Processes ".

[4]	3GPP TS 32.255:" Charging management;5G Data connectivity domain charging; stage 2".

[5]	3GPP TS 32.256:" Charging management; 5G connection and mobility domain charging; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

Second change

### 7.1.4	Potential solutions

#### 7.1.4.1	Solution #1.1: CDR in VPLMN for wholesale of 5G data connectivity

##### 7.1.4.1.1	General

A possible solution for key issue 1a, wholesale charging for number of UEs and 5G data connectivity provided to the home MNO by the visited MNO, covering both home routed and local breakout case.

##### 7.1.4.1.2	Reference architecture

Figure 7.1.4.1.2-1: Roaming 5G data connectivity scenario in service-based interface representation

Figure 7.1.4.1.2-2: Roaming 5G data connectivity scenario in reference point representation

The SMF with CTF interact with CHF using Nchf interface. The CHF  produces CDRs that is sent to a CGF which may send this for interconnect charging (shown as dashed in the figures) which then aggregates and calculates the amount of data per home MNO, the interconnect part is only included for completeness and is currently outside the scope of 3GPP SA5.

##### 7.1.4.1.3	Message flows

For both home routed roaming and LBO, the flows for 5G data connectivity charging would be the same as in clause 5.2.2.12 of TS 32.255 [4] with the difference that for the LBO case would be that there would be no interaction with the home PLMN’s UPF, SMF and CHF.

#### 7.1.4.2	Solution #1.2: CDR in VPLMN for wholesale of 5G connection and mobility

##### 7.1.4.2.1	General

A possible solution for key issues 1b, wholesale charging for 5G connection and mobility (e.g. number of UEs) provided to the home MNO by the visited MNO, covering both home routed and local breakout case.

##### 7.1.4.2.2	Reference architecture

Figure 7.1.4.2.2-1: Roaming 5G connection and mobility scenario in service-based interface representation

Figure 7.1.4.2.2-2: Roaming 5G connection and mobility scenario in reference point representation

The AMF with CTF interact with CHF using Nchf interface. The CHF produces CDRs that is sent to a CGF which may send this for interconnect charging (shown as dashed in the figures) which then aggregates and calculates the number of connected UEs per home MNO, the interconnect part is only included for completeness and is currently outside the scope of 3GPP SA5.

##### 7.1.4.2.3	Message flows

The flows for 5G connection and mobility charging would be the same as in clause 5.2.2.5 of TS 32.256 [5], with the exception that the there is no need to interact with the home PLMN’s CHF and that only PEC is required. The CDRs generated in the CHF in VPLMN would in this case be used as input to the wholesale charging of the HPLMN. The following figure 7.1.4.2.2-1 describes a Registration charging message flow in PEC in VPLMN scenario for roaming, based on figure 4.2.2.2.2-1 of TS 23.502 [6] description

Figure 7.1.4.2.3-1: Roaming Registration – PEC in VPLMN

1-20a.	Registration procedure initiated by UE per steps 1 to 20a of figure 4.2.2.2.2-1 of TS 23.502 [6], with PCF, AUSF and UDM in HPLMN.

21.	Upon successful procedure, Registration Accept sent to the UE.

21ch-a. The AMF in VPLMN sends Charging Data Request [Event] to the CHF in VPLMN for the UE successful registration, indicating "roamer in".

21ch-b. The CHF in VPLMN creates the CDR for this registration.

21ch-c. The CHF in VPLMN acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

21b to 24. Same steps as in figure 4.2.2.2.2-1 of TS 23.502 [6].

#### 7.1.4.3	Solution #1.3: Nchf to HPLMN for wholesale of 5G data connectivity

7.1.4.3.1	General

##### 7.1.4.3.2	Reference architecture

The reference architecture would be the same as in clause 4.2.2 of TS 32.255 [4].

##### 7.1.4.3.3	Message flows

For local breakout roaming, the flows for 5G data connectivity charging are present as the following figure 7.1.4.3.2-1.

Figure 7.1.4.3.2-1: Local breakout roaming for 5G data connectivity charging

1-3a.	UE initiates a new PDU session. V-SMF selection by the AMF.

4ch-a. The UE is identified as a roamer (PLMN ID of the received SUPI is different from VPLMN PLMN ID), the V-CHF is selected accordingly. A Charging Data Request [Initial, Roaming Charging profile] is sent to V-CHF, indicating "in-bound roamer".

4ch-b. The V-CHF opens a CDR (indicating "in-bound roamer")

4ch-c. The V-CHF acknowledges by sending Charging Data Response [Initial, Roaming Charging profile] to the V-SMF.

7ch-a. A Charging Data Request [Update] is sent to V-CHF with charging information received from V-SMF.

7ch-b. The V-CHF updates the CDR.

7ch-c. The V-CHF acknowledges by sending Charging Data Response[Update] to the V-SMF.

10ch-a. A Charging Data Request [Termination] is sent to V-CHF.

10ch-b. The V-CHF closes the CDR

10ch-c. The V-CHF acknowledges by sending Charging Data Response[Termination] to the V-SMF.

Step 5, Step 8 and Step 11 are detailed in clause x.

End of changes