3GPP TSG-SA5 Meeting #140-e 	S5-216091

e-meeting, 15 - 24 November 2021	(revision of S5-215086)

- Source:	Huawei, China Telecom, CATT, AsiaInfo, China Unicom, China Mobile

- Title:	New WID on Enhanced intent driven management service for mobile networks

- Document for:	Approval

- Agenda Item:	6.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Enhanced intent driven management services for mobile networks

Acronym:   eIDMS_MN

Unique identifier:

Potential target Release:	{Rel-18}

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 810027 | Intent driven management service for mobile networks | SA5 work item |

Dependency on non-3GPP (draft) specification: none.

# 3	Justification

Several management scenarios including scenarios related to intent expectation for coverage assurance, intent expectation for throughput assurance, intent expectation for delivering radio network, and intent expectation for delivering radio service are specified in IDMS_MN WI. However, to use intent driven management approach to support more scenarios in 3gpp system, such as intent expectation for capacity assurance, intent expectation for energy saving/efficiency, intent for performance monitoring (consumer’s expression is for monitoring the performance (e.g. coverage performance, capacity performance) of the network. Then the MnS producer report the abstract network performance (e.g. weak coverage, good coverage) to MnS consumer instead of detailed measurements), more intent expectation given to 3gpp system needs to be described.

Management operations and information are also specified based on the defined scenarios. For the newly defined scenarios and requirements, the management operation and information needs to be defined accordingly.

When there are multiple intents in the MnS producer, the conflict between multiple intents is inevitable. Conflicting intents cannot be executed simultaneously, how to detect and resolve the conflict needs to be considered.

In order to allow the MnS consumer to get more information of the intent execution status, then the consumer can adjusts the intent based on the information, e.g. modify the intent target or constraint. Defining intent feedback information for different scenarios also needs to be considered.

Also there are discussion in other SDOs (e.g. TM Forum, ETSI ZSM) for intent driven management, the alignment/collaboration work for the intent driven management needs to be considered.

# 4	Objective

The objective of this work item is to enhance the intent driven management services of 5G networks by specifying the following aspects:

- Specify the new management scenarios and corresponding requirements for intent expectations given to 3gpp system, including but not limited to:

- Intent expectations for performance monitoring (e.g. consumer’s expectation for monitoring the coverage performance (e.g. weak coverage, good coverage) of the network).

- Intent expectations for radio network energy saving/efficiency to be assured.

- Intent expectations for radio network capacity performance to be assured.

- Specify the new generic capabilities for intent driven management, which includes but not limited to:

- Intent capability obtaining, allowing MnS consumer to obtain which intent expectation capability (e.g. coverage target and corresponding value range, RAN UE throughput target and corresponding value range, recommendations regarding partial or best effort fulfilling of the target) can be fulfilled by MnS producer.

- Enablers for automation of intent LCM (including adjustment the intent expectation in an autonomous way)

- Specify the solution (including enhance the intent information model based on R17 generic intent information model) to support:

  - Above newly defined requirements and capabilities

  - Rel-17 leftover requirements (including intent expectation and intent report for radio network coverage performance to be assured and RAN UE throughput performance to be assured, IntentExpectation for radio service).

- Collaboration/alignment for intent driven management with other SDOs (e.g. ETSI ZSM, TM Forum) should be considered.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 28.312 | -Add management scenario and corresponding requirements -Specify the new capabilities for intent driven management -Enhance the intent information model | June 2023(SA#100) |  |

# 6	Work item Rapporteur(s)

Xu Ruiyue, Huawei, Xuruiyue@huawei.com (Responsible for intent information model and new capabilities for intent driven management): primary Rapporteur

Lv Feibi,China Unicom : lvfb@chinaunicom.cn (Responsible for typical management scenario and corresponding requirements)

# 7	Work item leadership

SA WG5

# 8	Aspects that involve other WGs

Co-ordination with SA2, RAN3, ETSI ZSM, TM Forum where appropriate

# 9	Supporting Individual Members

Supporting IM name

Huawei

China Telecom

CATT

AsiaInfo

China Unicom

China Mobile

Deutsche Telekom