3GPP TSG-SA5 Meeting #140-e 	S5-216098

- e-meeting, 15 - 24 November 2021	revision of S5-215251

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.537 | CR | Draft CR | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 draftCR 28.537 Add requirements for data management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MADCOL |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add two requirements for data management production |  |  |  |  |  |  |  |  |
| Summary of change: |  | Consumer can request for data management production for a specific area of interest and according to a certain time scheduling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing requirements for data management production |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1, 6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Input to DraftCR to 28.537 related to the WI MADCOL |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 6	Managing management data

## 6.1	Producing and reporting management data

### 6.1.1	Description

Management data is referring to data produced by radio access network functions, core network functions or management functions and used for management purposes. Management data specified by 3GPP for 5G management is classified into 5G performance measurements as defined by TS 28.552 [4], 5G end to end key performance indicators as defined by TS 28.554 [5] and Trace/MDT data as defined by TS 32.422 [6]. The combined performance measurements and key performance indicators are also called performance metrics.

Management data is produced on request. Therefore, the 3GPP management system needs to enable a data consumer to request management data to be produced. The data requestor must specify the type of data to be produced as well as the radio access network functions, core network functions and management functions where the data shall be produced. The target managed object instances can be identified in multiple ways:

-	The requestor can specify the target managed object instances based on the managed object tree (as defined in the 3GPP Network Resource Models) representing the network and management functions. The simplest approach is to directly identify the managed object instances where data shall be produced. More sophisticated approaches allow to specify one or more subtrees where data shall be produced and may allow also to specify managed object classes to select only object instances of specific classes.

-	The requestor can specify an area of interest. The area of interest can be expressed for example with a geographical area, one or several cells or one or several tracking areas. The system needs to translate this information into the target managed object instances representing network functions serving that area of interest. Geographical areas can be expressed for example with multiple longitude-latitude pairs that define a convex polygon. In the radio domain the geographical area needs to be mapped to the coverage area of cells supported by RAN NE(s). The managed object instances (e.g. NRCellCU, GNBDUFunction) providing service to these cells can be identified as target managed object instances.

The management data can be requested according to a certain time scheduling. The consumer can e.g. specify a start and stop time or can request for data at specific days of a week or specific hours of a day.

After production the data needs to be reported to the data consumers. Reporting can be based on multiple reporting methods such as file or streaming. Data reporting needs to be requested by the data consumer. The requestor needs to specify the control parameters for reporting such as the reporting method and the address the data shall be delivered to.

Depending on access rights and security settings, data consumers may be subject to restrictions regarding the data they can access.

### 6.1.2	Use cases

This clause describes the benefits of the subject capability.

Editor's note: This clause will be extended with the benefits of the subject capability.

### 6.1.3	Requirements

REQ-MDMPR-CON-1: The 3GPP management system shall enable an authorized data consumer to request management data specified by 3GPP to be produced.

REQ-MDMPR-CON-2: The 3GPP management system shall enable an authorized data consumer to request management data specified by 3GPP to be produced for a certain area of interest.

REQ-MDMPR-CON-3: The 3GPP management system shall enable an authorized data consumer to request management data specified by 3GPP to be produced according to a certain time scheduling.

REQ-MDMPR-CON-4: The 3GPP management system shall enable an authorized data consumer to request management data specified by 3GPP to be reported to the requesting or any other authorized data consumer.

NOTE: The term "management data specified by 3GPP" relates to

-	5G performance measurements as defined by TS 28.552 [4]

-	5G end to end key performance indicators as defined by TS 28.554 [5], and

-	Trace/MDT data as defined by TS 32.422 [6].

6.2	Coordinating management data production

### 6.2.1	Description

Many consumers can request network or management functions to produce management data. In this context it is beneficial to coordinate data requests at the management level to optimize management data production.

### 6.2.2	Use cases

This clause describes the benefits of the subject capability.

Editor's note: This clause will be extended with the benefits of the subject capability.

### 6.2.3	Requirements

REQ-MDMC-CON-1: The 3GPP management system shall coordinate requests from several data consumers to avoid producing multiple times the same data at a certain point of time.

Editor's note: It is tbc what exactly is "same data".

End of changes