3GPP TSG-SA5 Meeting #140-e 	S5-216144

- e-meeting, 15 - 24 November 2021													revision of S5-21xabc

- Source:	Huawei Technologies

- Title:	pCR TR 28.822 Remove the editor’s note for topic 4

- Document for:	Approval

Agenda Item:	7.5.4

# 1	Discussion

This is a pCR to TR 28.822 removing the editor’s note for topic 4.

# 2	References

[1]	3GPP TR 28.822 "Telecommunication management; Charging management; Study on charging aspect of 5G LAN-type Services".

# 3	Rationale

This pCR proposes to remove the editor’s note for usage of 5G VN group communication in TR 28.822 " Telecommunication management; Charging management; Study on charging aspect of 5G LAN-type Services".

The duration of PDU session in context of 5G VN group communication is the aggregation of the duration of PDU session per UE in the 5G VN group communication. The solution #4.2: CHF aggregates the usage of 5G VN Group per PDU session is selected. Suggest to remove the editor’s note.

# 4	Detailed proposal

Propose to incorporate the following change into the TR 28.822[1].

1st change

### 5.4.2	Potential charging requirements

REQ-3GPPCH-UVN-01: The 5G system should support the capability to collect charging information of 5G VN group communication related to the data volume.

REQ-3GPPCH-UVN-02: The 5G system should support the capability to collect charging information of 5G VN group communication related to the duration of PDU sessions.

REQ-3GPPCH-UVN-03: The 5G system should support the capability to collect charging information of 5G VN group communication related to the number of terminals.

Next change

##### 5.4.4.1	General description

This solution#4.2 which relying on CHF/5G Converged Charging System for total volume and duration of 5G VN group communication charging, addresses the Key Issue#4.1.

As per the TS 23.501, in this release of the specification (R17), only a 1:1 mapping between (DNN, S-NSSAI) combination and 5G VN group is supported. In order to support the total volume and duration of 5G VN within a 5G VN, the SMF should aggregates the volume of the PDU sessions of the membership of the 5G LAN VN Group.

For the volume based charging of the 5G VN Group, the total volume is the aggregation of the volume per PDU session for uplink direction and downlink direction.

For the duration based charging of the 5G VN Group, the total duration is the aggregation of the duration per PDU session based on the start timestamp, stop timestamp and duration for each PDU session. The duration of one PDU session is counted as the duration between PDU session establishment and release.

For the terminals based charging of the 5G VN Group, the number of terminals is the number of the membership of 5G LAN VN Group.

End of changes