3GPP TSG-SA5 Meeting #140-e 	S5-216190rev3

- e-meeting, 15 - 24 November 2021

- Source:	Huawei, China mobile, China Telecom, AsiaInfo

- Title:	PCR TS 28.104 add MDA assisted Energy Saving use case and requirement

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	SP-210132 Enhancements of Management Data Analytics Service

[2]	3GPP TS 28.104-020 “Management and orchestration; Management Data Analytics”

# 3	Rationale

In the new WI of ‘Enhancements of Management Data Analytics Service’ MDA assisted Energy Saving is one of the MDA capabilities. This contribution proposes the description and requirements of the use case of the MDA assisted Energy Saving.

# 4	Detailed proposal

It is proposed to apply following changes in [2].

1st modified section

### 7.2.4	MDA assisted Energy Saving(UC-ES_MDA)

#### 7.2.4.1	Description

This MDA capability is  to provide the energy saving analysis.

#### 7.2.4.2	Use cases

Operators are aiming at decreasing power consumption in 5G networks to lower their operational expense with energy saving management solutions. Energy saving is achieved by activating the energy saving mode of the NR capacity booster cell or 5GC NFs (e.g., UPF etc). The energy saving activation decision making is typically based on the load information of the related cells/UPFs, the energy saving policies set by operators and the energy saving recommendations provided by MDAS producer. Under the energy saving state, the required network performance and network experience should also be guaranteed. Therefore, it is important to formulate appropriate energy saving policies (start time, dynamic threshold setting, base station parameter configuration, etc.).

To achieve an optimized balance between the energy consumed and the network performance, MDA can be used to assist the MDAS consumer to make energy saving decisions. To make the energy saving decision, it is necessary for MDAS consumer to determine where the energy efficiency issues (e.g., high energy consumption, low energy efficiency) exist, and the cause of the energy efficiency issues. Therefore, it is desirable for MDA to correlate and analyze the energy saving related performance measurements (e.g. PDCP data volume of cells, power consumption, etc.) and the network analysis data (e.g., observed service experience related network data analytics) to provide the analytics results of current network energy efficiency.

To make the energy saving decision, it is necessary for MDAS consumer to determine which EE KPI related factor(s) (e.g., traffic load, end-to-end latency, active UE numbers, etc) are affected or potentially affected. The MDAS producer can utilize historical data to predict the efficiency KPI related factors (e.g., load variation of cells at some future time, etc). The prediction result of these information can then be used by operators to make energy-saving decision to guarantee the service experience.

The MDAS producer may also provide energy saving related recommendation to the MDAS consumer, The MDAS consumer may take the recommendations into account for making energy saving decisions. After the recommendations have been executed, the MDA producer may start evaluating and further analyze network management data to optimize the recommendations.

Edior Note: The energy saving related recommendation and the current energy saving state of cell are both necessary for consumer. The current energy saving state of cell can be discussed in the future.

#### 7.2.4.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ES_MDA-CON-1 | 3GPP management system shall have the capability to identify the energy efficiency issue (including high energy consumption, low energy efficiency), and identify the cell/NFs or location area of where the indicated energy efficiency issue exists. | (UC-ES_MDA) MDA assisted Energy Saving |
| REQ-ES_MDA-CON-2 | 3GPP management system shall have the capability to identify the root cause of the energy efficiency issue when necessary. | (UC-ES_MDA) MDA assisted Energy Saving |
| REQ-ES_MDA-CON-3 | 3GPP management system shall have the capability to utilize the the network status analysis and predictions information of the energy efficiency KPI factors (e.g., traffic load trends) to assist achieving energy saving. | (UC-ES_MDA) MDA assisted Energy Saving |
| REQ-ES_MDA-CON-4 | 3GPP management system shall be able to provide the energy saving recommendation, such as policies and simple configuration actions to guarantee the network performance and network experience. | (UC-ES_MDA) MDA assisted Energy Saving |

End of modified section