3GPP TSG-SA5 Meeting #140-e 	S5-216191rev3

- e-meeting, 15 - 24 November 2021

- Source:	Huawei

- Title:	PCR TS 28.104 add MDA assisted Energy Saving use case solution

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TS 28.104-020 “Management and orchestration; Management Data Analytics”

# 3	Rationale

The objectives in approved WI include defining the use case of MDA assisted Energy Saving. This contribution is proposed to define MDA assisted Energy Saving use case solution for MDAS in [1].

# 4	Detailed proposal

It is proposed to apply following changes in [1].

1st modified section

### 8.4.X.1	MDA assisted energy saving

#### 8.4.X.1.1	MDA type

The MDA type for MDA assisted energy saving is: EnergySavingAnalysis.

#### 8.4.X.1.3	Analytics output

The specific information elements of the analytics output (MDA report) for MDA assisted energy saving analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.X.1.3-1.

Table 8.4.X.1.3-1: Analytics output for MDA assisted energy saving analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| Location | The list of cells where the energy efficiency issue exists. | M | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TrafficLoadTrends | Predictions related to the trends of traffic load (e.g., PRB utilization rate) which could be used as references for making decision on energy saving. | M | type:TrafficLoadTrend multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RootCause | Indicates the root cause of the energy consumption that may be conserved, e.g., ultra-low traffic load area with energy consumption, excessive energy consumption. . | O | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

2nd modified section

X.Y.1	<<DataType>>TrafficLoadTrend

#### X.Y.1.1	Definition

This data type specifies the type of TrafficLoadTrend.

#### X.Y.1.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| CellId | It indicates the cell for which the traffic load prediction is performed. | M | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Timestamp | It provides the time point corresponding to the predicted traffic load. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TrafficLoad | It provides the predicted traffic load, e.g., PRB utilization rate and its value. | M | type: Integar multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of modified section