3GPP TSG SA WG5 Meeting 140-e		S5-216237

- Electronic meeting, online, 15 - 24 Nov. 2021

- Source:	Huawei

- Title:	Add E2E latency analysis solution

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-020 “Management and orchestration; Management Data Analytics”

# 3	Rationale

E2E latency is an important parameter for 5G services such as URLLC. User data packets should be successfullydelivered within certain time constraints to satisfy the end users requirements. Latency could be impacted by the network capability and network configurations, e.g. configuration of service priority, RAN capacity, network load, number of re-transmissions, Wireless channel environment and the processing time of the network functions, etc. And the affected objects, the root cause of the too high latency and solutions to solve the latency issue need to be discussed.

This contribution is proposed to provide the E2E latency analysis capability of MDA in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

First of the change

- x.x.x	SLS analysis

- x.x.x.x	  E2E latency analysis

- x.x.x.x.1	  MDA type

The MDA type for Capability-E2E latency analysis is: E2ElatencyAnalysis.SLSAnalysis

- x.x.x.x.2	Analytics output

Table x.x.x.x.2-1:  E2E latency analytics output

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| E2E latency issue analysis Identifier | The identifier indicates the output is for E2E latency issue analysis | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Indication of E2E latency issue type | Indication the type of the E2E latency issue, e.g., RAN latency issue, CN latency issue, TN latency issue, UE latency issue, service provider latency issue. | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Affected objects | The MOIs of subnetwork or network slices, UEs or service provider servers where the latency issue happens; | O | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Root cause | The root cause of the E2E latency degradation | O | type: FFS multiplicity: FFS isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Recommended actions | The recommended actions to solve the latency issue | O | type: FFS multiplicity: FFS isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of the change