3GPP TSG SA WG5 Meeting 140-e		S5-216239

- Electronic meeting, online, 15 - 24 Nov. 2021

- Source:	Huawei

- Title:	Add network slice throughput analysis solution

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-020 “Management and orchestration; Management Data Analytics”

# 3	Rationale

Multiple SLS related attributes are defined in ServiceProfile, e.g., maxNumberofUEs, latency, throughput or etc, which are of great importance and should be satisfied. SLS analysis issues are studied in [1], including not only latency analysis, load analysis, jitter analysis.For different network services or vertical users, the requirements may varies a lot. This contribution is proposed to add the network slice throughput analysis capability of MDA in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

First of changes

- 8.4.x	SLS analysis

- 8.4.x.x	  Network slice throughput analysis

- 8.4.x.x.1	  MDA type

The MDA type for Capability- Network slice throughput analysis is: NetworkSliceThroughputAnalysis.SLSAnalysis

- 8.4.x.x.2	Analysis output

The specific information elements of the analytics output (MDA report) for network slice throughput analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.x.2-1.

Table 8.4.x.x.2-1:  Network slice throughput analysis output

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| Network slice throughput issue identifier | Network slice throughput issue identifier | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Indication of issue type | Indication of the network slice throughput issue type, e.g. RAN issue, CN issue. | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Network slice throughput statistics | Average percentage of users, for which the required SLS throughput satisfies Average percentage of time, during which the required SLS throughput satisfies | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Network slice throughput predictions | Average percentage of users, for which the required SLS throughput could be met. Average percentage of time, during which the required SLS throughput could be met. | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Root cause | The root cause of the network slice throughput issues, e.g. PRB resource shortage; | O | type: FFS multiplicity: FFS isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Recommended actions | The recommend actions to solve the network slice throughput issues, e.g. radio resource reconfiguration | O | type: FFS multiplicity: FFS isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes