3GPP TSG-SA5 Meeting #140e 	S5-216283rev3

- e-meeting, 15 - 24 November 2021												  Revision of S5-21xxxx

- Source:	Alibaba Group, AsiaInfo

- Title:	Add use case for the exposure without going through BSS

- Document for:	Approval

Agenda Item:	6.5.4

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	TR 28.824 0.2.0: Study on network slice management capability exposure

# 3	Rationale

This contribution proposes to add a use case for the exposure without going through BSS.

# 4	Detailed proposal

This document proposes the following changes in TR 28.824 [1].

1st Change

# 4	Overview

## 4.1	General

### 4.1.1	Concept related to network management capability exposure

#### 4.1.1.4	Exposure without going through BSS

##### 4.1.1.4.1	General

Exposure of service data to companies that are external to the operator are regulated by contracts. The exposure usually goes through BSS, as explained in clause 4.1.1.2. However, based on specific contract, the customer may interact with the Operator without going through BSS. There are different scenarios for how a customer having such contract interacts with Operator for the network slice management capability exposure.

##### 4.1.1.4.2	Exposure scenarios

Scenario 1: The NOP and NSP belong to the same Operator. The OSS/SML and OSS/NML are connected via internal interface. The external customer has an external interface with the NSP based on the contract between the external customer and NSP.

NOTE 1: External customer may have connection with NSP BSS for the product-level interaction. If not, the OSS/SML may have an embedded BSS functionalities for the product-level interaction.

Editor’s notes: Whether the embedded BSS functionalities are implemented in EGMF or other management function is FFS.

NOTE 2: The Company-B BSS connects to the BSS part of external customer and the Company-B OSS/SML connect to the OSS/SML part of external customer.

Figure 4.1.1.4.2-1 NSP OSS/SML to customer being an external interface

Scenario 2: The NOP may interface to an external NSP that has a contract with NOP for the exposure directly via OSS. The NOP’s OSS/SML have a direct machine to machine interface with the NSP’s OSS/SML. The external customer has an external interface with the NSP OSS/SML based on the contract between external customer and NSP.

NOTE 3: External customer may have connection with NSP BSS for the product-level interaction. If not, the NSP OSS/SML may have an embedded BSS functionalities for the product-level interaction.

Editor’s notes: Whether the embedded BSS functionalities are implemented in EGMF or other management function is FFS.

NOTE 4: If the external customer can get access to the OSS directly, it must maintain a copy of a part of the operator’s MIB.

Editor’s notes: Whether and how does the external customer obtain the copy of a part of the operator’s MIB is FFS.

NOTE 5: The Company-B BSS connects to the BSS part of external customer and the Company-B OSS/SML connect to the OSS/SML part of external customer.

Figure 4.1.1.4.2-2 NOP OSS/SML to NSP OSS/SML interface being an external interface

Scenario 3: The NOP OSS/SML may interface to an external NSP OSS/SML based on the contract between the two parties. The "External NSP OSS/SML" has internal interface with NSP BSS. The NSP might have a machine to machine interface towards their customer (e.g. a vertical) via their BSS.

NOTE 6: External customer may have connection with BSS for the product-based interaction. If not, the NSP OSS/SML may have an embedded BSS functionalities for the product-based interaction.

Editor’s notes: Whether the embedded BSS functionalities are implemented in EGMF or other management function is FFS.

NOTE 7: If the external customer can get access to the OSS directly, it must maintain a copy of a part of the operator’s MIB.

Editor’s notes: Whether and how does the external customer obtain the copy of a part of the operator’s MIB is FFS.

Figure 4.1.1.4.2-3 NOP OSS/SML to NSP OSS/SML being an external interface, NSP BSS to customer being an external interface

End of changes