- 3GPP TSG-SA5 Meeting #140e	S5-216334rev2

- 15 - 24 November 2021, E-meeting

- Source:	Intel, HUAWEI

- Title:	pCR 28.815 Add evaluation and conclusion of edge application access charging via EAS

- Document for:	Approval

- Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.815-130 “Study on Edge Computing charging”

# 3	Rationale

The possible solution for edge application access charging via EAS is not evaluated and concluded.

# 4	Detailed proposal

Start of modification

#### 7.1.4.3	Possible solutions for edge application access charging via EAS

Solution #1l: The possible solutions to support the subscriber based charging related potential requirements REQ-CH_EC_5GS-02, and Key Issue #1b could be the following architecture.

The solution is appliable to the scenario where EAS is provided by the MNO.

Figure 7.1.4.3-1: EAS based solution

The solution with CTF embedded in the EAS is illustrated in Figure 7.1.4.3-2.

Figure 7.1.4.3-2: duration based charging for 5GS capabilities (with EAS supporting CTF)

The CHF of MNO is configured with quota and reporting threshold for duration charging.

The operations is the same with the 7.1.4.2-2, with EAS instead of the SMF.

Next modification

#### 7.1.5.x	Evaluation of possible solutions for edge application access charging via EAS

Solution #1l support edge application access charging via EAS, for the scenario that the EAS is provided by the MNO. However, EAS may be provided by an ASP which does not belong to the same enterprise of the MNO, and in Rel-17 3GPP has no standards on application level communication between EAS and UE, so it is not feasible to have standard charging inforamtion to support CTF in EAS in Rel-17.

Next modification

#### 7.1.6.x	Conclusion of possible solutions for edge application access charging via EAS

Based on the evaluation in clause 7.1.5.x, the Solution #1l for edge application access charging via EAS as described in clause 7.1.4.3 is feasible with restrictive to some special deployment scenario (where the EAS is provided by MNO). Because of the lack of standard solutions to define the application level communication to support CTF in EAS based on the 3GPP R17 MEC standardizationEdge Computing related specifications, therefore it is not recommended to be a normative solution in Rel-17.

End of modifications