3GPP TSG-SA5 Meeting #140-e 	S5-216454

e-meeting, November 15 – 24 2021

- Source:	Huawei,Nokia

- Title:	Update RadioNetworkExpectation in clause 6.2.1.2.4

- Document for:	Approval

- Agenda Item:	6.4.10

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v0.6.0”.

# 3	Rationale

This contribution proposes to update RadioNetworkExpectation in clause 6.2.1.2.4 to align with the common structure for all domain/scenario specific intent expectations in 6.2.1.2.2.

# 4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

# Annex C			IntentExpection Example: Intent Expectation information elements for radio network

- Editor's note: Some content of this Annex can become a normative content, which is FFS.

## C.1	Definition

Expectation for Radio Network is an example of IntentExpectation which can be used by MnS consumer to express requirements, goals and contexts for a radio network.

Editor's Note: The following information will be restructured to align with generic IntentExpectation.

## C.2	ExpectationObject

For the Expectation for Radio Network, the value for attribute "ExpectationObject" is

FFS.

## C.3	ExpectationObjectContexts

For the Expectation for Radio Network, the ExpectationObjectContext can be a list of coverage areas for radio network. This can be implemented by multiple choices, including CoverageAreaPolygon, TrackingAreaCode. Following is the attribute table and attribute constraints for an expected radio network:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| coverageAreaPolygonContext | CM | T | T | F | T |
| tACContext | CM | T | T | F | T |

| coverageAreaPolygonContext support Qualifier | Condition: MnS consumer express the area in CoverageAreaPolygon. |
| --- | --- |
| tACContext support Qualifier | Condition: MnS consumer express the area in TrackingAreaCode. |

Editor's Note: the attribute "coverageAreaPolygonContext" as context for IntentExpectation for Radio Network is FFS

## C.4	ExpectationTargets

For the Expectation for Radio Network, the expectationTargets can include one or multiple of the following targets for an expected radio network. When implementing it, this also can be extended to include other radio network related targets.

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| WeakRSRPRatioTarget | O | T | T | F | T |
| LowSINRRatioTarget | O | T | T | F | T |
| AveULRANUEThptTarget | O | T | T | F | T |
| AveDLRANUEthptTarget | O | T | T | F | T |
| LowULRANUEThptRatioTarget | O | T | T | F | T |
| LowDLRANUEThptRatioTarget | O | T | T | F | T |

Editor's Note:  the list of targets is FFS.

## C.5	ExpectationContexts

For the RadioNetworkExpectation, the ExpectationContexts can include one or multiple of the following contexts for an expected radio network:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdContext | O | T | T | F | T |
| cellIdContext | O | T | T | F | T |
| gNBIdContext | O | T | T | F | T |
| pCIContext | O | T | T | F | T |
| arfcnDLContext | O | T | T | F | T |
| arfcnULContext | O | T | T | F | T |
| bSChannelBwDLContext | O | T | T | F | T |
| bSChannelBwULContext | O | T | T | F | T |
| ngcLocalIpAddressContext | O | T | T | F | T |
| nguRemoteIpAddressContext | O | T | T | F | T |
| logicInterfaceContext | O | T | T | F | T |

C.6	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| WeakRSRPRatioTarget | It describes the weak coverage ratio target for the expected radio network. WeakRSRPRatioTarget include the following targetContext: - WeakRSRPContext, which used to describe threshold for weak RSRP. | type: IntentTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| WeakRSRPRatioTarget.WeakRSRPContext | It describes the threshold for weak RSRP for the expected radio network. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| LowSINRRatioTarget | It describes the low SINR ratio target for the expected radio network. LowSINRRatioTarget include the following targetContext: WeakRSRPContext, which used to describe threshold for weak RSRP. | type: IntentTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| LowSINRRatioTarget.LowSINRContext | It describes the threshold for low SINR for the expected radio network. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| AveULRANUEThptTarget | It describes the average UL RAN UE throughput target for the expected radio network. | type: IntentTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| AveDLRANUEThptTarget | It describes the average DL RAN UE throughput for the expected radio network. | type: IntentTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| LowULRANUEThptRatioTarget | It describes the low UL RAN UE throughput ratio target for the expected radio network. LowULRANUEThptRatioTarget include the following targetContext: LowULRANUEThptContext, which used to describe threshold for low UL RAN UE throughput. | type: IntentTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| LowULRANUEThptRatioTarget.LowULRANUEThptContext | It describes the low UL RAN UE throughput for the expected radio network | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| LowDLRANUEThptRatioTarget | It describes the low DL RAN UE throughput ratio target for the expected radio network. LowDLRANUEThptRatioTarget include the following targetContext: LowDLRANUEThptContext, which used to describe threshold for low DL RAN UE throughput. | type: IntentTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| LowDLRANUEThptRatioTarget.LowDLRANUEThptContext | It describes the low DL RAN UE throughput for the expected radio network | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| cellIdContext | It describes a range of cell identity which can be assigned to the expected radio network. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBIdContext | It describes a range of gNB Id which can be assigned to the expected radio network. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pLMNIdContext | It describes a list of PLMN Id which can be assigned to the expected radio network. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pCIRanges | It describes a range of PCI which can be assigned to the expected radio network. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnDLContext | It describes the value of arfcnDL for the expected radio network. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnULContext | It describes the value of arfcnUL for the expected radio network to be deployed. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwDLContext | It describes the bSChannelBwDL for the expected radio network. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwULContext | It describes the bSChannelBwUL of the expected radio network. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ngcLocalIpAddressContext | It describes a list of ngcLocalIpAddress of the NG control plane interface (NG-C) for the expected radio network. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nguRemoteIpAddressContext | It describes a list of nguLocalIpAddress of the NG user plane interface (NG-U) for the expected radio network to be deployed. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| logicInterfaceContext | It describes a logical transport interface for the expected radio network to be deployed | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes