3GPP TSG-SA5 Meeting #141-e 	S5-221096

- e-meeting, 17th- 26th Janaray 2022													revision of S5-21xabc

- Source:	Huawei Technologies

- Title:	pCR TR 28.822 Correction on the solution and conclusion for Topic 4

- Document for:	Approval

Agenda Item:	7.5.1

# 1	Discussion

This is a pCR to TR 28.822 correcting the solution and conclusion for Topic 4.

# 2	References

[1]	3GPP TR 28.822 "Telecommunication management; Charging management; Study on charging aspect of 5G LAN-type Services".

# 3	Rationale

This pCR proposes to correct the CHF aggregation solution of TR 28.822.

- Change CHF aggregation to CHF/BD aggreation.

# 4	Detailed proposal

Propose to incorporate the following change into the TR 28.822[1].

1st change

#### 5.4.4.2		Solution #4.2: CCS aggregates the usage of 5G VN Group per PDU session

##### 5.4.4.2.1	General description

This solution#4.2 which relying on 5G Converged Charging System for total volume and duration of 5G VN group communication charging, addresses the Key Issue#4.1.

The SMF reports the usage (e.g. uplink and downlink volume, duration) per PDU session, the CCS aggregates the charging information based on the total usage PDU session. In order to avoid the duplication aggregation, 5G VN group identifier, the uplink/downlink direction and traffic forwarding way should be reported to CHF.

##### 5.4.4.2.2	Architecture description

The converged charging architecture for SMF Charging Trigger Function (CTF) based solution as depicted in figure 5.2.4.1.2.1-1 SMF based solution-CTF for the session based charging is applicable for the solution #4.2.

##### 5.4.4.2.3	Procedures description

Figure 5.4.4.2.3-1: CCS aggregates per PDU session

### 5.4.5 	Evaluation

#### 5.4.5.1	Solutions evaluation for Key issue #4.1

Both solution #4.1 and #4.2 address Key issue #4.1.

For the solution #4.1, in many cases, charging information about the individual as well as the aggregated is required, the SMF aggregating would result in the double reporting from the SMF to the CHF. New charging resource for SMF aggregating 5G VN Group charging information should be defined. For the solution #4.2, the enhancement of CCS to aggregate the information for 5G VN Group is out of SA5 standard scope. PDU session charging information may be extended for CCS aggregating the 5G VN Group information.

### 5.4.6 	Conclusion

It is concluded to that the solution #4.2 is preferred which is out of SA5.

End of changes