3GPP TSG-SA5 Meeting #141-e 	S5-221124

- e-meeting, 17th – 26th Janaury 2022													revision of S5-21xabc

- Source:	Huawei Technologies

- Title:	pCR TR 28.827 Clarification on Roaming Charging Profile

- Document for:	Approval

Agenda Item:	7.5.4

# 1	Discussion

This is a pCR to TR 28.827 discussing about the roaming charging profile negotiation.

# 2	References

[1]	3GPP TR 28.827 "Telecommunication management; Charging management; 	Study on 5G charging for additional roaming scenarios and actors".

# 3	Rationale

This pCR proposes to discuss about the Roaming Charging Profile negotiation in TR 28.827.

# 4	Detailed proposal

Propose to incorporate the following change into the TR 28.827[1].

First change

#### 7.1.1.1	Use case #1a: Visited MNO charging home MNO for 5G data connectivity provided to the home MNO’s subscribers

This use case focuses on visited MNO and home MNO business roles.

A CSC (UE) has a subscription with the home MNO which allows usage of 5G data connectivity.

The visited MNO to home MNO charging could be based on data volume that the home MNO UE has used in the visited MNO’s network considering of the following aspects:

-	RAT type;

-	S-NSSAI;

-	DNN;

-	Rating Group

-	Service ID

-	Location

-	QoS information.

Editor’s Note: the list of the above aspects is ffs and should be check whether in the BCE.

The visited MNO to home MNO charging could be based on "Roaming Charging Profile", which may be negotiated between the VPLMN and the HPLMN.

This use case is applicable for the Home Routed Roaming and Local Breakout Roaming.

Next change

### 7.1.2	Potential charging requirements

REQ-CH_VMNO-01: The charging mechanism in visited MNO should support charging based on the 5G data connectivity usage for each home MNO.

REQ-CH_VMNO-02: The charging mechanism in visited MNO should support collecting charging information related to 5G data connectivity usage for each home MNO.

REQ-CH_VMNO-03: The charging mechanism in visited MNO should support charging based on the 5G connection and mobility usage for each home MNO.

REQ-CH_VMNO-04: The charging mechanism in visited MNO should support collecting charging information related to 5G connection and mobility usage for each home MNO.

REQ-CH_VMNO-05: The charging mechanism in visited MNO may support roaming charging profile negotiation related to 5G data connectivity charging with each home MNO.

End of change