3GPP TSG-SA5 Meeting #142e 	S5-222025rev1

- e-meeting, 4-12 April 2022					                                            Revision of S5-22xxxx

- Source:	Orange, AT&T, Telefonica

- Title:	New Key Issue Considering Containerized Network Functions

- Document for:	Approval

Agenda Item:	6.5.18

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	TR 28.913 0.0.0: Study on new aspects of Energy Efficiency (EE) for 5G Phase 2

# 3	Rationale

This contribution proposes a new key issue related to containerized network functions.

# 4	Detailed proposal

This document proposes the following changes in TR 28.913 [1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[Y]	ETSI GS NFV-EVE 004 V1.1.1 (2016-03): "Network Functions Virtualisation (NFV); Virtualisation Technologies; Report on the application of Different Virtualisation Technologies in the NFV Framework".

[Z]	ETSI GR NFV-IFA 029 V3.3.1 (2019-11): "Network Functions Virtualisation (NFV) Release 3; Architecture; Report on the Enhancements of the NFV architecture towards "Cloud-native" and "PaaS"".

Next Change

# X	Key Issues and potential solutions

## X.Y	Key Issue #X: Energy Consumption of Containerized Network Functions (CNF)

### X.Y.1	Description

The Rel-17 definition of the Energy Consumption (EC) of VNF/VNFCs (see TS 28.554 [X] – clauses 6.7.3.1.2 and 6.7.3.1.3) is valid for VM-based VNFs, i.e. when VNF/VNFC(s) are implemented on Virtual Machine(s) (VM).

ETSI ISG NFV started considering that VNF/VNFCs can be implemented using OS container technology (see e.g. [Y] and [Z]).

This key issue aims at investigating on potential definition(s) of EC for Containerized Network Functions (CNF).

### X.Y.2	Potential solutions

#### X.Y.2.i	Potential solution #<i>: <Potential Solution i Title>

##### X.Y.2.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

##### X.Y.2.i.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made.

End of change