3GPP TSG-SA5 Meeting #142-e 	S5-222031

e-meeting, 04 - 12 April 2022

- Source:	Nokia

- Title:	Update intent fulfillment tracking in information model

- Document for:	Approval

- Agenda Item:	6.6.3

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v1.0.0”.

# 3	Rationale

This contribution proposes to update the intent information model in clause 6.2.1 to support an elaborated tracking of the fulfillment of the different aspects of the intent . It proposes

- to expand filfillmentStatus to a fulfilmentinfo datatype which includes both the status as well as the reason behind or related to a particular status.

- to elaborate the fulfilment info such that it can eb provided for the whole intent, any of its expectations of even any of the specific targets in any one expectation.

- to expand the possible status values to includes the cases where the producer suspects an intent as well as the transient case for when the producer is still working on fulfilling the intent, expectation or target.

  - Relatedly, it proposes to remove the editor's note for the intentFulfilStatus attribute definition

# 4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

#### 6.2.1.2	Class definition

##### 6.2.1.2.1 	Intent <<IOC>>

6.2.1.2.1.1	Definition

This IOC represents the properties of an Intent driven management information between MnS consumer and MnS producer.

The Intent IOC contains one or multiple IntentExpectation(s) which includes MnS consumer’s requirements, goals and contexts given to a 3GPP system.

The Intent IOC includes the attribute objectClass and objectInstance from the TOP IOC. The value of attribute objectClass is “Intent” and the value of attribute objectInstance is the DN of the instance of Intent IOC.

6.2.1.2.1.2	Attributes

The Intent IOC includes attributes inherited from TOP IOC (defined in TS 28.622) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intentExpectations | M | T | T | F | F |
| userLabel | M | T | T | F | F |
| intentContexts | O | T | T | F | F |
| intentFulfillmentinfo | M | T | F | F | T |

6.2.1.2.1.3	Attribute constraints

None.

##### 6.2.1.2.3	IntentReport

6.2.1.2.3.1	Definition

IntentReport class represent intent fulfilment feedback information that MnS consumer can obtained from a 3gpp system.

Editor's Note: more description for IntentReport will be added later based on the further discussion.

6.2.1.2.3.2	Attributes

TBD

6.2.1.2.3.3	Attribute constraints

TBD

2nd Change

#### 6.2.1.3	DataType definition

- 6.2.1.3.1	IntentExpectation <<dataType>>

- 6.2.1.3.1.1	Definition

IntentExpectation <<dataType>>represents MnS consumer’s requirements, goals and contexts given to a 3GPP system. The information of IntentExpectation is generated by MnS consumer and delivered to MnS producer.

- 6.2.1.3.1.2	Attributes

The IntentExpectation includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| expectationId | M | T | T | T | F |
| expectationObject | O | T | T | F | F |
| expectationTargets | M | T | T | F | F |
| expectationContexts | O | T | T | F | F |
| expectationfulfilmentInfo | O | T | F | F | T |

Note: The scenario/requirements-specific IntentExpectations are defined utilizing the constructs of this generic IntentExpectation model.

- 6.2.1.3.1.3	Attribute constraints

TBD

- 6.2.1.3.3	ExpectationObject <<dataType>>

- 6.2.1.3.3.1	Definition

The ExpectationObject <<dataType>> represents the Object of the IntentExpectation that are required to be applied on.

- 6.2.1.3.3.2	Attributes

The ExpectationObject includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ObjectType | CM | T | T | F | F |
| ObjectInstance | CM | T | T | F | F |
| ObjectContexts | O | T | T | F | F |

- 6.2.1.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| ObjectType Support Qualifier | Condition: The intent expectation is not for a specific object instance or MnS consumer have no knowledge of the DN of this specific object instance. |
| ObjectInstance Support Qualifier | Condition: The intent expectation is for a specific object instance and MnS consumer have the knowledge of the DN of this specific object instance |

- 6.2.1.3.4 	ExpectationTarget <<dataType>>

- 6.2.1.3.4.1	Definition

The ExpectationTarget  <<dataType>> represents the targets of the IntentExpectation that are required to be achieved.

- 6.2.1.3.4.2	Attributes

The ExpectationTarget includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetName | M | T | T | F | F |
| targetCondition | M | T | T | F | F |
| targetValueRange | M | T | T | F | F |
| targetContexts | O | T | T | F | F |
| targetfulfillmentInfo | O | T | F | F | T |

- 6.2.1.3.4.3	Attribute constraints

None

- 6.2.1.3.5 	Context <<dataType>>

- 6.2.1.3.5.1	Definition

The Context <<dataType>> represents the properties of a context. A context describes the list of constraints and conditions that should evaluate to true when the targets are fulfilled but are themselves not to be enforced. The context may apply to the intent, the intent expectation, the intent targets or to the object.

- 6.2.1.3.5.2	Attributes

The Context includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| contextAttribute | M | T | T | F | F |
| contextCondition | M | T | T | F | F |
| contextValueRange | M | T | T | F | F |

- 6.2.1.3.5.3	Attribute constraints

None

- 6.2.1.3.n 	FulfillmentInfo << dataType >>

- 6.2.1.3.n.1	Definition

This dataType represents the properties of a specific fulfilment information for an aspect of the intent (i.e. either an expectation, a target or the whole intent). The fulfilment information describes the producer's assessment of the degree to which a specific aspect of the intent has been fulfilled. The consumer may however assess the fulfilment differently e.g., the consumer may evaluate the delivered outcome or network state to compute its fulfilment satisfaction.

The fulfillmentStatus field indicates whether the intent is fulfilled or not fulfilled. The possible values of the fulfilment include:

- NOTFULFILLED: This is the default status for any aspect of the intent and the fulfillmentStatus remains as " NOTFULFILLED " until the producer is satisfied that the undertaken actions meets the requirements as staed by the consumer.

- FULFILLED: This is the status if the producer considers that the intent, expectation or target has been fulfilled as desired by the consumer that created the intent. The consumer may provide a fulfilment satisfaction report that either confirms the fulfilment or describes its evaluation the fulfilment.

The degree of fulfilment of an intent with the NOTFULFILLED status may have multiple explanations and related states. These different progress states and conditions are recorded in the notFullfilledState field. The possible values of the notFullfilledState include:

- ACKNOWLEDGED: this is the default status and is the initial notFullfilledState right after the intent has been received.

- COMPLIANT: this is the state after the feasibility check has been run for the intent and the intent accepted as being compliant for fulfillment

- DEGRADED: this is the state if an intent that was previously fulfilled is found not be meeting al the initially stated requirements.

- SUSPENDED: this is the state if the producer decides to suspect the fulfilment of the intent, expectation or target for whatever reason. This is notFullfilledState must be supported by a reason such as the event(s) that were observed when fulfilment was attempted.

- "TERMINATED": This state is registered if the respective aspect of the intent (i.e. either an expectation, a target or the whole intent) shall not be considered for fulfilment e.g. when an authorized consumer sends an indication terminating the specific aspect of the intent. For instance, if the consumer sends an update of the intent in which a particular target is eliminated, then that target shall be marked as cancelled.

- FULFILLMENTFAILED: This is the state when the producer decides that the intent, expectation or target cannot be fulfilled. This is state must be supported by a reason such as the event(s) that were observed when fulfilment was attempted.

For some scenarios (in particular for the "suspended" and the "fulfillmentFailed" notFullfilledStates ), the notFullfilledState must be supported by extra information describing or related to the state. This extra information is recorded into the notFullfilledReasons field.

- 6.2.1.3.n.2	Attributes

The fulfillmentInfo includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fulfillmentStatus | M | T | F | F | T |
| notFullfilledState | CM | T | F | F | T |
| notFullfilledReasons | CO | T | F | F | T |

- 6.2.1.3.n.3	Attribute constraints

##### None

- 6.2.1.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| notFullfilledState Support Qualifier | Condition: when FulfillmentInfo is implemented for IntentFulfillmentInfo |
| notFullfilledReasons Support Qualifier | Condition: when FulfillmentInfo is implemented for IntentFulfillmentInfo |

3rd Change

#### 6.2.1.4	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |  |
| --- | --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: None isNullable: False |  |
| intentExpectations | It describes the expectations including requirements, goals and contexts (including constraints and filter information) given to a 3GPP system. It states the list of specific outcomes desired to be realized for expectation object(s). allowedValues: Not Applicable | type: IntentExpectation multiplicity: 1..* isOrdered: F isUnique: F defaultValue: None isNullable: False |  |
| intentFulfillmentInfo | It describes status of fulfilment of an intent and the related reasons for that status. It can be applied to the, the or allowedValues: Not Applicable |  | type: fulfillmentInfo multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectationFulfillmentInfo | It describes status of fulfilment of an intentExpectation and the related reasons for that status. allowedValues: Not Applicable |  | type: fulfillmentInfo multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| targetFulfillmentInfo | It describes status of fulfilment of an expectationTarget and the related reasons for that status. allowedValues: Not Applicable |  | type: fulfillmentInfo multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| intentFulfilStatus | It describes the current status of fulfilment for the intent, intentexpectation or expectationTarget It is configured/written by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", “NOT_FULFILLED | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| notFullfilledState | It describes the current progress of or the reason for not achieving fulfilment for the intent, intentExpectation or expectationTarget. It is configured/written by MnS producer and can be read by MnS consumer. allowedValues: " ACKNOWLEDGED", "COMPLIANT", "DEGRADED", "SUSPENDED", "TERMINATED" "FULFILLMENTFAILED", |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notFullfilledReason | It describes the reasons/observations related to the specific noted notFullfilledState allowedValues: Not Applicable |  | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentContexts | It describes the list of IntentContext(s) which represents the constraints and conditions that should apply for the entire intent even if there may be specific contexts defined for specific parts of the intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: * isOrdered: False isUnique: False defaultValue: None isNullable: False |  |
| expectationId | A user-friendly (and user assignable) name of the intentExpectation. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |  |
| expectationObject | It describes the expectation objects of the IntentExpectation that are required to be applied on. allowedValues: Not Applicable | type: ExpectationObject multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |  |
| objectType | It describes the type of expectation object of the IntentExpectation that are required to be applied on. It can be class name of the managed object. allowedValues: see scenario specific Intent Expectation | type: Enum multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |  |
| objectInstance | It describes an specific object instance (e.g. instance of managed object) to which the intentExpectation should apply. allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |  |
| objectContexts | It describes the list of ObjectContext(s) which represents the constraints and conditions to be used as filter information to identify the object(s) to which a given intentExpectation should apply. Note there may be other constraints and conditions defined either for the entire intent, for the specific intentExpectation or for the expectationTarget of the considered intentExpectation. The concrete ObjectContext depends on the ExpectationObject, which is defined in clause 6.4. All the concrete ObjectContexts follow the common structure of ObjectContext | type: Context multiplicity: * isOrdered: False isUnique: False defaultValue: None isNullable: False |  |
| expectionTargets | It describes the list of ExpectationTarget(s) which represent specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. The concrete ExpectationTarget depends on the ExpectationObject, which is defined in clause 6.4. All the concrete ExpectationTargets follow the common structure of ExpectationTarget | type: ExpectationTarget multiplicity: 1..* isOrdered: False isUnique: False defaultValue: None isNullable: False |  |
| expectationContexts | It describes the list of context(s) which represents the constraints and conditions that should apply for a specific intentExpectation. Note there may be other constraints and conditions defined for the entire intent or for specific parts of the intentExpectation. allowedValues: depends on Expectation Object in the IntentExpectation | type: Context multiplicity: 1..* isOrdered: False isUnique: False defaultValue: None isNullable: False |  |
| targetName | It describes the name of the Expectation of the expectation target which represents specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation.allowedValues: depends on ExpectationObject in the IntentExpectation | type: String multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |  |
| targetCondition | It expresses the limits within which the targetName is allowed/supposed to be allowedValues: is equal to; is less than; is greater than: "is within the range" ; "is outside the range" c | type: Enum multiplicity: 1 isOrdered: False isUnique: False defaultValue: "is equal to" isNullable: False |  |
| targetValueRange | It describes the range of values that applicable to the targetName and the TargetCondition. allowedValues: depends on the targetName | type: Real multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |  |
| targetContexts | It describes the list of constraints and conditions that should apply for a specific expectationTarget. Note there may be other constraints and conditions defined for the entire intent or the intentExpectation. allowedValues: Not Applicable | type: Context multiplicity: 1..* isOrdered: False isUnique: False defaultValue: None isNullable: False |  |
| contextAttribute | It describes a specific attribute of or related to the object or to characteristics thereof (e.g. its control parameter, gauge, counter, KPI, weighted metric, etc) to which the expectation should apply or an attribute related to the operating conditions of the object (such as weather conditions, load conditions, etc). | type: String multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |  |
| contextCondition | It expresses the limits within which the ContextAttribute is allowed/supposed to be allowedValues: is equal to; is less than; is greater than; "is within the range" ;"is outside the range" . | type: Enum multiplicity: upto 1 isOrdered: False isUnique: False defaultValue: "is equal to" isNullable: False |  |
| contextValueRange | It describes the range of values that applicable to the ContextAttribute and the ContextCondition. AllowedValue: depends on the contextAttribute | type: Real multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |  |

End of changes