3GPP TSG-SA5 Meeting #143-e 	S5-223135rev1

# e-meeting, 9 - 17 May 2022

# Source:	China Mobile

# Title:	Rel-18 pCR 28.828 Add charging scenarios and key issues for PNI-NPN on converged charging for access connection

# Document for:	Approval

Agenda Item:	7.5.4

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.828: "Charging management; Study on charging aspects for enhanced support of non-public networks".

3	Rationale

This pCR proposes to add charging scenarios and key issues for PNI-NPN on converged charging for access connection to TR 28.828 [1].

4	Detailed proposal

The following changes are proposed to be incorporated into TR 28.828.

First change

## 6.x	Topic X: Converged charging for access connection in PNI-NPN

### 6.x.1	Use cases

6.x.1.1	Use case #Xa: Network access control

Relying on PLMN network, PNI-NPN provides logically independent 5G non-public network for vertical industry customers through network slice instances/ dedicated DNNs, and supporting of CAG feature. CAG is used for access control to prevent unauthorized UEs from trying to access a PNI-NPN.

The NPN-OP provides the PNI-NPN (e.g. vertical industry customers) to NPN-SP, and NPN-SP provides NPN services to NPN-SC. It is important for NPN-OP to collect usage for PNI-NPN resources associated to access connection for the purpose of charging or statistics.

The potential charging requirements for this UC are: REQ-eNPN_CH_PNI-NPN_NAC-01, REQ-eNPN_CH_PNI-NPN_NAC-02.

### 6.x.2	Potential charging requirements

REQ-eNPN_CH_PNI-NPN_NAC-01: The 5G system should support converged charging per UE for PNI-NPN network access and usage.

REQ-eNPN_CH_PNI-NPN_NAC-02: The 5G system should support converged charging per vertical industry consumer for PNI-NPN network access and usage.

### 6.x.3	Key issues

This key issue is for investigating how to support converged charging for access connection in PNI-NPN considering REQ-eNPN_CH_PNI-NPN_NAC-01 and REQ-eNPN_CH_PNI-NPN_NAC-02. This investigation covers the following:

-	Key Issue #xa: Identification of the charging information for converged charging for PNI-NPN network access and usage;

-	Key Issue #xb: Identification of the main interactions with the NFs to obtain the charging information;

-	Key Issue #xc: The charging mechanism supporting converged inter-provider charging for PNI-NPN network access and usage.

6.x.4	Possible solutions

TBD

6.x.5	Evaluation

TBD

6.x.6	Conclusion

TBD

End of changes