3GPP TSG-SA5 Meeting #143-e 	S5-223655

# Online, , 9th May 2022 - 17th May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0115 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 TS 28.531 Update operations of deallocateNsi, deallocateNssi |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2022-05-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Return in deallocateNsi/deallocateNssi is not correct |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix incorrect return in deallocateNsi and deallocateNssi. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect/incomplete standards is not implementable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.3, 9.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

Next Change

Next Change

Next Change

#### 9.1.1.3	Operation deallocateNsi

This operation deallocate a service profile in an NSI. The provider may terminate the requested NSI or modify the requested NSI without termination to satisfy the request.

Table 9.1.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| networkSliceDN | request body | n/a | Resource | M |
| serviceProfileId | Request body | n/a | Resource | M |

Table 9.1.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |

The message flow for deallocation is as follows:

1.	The MnS consumer sends a HTTP DELETE request to the MnS producer.

- The target URI is equal to the concatenation of URI of the parent resource and the resource (in this case ServiceProfile) to be deleted.

- The message body shall contain the networkSliceDN identifying the NetworkSlice MOI.

2.	The MnS producer sends a HTTP DELETE response to the MnS consumer.

- On success of terminating the NetworkSlice Instance, "204 No content" shall be returned.

- On success of modification of the NetworkSlice Instance, "212 Resource Not Deleted" shall be returned.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

Next Change

Next Change

#### 9.2.1.3	Operation deallocateNssi

This operation deallocate a slice profile in an NSSI. The provider may terminate the requested NSSI or modify the requested NSSI without termination to satisfy the request.

Table 9.2.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| networkSliceSubnetDN | request body | n/a | Resource | M |
| sliceProfileId | request body | n/a | Resource | M |

Table 9.2.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |

The message flow for deallocation is as follows:

1.	The MnS consumer sends a HTTP DELETE request to the MnS producer.

- The message body shall contain the networkSliceSubnetDN identifying the NetworkSliceSubnet MOI.

2.	The MnS producer sends a HTTP DELETE response to the MnS consumer.

- On success of terminating the NetworkSliceSubnet Instance, "204 No content" shall be returned.

- On success of modification of the NetworkSliceSubnet Instance, "212 Resource Not Deleted" shall be returned.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

End of Changes