3GPP TSG-SA5 Meeting #144-e 	S5-224051

- e-meeting, 27 June - 1 July 2022

- Source:	Huawei, Deutsche Telekom

- Title:	Add issue for intent report

- Document for:	Approval

Agenda Item:	6.7.3.2

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TR 28.912: “Management and orchestration; Study on enhanced intent driven management services for mobile networks v0.2.0”.

[2]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks"

# 3	Rationale

This contribution proposes to discuss the issue for intent report to address the Note " Note: The model for IntentReport is not addressed in the present document." in TS 28.312 [2]. The intent report can contain the current and optional predicted value for performance indicated by corresponding expectation targets (e.g. WeakRSRPRatio for the weakRSRPRatioTarget, Average UL RAN UE Throughput for aveULRANUEThptTarget), which can be used by MnS consumer to validate whether the intent is really fulfilled and to evaluate whether the intent (especially for expectation targets) needs to be updated (improve the target value when corresponding target is fulfilled or reduce the target value when corresponding target is not fulfilled or not fulfilled with the reason of target confliction).

# 4	Detailed proposal

1st Change

## 4.X	Issue#4.X: Intent Report

### 4.X.1	Description

In TS 28.312 [2], the Fulfilment information (including the intentFulfilmentinfo, expectationfulfilmentInfo and targetfulfilmentInfo) are defined for the MnS consumer obtain the intent fulfilment information. However, the intent report haven't been defined. The intent report can contain the current and optional predicted value for performance indicated by corresponding expectation targets (e.g. WeakRSRPRatio for the weakRSRPRatioTarget, Average UL RAN UE Throughput for aveULRANUEThptTarget), which can be used by MnS consumer to validate whether the intent is really fulfilled and to evaluate whether the intent (especially for expectation targets) needs to be updated if needed (improve the target value when corresponding target is fulfilled or reduce the target value when corresponding target is not fulfilled or not fulfilled with the reason of target confliction).

Different MnS consumer may have different requirements for intent report (e.g. Some MnS consumer may want to have corresponding performance value information while others don't want, Different MnS consumer may want to calculate or monitor the performance value in different period).

REQ-Intent_Driven_MnS: The intent driven MnS should have capability enabling MnS consumer to request intent report information.

REQ-Intent_Driven_MnS: The intent driven MnS shouldl have capability enabling MnS consumer to obtain intent report information with current performance value for corresponding expectation targets.

### 4.X.2	Potential solutions

#### 4.X.2.1	Potential solution#1

Following are the proposed solution based on intent information model defined in TS 28.312 [2].

- Extend the IntentExpectation <<dataType>> with the following attribute:

- - "reportIndicator", represents MnS consumer's requirements for intent Report, which is a Boolean value. If the value is TRUE, MnS producer needs to create an instance of IntentReport IOC to capture the current performance values for corresponding Expectation Targets.

- "observationTime", represents MnS consumer's requirements for the observation period of performance value for corresponding Expectation Targets.  The performance value for corresponding Expectation Targets will be observed from the start of each observation period, then at the end of each observation period, the value will be derived and configured

- Introduce the IntentReport <<IOC>>, represents the performance values for the Expectation Targets in the associated Intent.  Then The MnS consumer can use the "getMOIAttributeValue" operation to query the IntentReport <<IOC>> to obtain the intent report information. The IntentReport <<IOC>> include the following attributes:

- "DN of the Intent <<IOC>>", indicates the associated instance of Intent <<IOC>>

- "TargetMetricInfoList", which is the list of [targetMetricName, currentValue]. The targetMetricName is same as ExpectationTarget Name in the associated Intent <<IOC>.

#### 4.X.2.2	Potential solution#2

Following are the proposed solution based on intent information model defined in TS 28.312 [2].

- Extend the IntentExpectation <<dataType>> with the following attribute:

- "observationTime", represents MnS consumer's requirements for the observation period of performance value for corresponding Expectation Targets.  The performance value for corresponding Expectation Targets will be observed from the start of each observation period, then at the end of each observation period, the value will be derived and configured

- Introduce the "achieveValue" attribute in the ExpectationTarget to represent the performance value for the ExpectationTarget.  Then The MnS consumer can use the "getMOIAttributeValue" operation to query the Intent <<IOC>> to obtain the achieveValue for a given ExpectationTarget.

End of changes