3GPP TSG-SA5 Meeting #144-e 	S5-224088

- e-meeting, June 27th – July 1st, 2022

- Source:	Nokia, Nokia Shanghai Bell

- Title:	DP on Modelling AIML Entity

- Document for:		Discussion and Endorsement

Agenda Item:	6.7.5.1

# 1	Decision/action requested

Please discuss the presented rationale and endorse the proposals

# 2 	References

[1]	3GPP TR 28.105: “Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management”

# 3 	Rationale

## 3.1 	Introduction

Telecommunication industry including the mobile networks are evaluating the application of various AIML techniques to efficiently solve the issues pertaining to the industry. Following the suit, the management of AIML is being discussed and specified in TS 28.105 [1].

Classical AIML process advocates various stages of activity such as data collection, model training, model testing and model deployment. TS 28.105 [1] plan to specify all the aspects discussed above except data collection. It is necessary that the specification of all these stages of AIML process are seamlessly integrated with the existing management system. This will help the existing management systems to implement, integrate and use the AIML techniques specified to improve the performance of the management systems.

Observation 1

In the context of SBMA, the different stages of the AIML process including the model training, model testing and model deployment could be requested or performed by independent MnS consumers or the producers. The primary output of AIML activity, an AIML model or an AIML entity, shall be able to get referenced by the management systems to produce or consume different stages of AIML process.

For instance, the model training could be performed by a MnS entity A and the testing of the AIML entity generated by the MnS entity A could be performed by a different MnS entity B. The inference using the AIML entity produced by MnS entity A shall be used by a different MnS entity C. Currently, the IE aIMLEntityId is defined as a string with a value unique in each MnS producer.  This approach is not accommodating the fact that any MnS entity could independently perform the actions such as training, testing and inference.

An independent inference function would not have access to the aIMLEntityId which is an attribute of the AIMLTrainingFunction MOI.

Observation 2

In case, an existing AIML model is retrained, it is not clear whether the same or different aIMLEntityId shall be used. If the same ID is used, it is not possible for a consumer to refer to older models for inference. In case of new ID being used for a retrained model, it is not clear on how to specify the relationship between the older and the newer model. It is also not clear on how a consumer would be informed about the availability of different models for the same use case.

Recommendation

The aIMLEntityId shall be modelled using an IOC. A MOI shall be created for each model (new or retrained model). Since the AIML model or AIML entity is modelled using an IOC, each MOI shall be referred with a DN which is unique across the 3GPP management system. Independent and standalone functions performing inferences shall refer to the DN of the required model for requesting the inference. Relationships of different models for the same use case can also be modelled using the attributes of this IOC.

# 4	Detailed Proposal

This section details the proposed changes in TS 28.105 [1].

First changes

## 7.2	Class diagram

### 7.2.1	Relationships

This clause depicts the set of classes (e.g., IOCs) that encapsulates the information relevant to AI/ML model training. For the UML semantics, see 3GPP TS 32.156 [13].

Figure 7.2.1-1: NRM fragment for AI/ML model training

### 7.2.2	Inheritance

Figure 7.2.2-1: Inheritance Hierarchy for AI/ML model training related NRMs

## 7.3	Class definitions

### 7.3.1	AIMLTrainingFunction

#### 7.3.1.1	Definition

The IOC AIMLTrainingFunction represents the entity that undertakes AI/ML training and is also the container of the AIMLTrainingRequest IOC(s).

The entity represented by AIMLTrainingFunction MOI supports training of one or more AIMLEntity(s).

#### 7.3.1.2	Attributes

The NetworkSlice IOC includes attributes inherited from Top IOC (defined in TS 28.622[12]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |
| aIMLEntityRefList | M | T | F | F | F |

#### 7.3.1.3	Attribute constraints

None.

#### 7.3.1.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

- 7.3.2	AIMLTrainingRequest

- 7.3.2.1	Definition

The IOC AIMLTrainingRequest represents the AI/ML model training request that is created by the AI/ML training MnS consumer.

The AIMLTrainingRequest MOI is contained under one AIMLTrainingFunction MOI. Each AIMLTrainingRequest is associated to at least one AIMLEntity.

The AIMLTrainingRequest may have a source to identify where it is coming from, and which may be used to prioritize the training resources for different sources. The sources may be for example the network functions, operator roles, or other functional differentiations.

Each AIMLTrainingRequest may indicate the expectedRunTimeContext that describes the specific conditions for which the AIMLEntity (either AIML Model or AIML-enabled function) should be trained for.

In case the request is accepted, the AI/ML training MnS producer decides when to start the AI/ML training. Once the MnS producer decides to start the training based on the request, the AI/ML training MnS producer instantiates one or more AI/MLTrainingProcess MOI(s) that are responsible to perform the followings:

-	collects (more) data for training, if the training data are not available or the data are available but not sufficient for the training,

-	prepares and selects the training data, with consideration of the consumer provided candidate training data if any. The AI/ML training MnS producer may examine the consumer's provided candidate training data and select none, some or all of them for training. In addition, the AI/ML training MnS producer may select some other training data that are available,

-	trains the AIMLEntity using the selected and prepared training data.

The AIMLTrainingRequest may have a requestStatus field to represent the status of the specific AIMLTrainingRequest:

-	The attribute values are "NOT_STARTED", "TRAINING_IN_PROGRESS", "SUSPENDED", "FINISHED", and "CANCELLED".

-	When value turns to "TrainingInProcess", the AI/ML training MnS producer instantiates one or more AIMLTrainingProcess MOI(s) representing the training process(es) being performed per the request and notifies the MnS consumer(s) who subscribed to the notification.

When all of the training process associated to this request are completed, the value turns to "FINISHED.

- 7.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| candidateTraingDataSource | O | T | T | F | T |
| traingDataQualityScore | O | T | T | F | T |
| trainingRequestSource | M | T | T | F | T |
| requestStatus | M | T | T | F | T |
| expectedRuntimeContext | O | T | T | F | T |
| performanceRequirements | M | T | T | F | T |
| cancelRequest | O | T | T | F | T |
| suspendRequest | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| aIMLEntityRef | M | T | T | F | T |

- 7.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |

- 7.3.2.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

- 7.3.4	AIMLTrainingReport

- 7.3.4.1	Definition

The IOC AIMLTrainingReport represents the AI/ML model training report that is provided by the training MnS producer.

The AIMLTrainingReport MOI is contained under one AIMLTrainingFunction MOI.

- 7.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| areConsumerTrainingDataUsed | M | T | F | F | T |
| usedConsumerTrainingData | CM | T | F | F | T |
| confidenceIndication | O | T | F | F | T |
| modelPerformanceTraining | CM | T | F | F | T |
| areNewTrainingDataUsed | CO | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| trainingRequestRef | CM | T | F | F | T |
| trainingProcessRef | M | T | F | F | T |
| lastTrainingRef | CM | T | F | F | T |
| aIMLEntityRef | M | T | F | F | T |

- 7.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| usedConsumerTrainingData Support Qualifier | Condition: The value of areConsumerTrainingDataUsed attribute is ALL or PARTIALLY. |
| trainingRequestRef Support Qualifier | Condition: The AIMLTrainingReport MOI represents the report for the AI/ML model training that was requested by the MnS consumer (via AIMLTrainingRequest MOI). |
| lastTrainingRef Support Qualifier | Condition: The AIMLTrainingReport MOI represents the report for the AI/ML model training that was not initial training (i.e., the model has been trained before). |
| modelPerformanceTraining | The condition is use case " AI/ML training initiated by consumer" is supported. |
| areNewTrainingDataUsed | The condition is use case " AI/ML training initiated by producer" is supported. |

- 7.3.4.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

### 7.3.5	AIMLTrainingProcess

#### 7.3.5.1	Definition

The IOC AIMLTrainingProcess represents the AI/ML training process.

One AIMLTrainingProcess MOI may be instantiated for each AIMLTrainingRequest MOI or a set of AIMLTrainingRequest MOIs.

For each AIMLEntity under training, a AIMLTrainingProcess is instantiated, i.e., an AIMLTrainingProcess is associated with exactly one AIMLEntity. The AIMLTrainingProcess may be associated with one or more AIMLTrainingRequest MOI.

The AIMLTrainingProcess does not have to correspond to a specific AIMLTrainingRequest, i.e., a AIMLTrainingRequest does not have to be associated to a specific AIMLTrainingProcess. The AIMLTrainingProcess may be managed separately from the AIMLTrainingRequest MOIs, e.g., the AIMLTrainingRequest MOI may come from consumers which are network functions while the operator may wish to manage the AIMLTrainingProcess that is instantiated following the requests. Thus, the AIMLTrainingProcess may be associated to either one or more AIMLTrainingRequest MOI.

Each AIMLTrainingProcess instance needs to be managed differently from the related AIMLEntity, although the AIMLTrainingProcess may be associated to only one AIMLEntity. For example, the AIMLTrainingProcess may be triggered to start with a specific version of the AIMLEntity and multiple AIMLTrainingProcess instances may be triggered for different versions of the AIMLEntity. In either case the AIMLTrainingProcesse instances are still associated with the same AIMLEntity but are managed separately from the AIMLEntity.

Each AIMLTrainingProcess has a priority that may be used to prioritize the execution of different AIMLTrainingProcesse instances. By default, the priority of the AIMLTrainingProcess may be related in a 1:1 manner with the priority of the AIMLTrainingRequest for which the AIMLTrainingProcess is instantiated.

Each AIMLTrainingProcess may have one or more termination conditions used to define the points at which the AIMLTrainingProcess may terminate.

The "ProgressStatus" attribute represents the status of the AI/ML model training and includes information the MnS consumer can use to monitor the progress and results. The data type of this attribute is "ProcessMonitor" (see TS 28.622 [11]). The following specialisations are provided for this data type for the AI/ML training process:

The "ProgressStatus" attribute represents the status of the AI/ML model training and includes information the AI/ML training MnS consumer can use to monitor the progress and results. The data type of this attribute is "ProcessMonitor" (see TS 28.622 [12]). The following specialisations are provided for this data type for the AI/ML training process:

-	The "status" attribute values are "RUNNING", "CANCELLING", "SUSPENDED", "FINISHED", and "CANCELLED". The other values are not used;

-	The "timer" attribute is not used;

-	When the "status" is equal to "RUNNING" the "progressStateInfo" attribute shall indicate one of the following states: "COLLECTING_DATA", "PREPARING_TRAINING_DATA", "TRAINING".

-	No specifications are provided for the "resultStateInfo" attribute. Vendor specific information may be provided though.

When the training is completed with "status" equal to "FINISHED", the MnS producer provides the training report, by creating an AIMLTrainingReport MOI, to the MnS consumer.

7.3.5.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLTrainingProcessId | M | T | T | F | T |
| priority | M | T | T | F | T |
| terminationConditions | M | T | T | F | T |
| progressStatus | M | T | F | F | T |
| cancelProcess | O | T | T | F | T |
| suspendProcess | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| trainingRequestRef | CM | T | F | F | T |
| trainingReportRef | M | T | F | F | T |
| aIMLEntityRef | M | T | T | F | T |

#### 7.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| trainingRequestRef Support Qualifier | Condition: The AIMLTrainingReport MOI represents the report for the AI/ML model training that was requested by the training MnS consumer (via AIMLTrainingRequest MOI). |

#### 7.3.5.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

### 7.3.x	AIMLEntity

#### 7.3.x.1	Definition

This IOC represents an AI/ML entity which could be either an AI/ML model or AI/ML-enabled function containing the AI/ML model. AIML training may be requested for either an AI/ML model or AI/ML-enabled function. The algorithm of AI/ML model or AI/ML-enabled function is not to be standardized.

For each AIMLEntity under training, one or more AIMLTrainingProcess are instantiated.

The AIMLEntity may contain 3 types of contexts - TrainingContext which is the context under which the AIMLEntity has been trained, the ExpectedRunTimeContext which is the context where an AIMLEntity is expected to be applied or/and the RunTimeContext which is the context where the model is being applied.

#### 7.3.x.2	Attributes

The AIMLEntity IOC includes attributes inherited from Top IOC (defined in TS 28.622[12]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceType | M | T | F | F | T |
| aIMLEntityVersion | M | T | F | F | T |
| expectedRunTimeContext | O | T | T | F | T |
| trainingContext | CM | T | F | F | T |
| runTimeContext | O | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| trainingProcessRef | M | T | F | F | T |
| trainingRequestRef | CM | T | F | F | T |
| trainingReportRef | M | T | F | F | T |

#### 7.3.x.3	Attribute constraints

None.

| Name | Definition |
| --- | --- |
| trainingContext Support Qualifier | Condition: The trainingContext represents the status and conditions related to training and should be added when training is completed |

#### 7.3.x.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

Next changes

Next changes

## 7.5	Attribute definitions

### 7.5.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| aIMLEntityRef | It is the DN of AIMLEntity MOI. allowedValues: N/A. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| candidateTraingDataSource | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| inferenceType | It indicates the type of inference that the AI/ML model supports. allowedValues: the values of the MDA type (see TS 28.104 [2]), Analytics ID(s) of NWDAF (see TS 23.288 [3]), types of inference for RAN-intelligence, and vendor's specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| areConsumerTrainingDataUsed | It indicates whether the consumer provided training data have been used for the AI/ML model training. allowedValues: ALL, PARTIALLY, NONE. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| usedConsumerTrainingData | It provides the address(es) where lists of the consumer-provided training data are located, which have been used for the AI/ML model training. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingRequestRef | It is the DN(s) of the related AIMLTrainingRequest MOI(s). allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingReportRef | It is the DN of the AIMLTrainingReport MOI that represents the reports of the AI/ML training. allowedValues: DN. | type: DN (see TS 32.156 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lastTrainingRef | It is the DN of the AIMLTrainingReport MOI that represents the reports for the last training of the AI/ML model. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| confidenceIndication | It indicates the confidence (in unit of percentage) that the AI/ML model would perform for inference on the data with the same distribution as training data. allowedValues: { 0..100 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aIMLEntityList | It describes the list of AIMLEntity. | type: AIMLEntity multiplicity: * isOrdered: False isUnique: N/True defaultValue: None isNullable: False |
| trainingRequestSource | It describes the entity that requested to instantiate the AIMLTrainingRequest MOI. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| requestStatus | It describes the status of a particular AI/ML training request. T. allowedValues: NOT_STARTED, TRAINING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aIMLTrainingProcessId | It identifies the training process. It is unique in each instantiated process in the MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| priority | It indicates the priority of the training process. The priority may be used by the AI/ML training to schedule the training processes. Lower value indicates a higher priority. allowedValues: { 0..65535 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| terminationConditions | It indicates the conditions to be considered by the AIMLTraining to terminate a specific training process. Editor's Note: The specific nature of the termination conditions is FFS allowedValues: FFS. | type: FFS multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| progressStatus | It indicates the status of the AI/ML training process. allowedValues: N/A. | type: ProcessMonitor (see TS 28.622 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aIMLEntityVersion | It indicates the version number of the AI/ML entity. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceRequirements | It indicates the expected performance for a trained AI/ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| performanceTraining | It indicates the performance score of the AI/ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aIMLTrainingProcess.progressStatus.progressStateInfo | It provides the following specialisation for the "progressStateInfo" attribute of the "ProcessMonitor" data type for the "AIMLTrainingProcess". When the AI/ML training is in progress, and the "status" is equal to " RUNNING" it provides the more detailed progress information. allowedValues for "status" = "RUNNING": - COLLECTING_DATA - PREPARING_TRAINING_DATA - TRAINING The allowed values for "status" = "CANCELLED" are vendor specific. | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| inferenceOutputName | It indicates the name of an inference output of an AI/ML entity. allowedValues: the name of the MDA output IEs (see TS 28.104 [2]), name of analytics output IEs of NWDAF (see TS 23.288 [3]), RAN-intelligence inference output IE name(s), and vendor's specific extensions. | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceMetric | It indicates the performance metric used to evaluate the performance of an AI/ML entity, e.g., "accuracy", "precision", "F1 score", etc. allowedValues: N/A. | Type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| performanceScore | It indicates the performance score (in unit of percentage) of an AI/ML entity when performing inference on a specific data set (Note). The performance metrics may be different for different kinds of AI/ML models depending on the nature of the model. For instance, for numeric prediction, the metric may be accuracy; for classification, the metric may be a combination of precision and recall, like the "F1 score". allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cancelRequest | It indicates whether the AI/ML training MnS consumer cancels the AI/ML training request. Setting this attribute to "TRUE" cancels the AI/ML training request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " TRAINING_IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| suspendRequest | It indicates whether the AI/ML training MnS consumer suspends the AI/ML training request. Setting this attribute to "TRUE" suspends the AI/ML training request. Suspension is possible when the requestStatus is the not "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| cancelProcess | It indicates whether the AI/ML training MnS consumer cancels the AI/ML training process. Setting this attribute to "TRUE" cancels the AI/ML training request. Cancellation is possible when the progressStateInfo is the not "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| suspendProcess | It indicates whether the AI/ML training MnS consumer suspends the AI/ML training process. Setting this attribute to "TRUE" suspends the AI/ML training request. Suspension is possible when the progressStateInfo is the not "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| managedEntityRef | It describes the entities that the AI/ML entity is responsible for managing or optimizing. | Type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| dataProviderRef | It describes the entities that have provided or should provide data needed by the AI/ML entity say for training or inference | Type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| areNewTrainingDataUsed | It indicates whether the other new training data have been used for the AI/ML model training. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traingDataQualityScore | It indicates numerical value that represents the dependability/quality of a given observation and measurement type. The lowest value indicates the lowest level of dependability of the data, i.e., that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| decisionConfidenceScore | the numerical value that represents the dependability/quality of a given decision generated by the ML-based function. The lowest value indicates the lowest level of dependability of the decisions, i.e., that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aIMLEntityListRef | It is the DN of AIMLEntity MOI. allowedValues: N/A. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| NOTE: when the performanceScore is to indicate the performance score for AI/ML training, the data set is the training data set. |  |  |

Next changes

Annex A (informative): 
PlantUML source code for NRM class diagrams

# A.1	General

This annex contains the PlantUML source code for the NRM diagrams defined in clause 7.2 of the present document.

# A.2	PlantUML code for Figure 7.2.1-1: NRM fragment for AI/ML model training

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

skinparam nodesep 60

class ManagedEntity <<ProxyClass>>

class AIMLEntity <<InformationObjectClass>>

class AIMLTrainingFunction <<InformationObjectClass>>

class AIMLTrainingRequest <<InformationObjectClass>>

class AIMLTrainingReport <<InformationObjectClass>>

class AIMLTrainingProcess <<InformationObjectClass>>

ManagedEntity "1" *-- "*" AIMLTrainingFunction: <<names>>

AIMLTrainingFunction "1" -> "*" AIMLEntity

AIMLTrainingFunction "1" *-- "*" AIMLTrainingProcess: <<names>>

AIMLTrainingFunction "1" *-- "*" AIMLTrainingRequest: <<names>>

AIMLTrainingFunction "1" *-- "*" AIMLTrainingReport: <<names>>

AIMLTrainingProcess "1" <-r-> "1" AIMLTrainingReport

AIMLTrainingReport "1" --> "1" AIMLTrainingReport

AIMLTrainingRequest "*" -l-> "1" AIMLEntity

AIMLTrainingRequest "1..*" <-r-> "*" AIMLTrainingProcess

AIMLTrainingProcess "*" -l-> "1" AIMLEntity

AIMLTrainingReport "*" -l-> "1" AIMLEntity

note left of ManagedEntity

This represents the following IOCs:

Subnetwork or

ManagedFunction or

ManagementFunction

end note

@enduml

# A.3	PlantUML code for Figure 7.2.2-1: Inheritance Hierarchy for AI/ML model training related NRMs

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class ManagedFunction <<InformationObjectClass>>

class AIMLTrainingFunction <<InformationObjectClass>>

class AIMLTrainingRequest <<InformationObjectClass>>

class AIMLTrainingProcess <<InformationObjectClass>>

class AIMLTrainingReport <<InformationObjectClass>>

ManagedFunction <|-- AIMLTrainingFunction

Top <|-- AIMLTrainingRequest

Top <|-- AIMLTrainingProcess

Top <|-- AIMLTrainingReport

Top <|-- AIMLEntity

@enduml

Annex B (normative):
OpenAPI definition of the AI/ML NRM

# B.1	General

This annex contains the OpenAPI definition of the AI/ML NRM in YAML format.

The information models of the AI/ML NRM are defined in clause 7.

Mapping rules to produce the OpenAPI definition based on the information model are defined in TS 32.160 [14].

# B.2	Solution Set (SS) definitions

## B.2.1	OpenAPI document "AiMlNrm.yaml"

openapi: 3.0.1

info:

title: AI/ML NRM

version: 17.0.0

description: >-

OAS 3.0.1 specification of the AI/ML NRM

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: TS 28.105; AI/ML Management

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.105/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

AIMLEntityList:

type: array

items:

$ref: '#/components/schemas/AIMLEntity'

RequestStatus:

type: string

enum:

- NOT_STARTED

- TRAINING_IN_PROGRESS

- SUSPENDED

- FINISHED

- CANCELLED

PerformanceRequirements:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

ModelPerformance:

type: object

properties:

inferenceOutputName:

type: string

performanceMetric:

type: string

performanceScore:

type: number

format: float

decisionConfidenceScore:

type: number

format: float

TrainingProcessMonitor:

description: >-

This data type is the "ProcessMonitor" data type defined in "genericNrm.yaml" with specialisations for usage in the "AIMLTrainingProcess".

type: object

properties:

aIMLTrainingProcessId:

type: string

status:

type: string

enum:

- RUNNING

- CANCELLING

- CANCELLED

- SUSPENDED

- FINSHED

progressPercentage:

type: integer

minimum: 0

maximum: 100

progressStateInfo:

type: string

enum:

- COLLECTING_DATA

- PREPARING_TRAINING_DATA

- TRAINING

resultStateInfo:

type: string

#-------- Definition of abstract IOCs --------------------------------------------

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

AIMLTrainingFunction:

$ref: '#/components/schemas/AIMLTrainingFunction-Multiple'

ManagedElement-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'genericNrm.yaml#/components/schemas/ManagedElement-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedElement-ncO'

- type: object

properties:

AIMLTrainingFunction:

$ref: '#/components/schemas/AIMLTrainingFunction-Multiple'

AIMLTrainingFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

aIMLEntityRefList:

$ref: 'comDefs.yaml#/components/schemas/DnList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

AIMLTrainingRequest:

$ref: '#/components/schemas/AIMLTrainingRequest-Multiple'

AIMLTrainingProcess:

$ref: '#/components/schemas/AIMLTrainingProcess-Multiple'

AIMLTrainingReport:

$ref: '#/components/schemas/AIMLTrainingReport-Multiple'

AIMLTrainingRequest-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

candidateTraingDataSource:

type: array

items:

type: string

traingDataQualityScore:

type: number

format: float

trainingRequestSource:

type: string

requestStatus:

$ref: '#/components/schemas/RequestStatus'

expectedRuntimeContext:

$ref: 'comDefs.yaml#/components/schemas/DateTime'

performanceRequirements:

$ref: '#/components/schemas/PerformanceRequirements'

cancelRequest:

type: boolean

suspendRequest:

type: boolean

aIMLEntityRef:

$ref: 'comDefs.yaml#/components/schemas/Dn'

AIMLTrainingProcess-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

aIMLTrainingProcessId:

type: string

priority:

type: integer

terminationConditions:

type: string

progressStatus:

$ref: '#/components/schemas/TrainingProcessMonitor'

cancelProcess:

type: boolean

suspendProcess:

type: boolean

trainingRequestRef:

$ref: 'comDefs.yaml#/components/schemas/DnList'

trainingReportRef:

$ref: 'comDefs.yaml#/components/schemas/Dn'

aIMLEntityRef:

$ref: 'comDefs.yaml#/components/schemas/Dn'

AIMLTrainingReport-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

areConsumerTrainingDataUsed:

type: boolean

usedConsumerTrainingData:

type: array

items:

type: string

confidenceIndication:

type: integer

modelPerformanceTraining:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

areNewTrainingDataUsed:

type: boolean

aIMLEntityRef:

$ref: 'comDefs.yaml#/components/schemas/Dn'

AIMLEntity-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

aIMLEntityId:

type: string

inferenceType:

type: string

aIMLEntityVersion:

type: string

expectedRunTimeContext:

type: string

trainingContext:

type: string

runTimeContext:

type: string

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

AIMLTrainingFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLTrainingFunction-Single'

AIMLTrainingRequest-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLTrainingRequest-Single'

AIMLTrainingProcess-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLTrainingProcess-Single'

AIMLTrainingReport-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLTrainingReport-Single'

AIMLEntity-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLEntity-Single'

#-------- Definitions in TS 28.104 for TS 28.532 ---------------------------------

resources-AiMlNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

- $ref: '#/components/schemas/AIMLTrainingFunction-Single'

- $ref: '#/components/schemas/AIMLTrainingRequest-Single'

- $ref: '#/components/schemas/AIMLTrainingProcess-Single'

- $ref: '#/components/schemas/AIMLTrainingReport-Single'

End of changes