- 3GPP TSG-SA5 Meeting #144e	S5-224122

- e-meeting, 27 June – 1 July 2022

- Source:	Huawei

- Title:	pCR 28.903 adding key issue for collocated management for MEC

- Document for:	Approval

- Agenda Item:	6.9.7.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TR 28.903-020 “Study on alignment with ETSI MEC for Edge computing management”

[2]	3GPP TR 23.700-98 V1.0.0 “Study on Enhanced architecture for enabling Edge Applications”

# 3	Rationale

In [2], it describes the deployment option of EDGEAPP and ETSI MEC platform in annex B, one of the option is the two platforms (EES and MEC Platform) are co-located, such as,

This pCR describes the key issue when considering co-locate the 2 platforms.

# 4	Detailed proposal

It is proposed to add a new key issue (Management of collocated platforms for EES and MEC platform) in new clause 5.x in 3GPP TR 28.903-020 [1].

1st change

- 5.X	Key Issue#x: Management of collocated platforms for EES and MEC platform

5.X.1	Description

As described in [5], one of the deployment option is that the EES and MEC platform can be collocated in an implementation.

The EES instantiation procedure is described in clause 7.1.4 of TS 28.538, which the PLMN operator or ECSP requests the EES instantiation via provisioning MnS to ECSP management system, the simplified procedure is show in Figure 5.x.1-1,

Figure 5.x.1-1: EES deployment procedure

The MEC platform as described in ETSI ISG MEC 003 [4] can be deployed as a VNF by using ETSI NFV MANO, the  the simplified procedure is show in Figure 5.x.1-2,

Figure 5.x.1-2: MEC platform deployment procedure

In the collocated scenario, the EES and MEC platform can be deployed in the same Physical/NFV Infrastructure by the same operator. The MEC platform can be deployed first, then the operator can deploy the EES at the same Physical/NFV Infrastructure where the MEC platform is deployed.

REQ-EES-deplyment-1: ECSP management system shall have the capability to instantiate the EES to a specific Physical/NFV Infrastructure that colllcated with MEC platform.

End of change