3GPP TSG-SA5 Meeting #144-e 	S5-224146rev1

- e-meeting, 27th June– 1st July 2022

- Source:	Huawei

- Title:	Clarification on the Roaming Charging Profile for HR

- Document for:	Discussion

Agenda Item:	7.5.3

# 1	Decision/action requested

Clarification on the Roaming Charging Profile for HR.

# 2	References

[1]	3GPP TR 28.827 Study on 5G charging for additional roaming scenarios and actors

# 3	Rationale

Clarification on the Roaming Charging Profile for HR in the topic 7.1.

The triggers for the Roaming Charging Profile update should be clarified in the TS 32.255.

# 4	Detailed proposal

First change

#### 7.1.4.4	Solution #1.4: Roaming Charging Profile in V-SMF in PDU session establishment

##### 7.1.4.4.1	General

A possible solution for key issue #1a and REQ-CH_VMNO-05, Roaming Charging Profile negotiation enhancement for 5G data connectivity charging provided to the home MNO by the visited MNO.

##### 7.1.4.4.2	Reference architecture

The reference architecture would be the same as in clause 4.2.2 of TS 32.255 [4].

##### 7.1.4.4.3	Message flows

The message flows about roaming charging profile negotiation for 5G data connectivity charging are present in the figure 5.2.2.12.2.1, PDU session establishment, of TS 32.255 [4].

In the step 13,

13ch-a. Based on "received PDU session conditions" trigger, a Charging Data Request [Update] is sent to CHF with charging information received from H-SMF, which includes the HPLMN selected "Roaming Charging Profile" and counts per QFI are started.

13ch-b. The CHF updates the CDR.

13ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the V-SMF.

During PDU session establishment, for Home Routed Roaming using QBC, the roaming charging profile is sent from the V-CHF to the V-SMF which is then forwarded to the H-CHF (using the H-SMF). The H-CHF may responed with a new roaming charging profile. The H-CHF provided roaming charging profile is then used by the V-SMF (as well as the H-SMF) for the QBC, the new charging roaming profile is also sent to the V-CHF from V-SMF. The corresponding trigger "received PDU session conditions" in the step 13ch-a should be clarified.

Next change

#### 7.1.4.x	Solution #1.x: Roaming Charging Profile Update for inter-PLMN V-SMF change

The possible solution for key issue #1a and REQ-CH_VMNO-05, is the clarification of the roaming charging profile mechanism for the figure 5.2.2.14.9, Roaming Home routed PDU session - inter-PLMN V-SMF change, of TS 32.255[4],

In the step 8ach-a,

8ach-a. A Charging Data Request [Update] is sent to H-CHF in HPLMN, indicating the V-SMF change and "Roaming Charging Profile" received from the VPLMN.

8ach-b. The H-CHF in HPLMN updates the CDR.

8ach-c. The H-CHF in HPLMN acknowledges by sending Charging Data Response [Update] to the H-SMF and supplies the HPLMN selected "Roaming Charging Profile" for this VPLMN.

In the step 8cch-a,

8cch-a. A Charging Data Request [Update] is sent to V-CHF with charging information received from H-SMF, which includes the HPLMN selected "Roaming Charging Profile".

8cch-b. The V-CHF updates the CDR.

8cch-c. The V-CHF acknowledges by sending Charging Data Response [Update] to the V-SMF.

How to report the updated "Roaming Charging Profile" to V-CHF should be clarified without the addiontion of a special trigger. The updated "Roaming Charging Profile"will be reported in the next Charging Data Request [Update] which is armed by subsequent triggers.

#### 7.1.4.x	Solution #1.y: Roaming Home routed PDU session Intra-PLMN V-SMF change

The possible solution is for key issue #1a and REQ-CH_VMNO-05,the message flows about roaming charging profile negotiation for roaming Home routed PDU session - intra-PLMN V-SMF change are present in the clause 5.2.2.14.10 of TS 32.255[4].

The Roaming Charging Profile is transferred from old V-SMF to the new V-SMF.

3- 4b.	SM Context retrieval by new V-SMF from the old V-SMF. The Charging Id containing Charging Id1 value, the "Home Roaming Charging Profile", and the V-CHF address, are conveyed to the new V-SMF.

No enhancement is required. The Roaming Charging Profile is NOT negotiated again.

End of change