3GPP TSG-SA5 Meeting #144-e 	S5-224150rev1

- e-meeting, 27th June– 1st July 2022

- Source:	Huawei

- Title:	Add the solution for the charging session granularity

- Document for:	Discussion

Agenda Item:	7.5.3

# 1	Decision/action requested

Add the solution for the charging session granularity.

# 2	References

[1]	3GPP TR 28.827 Study on 5G charging for additional roaming scenarios and actors

# 3	Rationale

Add the solution for the charging session granularity between V-CHF and H-CHF for the topic 7.2.

# 4	Detailed proposal

First change

#### 7.2.4.X	Solution #2.X: The session between V-CHF and H-CHF per UE

##### 7.2.4.X.1	General

A possible solution for key issue #2b, #2d, the charging session for V-CHF communicating with H-CHF is per UE.

##### 7.2.4.X.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1.

##### 7.2.4.X.3	Message flows

In this case, the SMF establish the charging session with the V-CHF per PDU session, specified in the TS 32.255[4] The V-CHF would establish the charging session with the H-CHF per UE.

#### 7.2.4.Y	Solution #2.Y: The session between V-CHF and H-CHF per PDU session

##### 7.2.4.Y.1	General

A possible solution for key issue #2b, #2d, the charging session for V-CHF communicating with H-CHF is per PDU session.

##### 7.2.4.Y.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1.

##### 7.2.4.Y.3	Message flows

In this case, the SMF establishe the charging session with the V-CHF per PDU session, specified in the TS 32.255[4] The V-CHF would establish the charging session with the H-CHF per PDU session, described in the solution 2.1

End of change