3GPP TSG-SA5 Meeting #144-e 	S5-224156

- e-meeting, 27th June– 1st July 2022

- Source:	Huawei

- Title:	Add new topic about access to PLMN services via SNPN

- Document for:	Discussion

Agenda Item:	7.5.4

# 1	Decision/action requested

Add new topic about access to PLMN services via SNPN.

# 2	References

[1]	3GPP TR 28.828 Study on charging aspects for enhanced support of non-public networks

# 3	Rationale

Add the new topic about access to PLMN services via SNPN in the TR 28.828.

# 4	Detailed proposal

First change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CAG	Closed Access Group

CSC 	Communication Service Customer

CSP 	Communication Service Provider

DNN	Data Network Name

N3IWF	Non-3GPP InterWorking Function

NID	Network identifier

NPN-OP	NPN Operator

NPN-SC	NPN Service Customer

NPN-SP	NPN Service Provider

PNI-NPN	Public Network Integrated NPN

SNPN	Stand-alone Non-Public Network

Next change

## 5.3	Topic 3: Converged charging for data connectivity in PLMN

### 5.3.1	Use cases

#### 5.3.1.1	Use case #3a: End users access the PLMN services via SNPN

SNPN 5GS deployments are based on the 5G system architecture depicted in clause 4.2.3 of TS 23.501[x], the architecture for access to PLMN services via SNPN is depicted in Figure 4.2.1-3.

To access PLMN services, a UE in SNPN access mode establishes the data connectivity to an N3IWF in the PLMN.

Charged Party: An UE who has a subscription with the SNPN which allows access of the PLMN service.

Charging Party:

-	SNPN operator who charges the End user for access and data connectivity.

-	PLMN operator who charges the 5G data connectivity for the End user.

#### 5.2.1.2	Use case #3b: Access the PLMN services via SNPN

The scenario description is the same with the use case #3a.

Charged Party: SNPN operator

### 5.1.2	Potential charging requirements

REQ-CH_SNPN_E-01: The SNPN should support collecting charging information of SNPN access and data connectivity usage per UE.

REQ-CH_SNPN_E-02: The 5G system should support collecting charging information based on the 5G data connectivity usage per UE for PLMN services

REQ-CH_SNPN_E-03: The 5G system should support collecting charging information based on the 5G data connectivity usage per SNPN for PLMN services.

### 5.1.3	Key issues

The following key issues are identified:

-	Key Issue #1a: the charging information collection and reporting in SNPN for services usage per UE

-	Key Issue #1b: the charging information collection and reporting in 5G system for 5G data connectivity usage for PLMN services per UE.

-	Key Issue #1c: the charging information collection and reporting in PLMN for 5G data connectivity usage for PLMN services per SNPN.

End of change