3GPP TSG SA WG5 Meeting 144-e	S5-224244

- electronic meeting, online, 27th June– 1t July 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.405 | CR |  | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of QoE measurement collection at handover in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eQoE |  |  |  |  |  | Date: |  |  | 2022-06-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The features are missing in the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Handling of measurement collection at handover in NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functionality is missing resulting in interoperability problem |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.z.2 (new), 4.z.2.1 (new), 4.z.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Draft CR |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

2 		References

[x]	3GPP TS 38.331: " NR; Radio Resource Control (RRC); Protocol specification".

Next change

## 4.z	Signalling based activation in NR

### 4.z.2 Handling of measurement collection at handover in NR

#### 4.z.2.1	Handover between cells within an gNB

When handover is made the measurement area is checked. If the mobile is inside the measurement area after the handover, the gNB sends  the RRCReconfiguration message [x] to the UE. The UE send the +CAPPLEVMCNR AT command [5] to the application, if there is no ongoing QoE measurements, to inform that recording shall start if the application becomes active.

#### 4.z.2.2	NG Based Handover for Signalling Based Activation

The figure 4.z.2.2-1 and the text below describes the handling at NG Based handover between gNBs for Signalling Based Activation case.

Figure 4.z.2.2-1: Handling of QMC activation in case of NG Based handover in NR for Signalling Based Activation

- Source gNB sends the message HANDOVER REQUIRED to AMF. The message includes the Source to Target Transparent Container which contains qoEReference, areaScope,serviceType, measConfigAppLayerId, qoECollectionEntityAddress, qoEMeasurementStatus, sliceScope and containerForApplicationLayerMeasurementConfiguration.

- AMF sends the message HANDOVER REQUEST to the Target gNB. The message includes the Source to Target Transparent Container which contains the same parameters as in step 1.

- The target gNB checks if the measurements will be continued or not.

- The target gNB sends the message HANDOVER REQUEST ACKNOWLEDGE to the AMF.

- The AMF sends the message HANDOVER COMMAND to the source gNB.

- The source gNB sends the message RRCReconfiguration to the UE.

- The UE sends the message RRCReconfigurationComplete to the gNB.

- The application continues QoE measurements.

- The application layer sends the AT command +CAPPLEVMRNR including measConfigAppLayerId and qoEMeasurementStatus indicating that a session has started to the access stratum.

- The UE sends the message MeasurementReportAppLayer including MeasReportAppLayerId and qoEMeasurementStatus to the gNB.

- The gNB sends a notification including qoEReference, recordingSessionId to the MnS Consumer.

- When the QMC is completed or end of periodic reporting is reached, the recorded information is collected in a QMC report [6], [7].

- The application layer sends the AT command +CAPPLEVMRNR including MeasConfigAppLayerId, qoEMeasurementStatus and the QMC report to the access stratum.

- The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId, qoEMeasurementStatus and the QMC report to the gNB.

- The gNB sends the QMC report to the MCE associated to the qoEReference. The report contains qoEReference, recordingSessionId and sliceScope (which contains only S-NSSAI that is used).

End change