- 3GPP TSG-SA5 Meeting #144e	S5- 224251rev1

- e-meeting 27 June - 1 July

- Source:	CATT

- Title:	pCR TR 28.908 Add use case on AI/ML model deployment

- Document for:	Approval

- Agenda Item:	6.5.5.4

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		SP-211443 New Study on AI/ ML management

[2]		3GPP TR 28.908-020 Management and orchestration; Study on Artificial Intelligence/Machine Learning (AI/ML) management

[3]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management".

# 3	Rationale

In the SI AI/ ML management [1], deployment of AI/ML model (new or updated model) and AI/ML-enabled function is to be studied.

AI/ML deployment refers to the process of making an AI/ML-enabled function available in the operational environments. After the AI/ML Entity is trained by the AI/ML training (AIMLT) MnS producer in the production environment, the AI/ML-enabled function containing the AI/ML model  needs to be deployed to conduct inference in operational environment. As an AI/ML-enabled function has deployed and activated in the production environment, to improve model performance with the changing data, the AI/ML-enabled function or the entity therein may need to be retrained and redeployed.

This pCR is to add the UC and potential requirements on AI/ML model deployment, especially the AI/ML deployment information retrieved by consumer.

# 4	Detailed proposal

Start of modification

# 5.X	Use cases, potential requirements and possible solutions

## 5.X.1	AI/ML deployment

### 5.X.1	Description

AI/ML deployment refers to the process of making an AI/ML-enabled function available in the operational environments, where it could start adding value by conducting inference (e.g., prediction). After trained AI/ML-enabled function meets the performance creiteria, the AI/ML-enabled function could be deployed in 3GPP system via a software installation or via a configuration management procedure and subsequently activated.

After an AI/ML-enabled function is deployed and activated in the production environment, the data fed to the AI/ML-enabled function may change to the level where it is different form the data used in the initioal prior training of the respective AI/MLEntity. To improve model performance with the changing data, the AI/ML-enabled function or the entity therein may need to be retrained and redeployed within the production environment.

### 5.X.2	Use cases

#### 5.X.2.1	AI/ML deployment information retrieved by consumer

After the AI/ML Entity is trained by the AI/ML training (AIMLT) MnS producer during development, the AI/ML-enabled function needs to be deployed to conduct inference in the operational environment.

Once the AI/ML functionality has been installed in the operational environment, as different consumers need to know the available information of AI/ML enabled functions and to determine the next appropriate action, the MnS consumer may request the MnS producer to retrieve the deployment information of AI/ML enabled function.

The general deployment information used to describe an AI/ML-enabled function may include:

-	Resource information, which describes the static parameters of AI/ML Entity (e.g. aIMLEntityVersion, aIMLEntityId, trainingContext, see TS 28.105 [3])

-	Management information, which describes the information model that is used for AI/ML Entity lifecycle management (e.g. activation flag, status, create time, last update time).

-	Capability information, which describes the capability information (e.g. inference type, performance metrics).

### 5.X.3	Potential requirements

REQ-MODEL_DPL-CON-1 The MnS producer responsible for AI/ML management shall have a capability to retrive deployment information of the AI/ML-enabled function and inform an authorized consumer about the information.

REQ-MODEL_DPL-CON-2 The MnS producer responsible for AI/ML management shall have a capability to allow an authorized consumer to retrive deployment information of the AI/ML-enabled function.

### 5.X.4	Possible solutions

TBD

### 5.X.5	Evaluation

TBD

End of modifications