3GPP TSG-SA5 Meeting #144-e 	S5-224302rev1

- e-meeting, 27 June - 1 July 2022

- Source:	China Unicom

- Title:	Add key issue on requirements and use cases supporting multiple MnS discovery service producers

- Document for:		Approval

Agenda Item:

# 1	Decision/action requested

Discuss and approve on the proposal.

# 2	References

[1]	3GPP TR 28.925 enhancement of service based management architecture

[2] 	3GPP TS 28.533 Management and orchestration; Architecture framework

[3] 	3GPP TS 28.537 Management and orchestration; Management capabilities

[4] 	3GPP TS 28.622 Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

[5] 	3GPP TR 28.623 Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions

[6]	3GPP TS 32.101 Telecommunication management; Principles and high level requirements

# 3	Rationale

The requirements and use cases of MnS discovery are introduced in TS 28.533[2] and TS 28.537[3], The related IOCs are specified in TS 28.622[4] and TS 28.623[5]. As presented in the architectural framework contents of TS 32.101[6], more than one MnS Discovery service producers may be available in the management system. Observing that use cases and requirements in clause 5.2 of TS 28.537[3] are specified under the assumption that only one MnS discovery service producer exists defaultly, the result, that in the use case of clause 5.2.1.3 Adding a new management service consumer, MnS consumers could not know send requests to which producers when multiple discovery service producers exist simultaneously, may be led to. In the process of adding a new management service consumer, step 4.3(M) has specified the scenario when more than one suitable version of MnS porducer exists. Since MnS service producer is the same as all other types of services(provisioning, fault management, performance management, file data reporting, streaming, heartbeat, etc.), use cases and requirements supporting multiple MnS discovery service producers are suggested to discuss.

# 4	Detailed proposal

This document proposes the following changes in TR 28.925 [1].

1st Change

## 4.x Issue #X Requirements and use cases supporting multiple MnS discovery service producers

### 4.X.1	Description

The architecture reference model is introduced in TS 28.533[2]. Clause 4.7.3 describes the concept of MnS discovery service. TS 28.537[3] specifies use cases and requirements. Clause 5.4 in TS 32.101[6] points out that one or more discovery service producers may exist in the management system, thereby the following aspects on the scenario of multiple MnS discovery producers in the management system are expected to be further discussed:

- The processes of new consumers sending requests in the management system with multiple MnS discovery service producers;

- The requirements supporting consumers obtaining information from more than one MnS discovery service producers.

End of Changes