3GPP TSG-SA5 Meeting #145e 	S5-225471

- E-meeting, 15 August – 24 August 2022

- Source:	Huawei

- Title:	pCR TR 28.863 Issue #1 Difference of KPI, KQI and QoE

- Document for:	Approval

Agenda Item:	6.9.4.1

# 1	Decision/action requested

This contribution is proposed to be approved.

# 2	References

[1]	 3GPP TS 28.863v0.2.0: "Study on Key Quality Indicators (KQIs) for 5G service experience"

[2]	 3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)"

[3]   ITU-T P.10/G.100 (11/2017): Vocabulary for performance and quality of service.

# 3	Rationale

The document gives the description and analysis of the difference of KPI, KQI and QoE.

# 4	Detailed proposal

It is proposed to apply the following changes as below in TR 28.863 [1].

Start of the first change

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

KQI: Key Quality Indicators which reflect the end-to-end service quality.

End of the first change

Start of the second change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

KQI	Key Quality Indicators

End of the second change

Start of the third change

- 4.1.1.x	Difference of KPI, KQI and QoE

###### 4.1.1.x.1	Difference between KPI and KQI

KPIs  are Key Performance Indicators for the 5G network and network slicing. KPIs are collected from the network or are calculated from the performance measurements. KPIs are defined in TS28.554 [2].

KQIs (Key Quality Indicators) are indicators that reflect the service quality. KQIs are collected or calculated according to service aspect.

The difference between KPI and KQI is that KPI is from network aspect and KQI is from service aspect. The measurements or related attributes for KPI are generic for all of the data at the interface (e.g. N3 interface), while the measurements or statistics for KQI are for a certain service. The service characteristics must be distinguished for KQI.

###### 4.1.1.x.2	Difference between KQI and QoE

In ITU-T P.10/G.100 [3] the definition of QoE is given:

Quality of experience (QoE)

The degree of delight or annoyance of the user of an application or service.

QoE is defined in ITU-T P.10/G.100 [3] as the degree of delight or annoyance of the user of an application or service. It includes the complete end-to-end system effects (client, terminal, network, services infrastructure, etc.) and may be influenced by user expectations and context.

From the above definition of QoE we see that QoE is an overall quality experience for a user of a service.

The difference between QoE and KQI is that QoE is the overall quality experience, while KQIs are the key indicators to reflect the service quality.

There are also QoE metrics in SA4. QoE metrics are the metrics related to QoE in SA4 aspect and these metrics collected by the client will be reported to the OAM or QoE server in the QoE metrics reporting. SA4 focuses on the definition of QoE metrics to be collected by the client. How to make use of these QoE metrics is up to OAM and QoE server. The difference is that the QoE metrics are collected from the client side for all of the aspects of the media e.g. the buffer, play list, MPD information, while KQIs are just the key indicators and some of them are chosen or calculated from QoE metrics to reflect the quality of the service from the management aspect. E.g. from the management aspect, the consumer could express the expectation for a service through KQI to the producer. KQIs may be collected from application layer, calculated from KPI, or calculated based on some of the collected QoE metrics. QoE metrics are one source of KQI.

End of the third change