3GPP TSG-SA5 Meeting #145e 	S5-225477

- E-meeting, 15 August – 24 August 2022

- Source:	Huawei

- Title:	pCR TR 28.863 Issue #4 Remote controlling description

- Document for:	Approval

Agenda Item:	6.9.4.1

# 1	Decision/action requested

This contribution is proposed to be approved.

# 2	References

[1]	 3GPP TS 28.863v0.2.0: "Study on Key Quality Indicators (KQIs) for 5G service experience"

[2]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains"

[3]	3GPP TS 22.261: "Service requirements for the 5G system"

# 3	Rationale

The document gives the detailed description of the scenarios of remote controlling.

# 4	Detailed proposal

It is proposed to apply the following changes as below in TR 28.863 [1].

Start of the first change

## 4.4	Issue # 4: KQIs for Remote Controlling

### 4.4.1	Description

The service of remote control is defined in TS22.104 [2]/TS22.261 [3]. Remote control is characterised by a UE being operated remotely by a human or a computer. E.g. the remote control of mobile cranes, mobile pumps and fixed portal cranes.

In remote controlling scenario, real-time interactions are involved. URLLC is used in 5G SA (Single Access) networking to meet ultra-low latency requirements. Real-time interactive services are mainly small-packet transmission services, which do not require network bandwidth but require ultra-low latency. For industrial-grade URLLC services, unqualified delay may cause service failures instead of performance problems. Therefore, it is important to analyze the delay and the delay fulfillment boundary. For example, if the SLA requirement is 10 ms, the evaluation focuses on ensuring that most service measurements are within the 10 ms boundary, or the proportion that exceed the delay boundary is as low as possible.

Furthermore, the packet loss is also an important factor to be considered in remote controll. If the packet loss rate is too high, the control instruction fails. Therefore, the packet loss rate and the threshold of the packet loss rete need to be considered when defining the quality indicator for the service.

End of the first change