3GPP TSG-SA5 Meeting #146Bis-e 	S5-231014

- Electronic meeting, 16 - 19 January 2023

- Source:	Huawei, Deutsche Telekom

- Title:	DP on solutions for expressing slicing requirements as intent expectations

- Document for:		Discussion and Endorsement

Agenda Item:	6.1.1

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP TR 28.836: " Study on intent-driven management for network slicing v0.3.0".

[2]	S5-226206 Add solution for expressing service and slice profile requirements as intent expectations.

[3]	S5-226580 Discussion paper on study intent driven management for network slicing

# 3	Rationale

This discussion paper related to FS_eIDMS_MN SI, FS_NETSLICE_IDMS SI and eNETSLICE_PRO WI.

In SA5#146 meeting, several contributions (e.g., S5-226206[2], S5-226580[3]) proposes the solution for expressing slicing requirements for delivering network slice (subnet) as intent expectations. The main controversial part is the relation with ServiceProfile/SliceProfile. Based on the discussion, following are two potential relationship:

Option#1: The parameters in ServiceProfile/SliceProfile are already intent level, which requires a reformation of ServiceProfile/SliceProfile to follow generic intent model methodology.

Option#2: The parameters in ServiceProfile/SliceProfile are not intent level, needs to define intent level parameters for slicing intent. The parameters in ServiceProfile/SliceProfile can be translated from intent level parameters.

Analysis:  In TS 28.541 [4], 6.3.3.1 described the service profile represents the properties of the network slice related requirements that should be supported by a NetworkSlice instance in a 5G network and clause 6.3.4.1 described the sliceprofile represents the properties of network slice subnet related requirement that should be supported by the NetworkSliceSubnet instance in a 5G network. Based on the definition for service profile and slice profile, it is obvious that service profile and slice profile represent MnS consumer’s expectations for network slice and network slice subnet, which is aligned with intent definition (expectations including requirements, goals and constraints given to a 3GPP system, without specifying how to achieve them).

Currently the ServiceProfile and SliceProfile is modelled as a set of properties, while IntentExpectation is composed of expectationObject, expectationTargets and expectationContexts, so it is necessary to investigate each attribute in ServiceProfile and SliceProfile to identify which attribute can be represented by which intent expectation components (incl. expectationObject, expectationTargets and expectationContexts).

Proposal#1: It proposes above Option#1 and investigate each attribute in ServiceProfile and SliceProfile to identify which attribute can be represented by which intent expectation components (incl. expectationObject, expectationTargets and expectationContexts).

# 4	Detailed proposal

Following above proposal#1, It proposes to add following content in TR 28.836.

/**************Content to be added in TR 28.836********************/

## 6.X		Solution for expressing service and slice profile requirements as intent expectations.

### 6.X.1	Description

In TS 28.541 [4], clause 6.3.3.1 the description says that the service profile represents the properties of the network slice related requirements and that these should be supported by a NetworkSlice instance in a 5G network and in clause 6.3.4.1 the description says that the sliceprofile represents the properties of network slice subnet related requirements and that these should be supported by the NetworkSliceSubnet instance in a 5G network. Based on the definition for service profile and slice profile, it is obvious that service profile and slice profile represent MnS consumer’s expectations for network slice and network slice subnet, which is aligned with intent definition (expectations including requirements, goals and constraints given to a 3GPP system, without specifying how to achieve them). So, expressing service and slice profile requirements as intent expectations can be used to support the separate lifecycle management of an MnS consumer’s requirements for a network slice (subnet), which decouples the lifecycle management for MnS consumer’s requirements for network slice (subnet) and the lifecycle management for network slice (subnet). It enables MnS consumer to focus on expressing network slicing requirements (including service profile and slice profile) to MnS producer, and obtain fulfilment information for those requirements, without knowing how to deal with the detailed management of a network slice and a network slice subnet. The decoupling of the lifecycle management for MnS consumer’s requirements of network slice (subnet) and the lifecycle management of network slice (subnet) supports asynchronous modes for network slice (subnet) management. Following are some benefits to introduce the intent driven approach for network slice (subnet) compared to existing slice solution:

- Model driven approach for Network Slice (Subnet) allocation and de-allocation functionalities

- Support asynchronous modes for network slice (subnet) allocation and de-allocation functionalities

- A unified and simplified solution (CRUD operation with intent NRM fragment) to support following slicing management functionalities

  - Network Slice (Subnet) allocation and de-allocation functionalities, existing solution uses operation allocateNSI/NSSI and deAllocateNSI/NSSI.

  - Network Slice (Subnet) modification and query functionalities, existing solution uses CRUD operation with Network Slicing NRM Fragment.

  - Network Slice (Subnet) SLS asuurance and monitoring functionalities, existing solution uses CRUD operation with assurance management NRM fragment.

  - Network Slice (Subnet) feasibility check functionalities, existing solution uses CRUD operation with feasibility check NRM fragment.

Currently the ServiceProfile and SliceProfile is modelled as a set of properties, while IntentExpectation is composed of expectationObject, expectationTargets and expectationContexts, so it is necessary to investigate each attribute in ServiceProfile and SliceProfile to identify which attribute can be represented by which intent expectation components (incl. expectationObject, expectationTargets and expectationContexts).

Following figure give one example to illustrate how ServiceProfile can be represented by intent expectation components.

Figure 6.X.1-1 Example of how ServiceProfile will be represented by which intent expectation components

In case service and slice profile requirements are modelled as intent expectations, the network slice (subnet) lifecycle management can be alternatively supported by intent driven MnS (which are implemented by CRUD operations and intent model). MnS consumer expresses intent expectations for network slice (subnet) and MnS producer performs the management tasks for network slice (subnet) to satisfy the intent expectations for network slice (subnet). MnS consumer also can obtain the intent fulfilment information to evaluate whether the requirements for network slice (subnet) is satisfied by MnS producer.

Following subclauses will give the detailed solution for how to model service profile and slice profile as intent expectations.

### 6.X.2	Modelling service profile attributes as intent expectations for network slice

#### 6.X.2.1	Definition of ExpectationObject for NetworkSlice

The definition of an expectationObject that is applicable to intent driven network slice management is shown in Table 6.X.2.1

| Attribute Name | Attribute Values |
| --- | --- |
| ObjectType | NetworkSlice |
| ObjectInstance | DN of NetworkSlice instance |
| ObjectContexts | plmnInfoList coverageArea sST radioSpectrum sliceSimultanuousUse |

Table 6.X.2.1 Definition of an ExpectationObject for a NetworkSlice instance

#### 6.X.2.2	Definition of expectationTargets

The expectationTargets that are applicable to intent driven network slice management is shown in Table 6.X.2.2

| Attributes | Attribute Values |
| --- | --- |
| expectationTargets | maxNumberofUEs dLLatency uLLatency availability dlThptPerUE UlThptPerUE dlThptPerSlice UlThptPerSlice maxNumberofPDUSessions Reliability maxDLDataVolume maxULDataVolume |

Table 6.X.2.2 Definitions of expectationTarget for a NetworkSlice instance

##### 6.X.2.3	Definition of expectationContexts

The expectationContexts that are applicable to intent driven network slice management is shown in Table 6.X.2.3

| Attributes | Attribute Values |
| --- | --- |
| expectationContexts | uEMobilityLevel networkSliceSharingIndicator delayTolerance dLDeterministicComm uLDeterministicComm dLMaxPktSize uLMaxPktSize v2XCommModels termDensity activityFactor uESpeed survivalTime nBIoT synchronicity positioning nssaaSupport n6Protection |

Table 6.X.2.3 Definitions of expectationContext for a NetworkSlice instance

### 6.X.3	Modelling slice profile attributes as intent expectations for network slice subnet

#### 6.X.3.1	Modelling RANSliceProfile attributes as intent expectation for RAN network slice subnet

TBD

#### 6.X.3.2	Modelling CNSliceProfile attributes as intent expectation for CN network slice subnet

TBD

/**************Content to be added in TR 28.836********************/

End of changes