3GPP TSG-SA5 Meeting #146Bis-e 	S5-231067rev1

Electronic meeting, 16 - 19 January 2023

- Source:	Huawei

- Title:	Add new solution on using RCG in LBO roaming

- Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Adding new solution on using RCG in LBO roaming.

# 4	Detailed proposal

First change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BCE	Billing and Charging Evolution

CHF	Charging Function

HR	Home Routed

LBO	Local Breakout

RCG	Roaming Charging Gateway

Next change

#### 7.2.4.x	Solution #2.x: using RCG in LBO roaming charging

##### 7.2.4.x.1	General

A possible solution for key issue #2a, #2b, #2f, #2g, #2h and #2i covering requirements REQ-CH_CVTOH-01, REQ-CH_CVTOH-02, REQ-CH_CVTOH-03, REQ-CH_CVTOH-04, REQ-CH_CVTOH-05, and REQ-CH_CVTOH-06, retail charging for 5G data connectivity provided, 5G connection and mobility, and SMS usage to the home MNO’s users by the visited MNO, in the case of local breakout.

In this solution, RCG is a logical function inside CCS. The RCG (Roaming Charging Gateway) is used in 5G LBO roaming charging, it is a NF (e.g. V-SMF, V-AMF and V-SMSF) producer  and a NF consumer of V-CHF and H-CHF., From reference point prospective, it:

-	interacts with V-SMF based on N40 interface for supporting 5G connectivity charging

-	interacts with V-AMF based on N42 interface for supporting 5G connection charging.

-	interacts with V-SMSF based on N46 interface for supporting SMS charging.

-	generates and conveys the charging information to V-CHF or H-CHF.

There is a single session between NF (e.g. AMF, SMF or SMSF) and RCG.

The solutions #2.6, #2.7, #2.8, #2.9, #2.10, #2.11, #2.13, #2.14 and #2.15 are all applicable to this solution by replacing V-CHF with RCG.

##### 7.2.4.x.2	Reference architecture

Figure 7.2.4.x.2-1: LBO Roaming 5G scenario in service-based interface representation

Figure 7.2.4.x.2-2: LBO Roaming 5G scenario in reference point representation

##### 7.2.4.x.3	Message flows

Figure 7.2.4.x.3-1 shows a message flow of using RCG for LBO roaming charging.

Figure 7.2.4.x.3-1: LBO roaming charging based on RCG and NF/CHF communication

1)	Request for service delivery: A request for session establishment is received in the NF (CTF). The service is configured to be authorized by the RCG before start.

2)	Units determination: the NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging data request [Initial, Quota requested]: The NF (CTF) sends an initial charging data request to the RCG for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

4)	Charging data request [Initial]: The RCG sends the request to the V-CHF for the service to be granted authorization to start.

5)	Open CDR: based on policies, the V-CHF opens a CDR related to the service.

6)	Charging data response [Initial]: The V-CHF grants authorization to RCG for the service to start, with the reserved number of units.

7)	Determine H-CHF: the RCG creates a charging resource for the V-SMF, determines that it need to interact with H-CHF and which H-CHF it should contact and generates an initial charging data request to H-CHF.

8)	Charging data request [Initial, Quota requested]: The RCG sends the request to the H-CHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

9)	Account, Rating, Reservation control: the H-CHF rates the charging data requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, the H-CHF performs the corresponding reservations.

10)	Open CDR: based on policies, the H-CHF opens a CDR related to the service.

11)	Charging data response [Initial, Quota granted]: The H-CHF grants authorization and grants quota to RCG for the service to start, with the reserved number of units.

12)	Charging data response [Initial, Quota granted]: The RCG grants authorization and grants quota to NF (CTF) for the service to start, with the reserved number of units.

13)	Granted units supervision: the NF (CTF) monitors the consumption of the granted units.

14)	Start of service delivery: the NF (CTF) starts to deliver the content/service based on the reserved number of units.

15)	Quota management trigger: A trigger associated to quota management is met. Unit determination is performed when applicable.

16)	Charging data request [Update, Unit used, Quota requested]: The NF (CTF) sends an charging data request to the RCG for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

17)	Charging data request [Update, Unit used]: the RCG sends an intermediate charging data request to the V-CHF, for more units to be granted for the service to continue and reporting the used units.

18)	 Update CDR: based on policies, the V-CHF updates the CDR with charging data related to the service.

19)	Charging data response [Update]: The V-CHF responds to RCG for the service to continue, with the reserved number of units.

20)	Charging data request [Update, Unit used, Quota requested]: the RCG sends an intermediate charging data request to the H-CHF, for more units to be granted for the service to continue and reporting the used units.

21)	Account, Rating, Reservation control: The H-CHF performs the process related to the reported usage and the requested reservation, involving rating entity and user's account balance.

22)	 Update CDR: based on policies, the H-CHF updates the CDR with charging data related to the service.

23)	Charging data response [Update, Quota granted]: The H-CHF grants quota to RCG for the service to continue, with the reserved number of units.

24)	Charging data response [Update, Quota granted]: The RCG grants quota to NF (CTF) for the service to continue, with the reserved number of units.

25)	Granted units supervision: the NF (CTF) monitors the consumption of the granted units.

The consequent Charging data request [Termination] is not described here.

End of change