3GPP TSG-SA5 Meeting #146Bis-e 	S5-231071

Electronic meeting, 16 - 19 January 2023

- Source:	Huawei Technologies

- Title:	Rel-18 pCR 28.828 Correction on the SNPN Topic 1 and SNPN Topic 3

- Document for:	Approval

Agenda Item:	7.5.4

# 1	Discussion

This is a pCR to TR 28.828 about correction on the SNPN Topic 1 and SNPN Topic 3.

# 2	References

[1]	3GPP TR 28.828 " Study on charging aspects for enhanced support of non-public networks".

# 3	Rationale

This pCR proposes to correction on the SNPN Topic 1 and SNPN Topic 3 in TR 28.828 "Study on charging aspects for enhanced support of non-public networks".

For the SNPN directly, the End user charging is considered.

For the interworking scenarios:

-	Access the PLMN service via SNPN, including following case:

- End user charging in SNPN

- End user charging in and PLMN.

- The inter-provider charging (SNPN Charged PLMN): Not considered without supported business case.

- The inter-provider charging (PLMN Charged PLMN)

-	Access the SNPN service via PLMN: End user charging in

- End user charging in SNPN

- The inter-provider charging (PLMN Charged SNPN)

- End user charging in and PLMN: Same operation specified in the TS 32.255, not be detailed in TR.

# 4	Detailed proposal

Propose to incorporate the following change into the TR 28.828 [1].

First changes

## 5.1	Topic 1: Converged charging for data connectivity in SNPN

### 5.1.1	Use cases

#### 5.1.1.1	Use case #1a: End users access the SNPN service directly

SNPN 5GS deployments are based on the 5G system architecture depicted in clause 4.2.3 of TS 23.501 [2].

Charged Party: An End User (UE) who access SNPN services.

Charging Party: SNPN operator who charges the End user for data connectivity in SNPN.

The potential charging requirements and key issues for this UC are: REQ-CH_SNPN_DC-01 and Key Issue #1a.

#### 5.1.1.2	Use case #1b: End users access the SNPN service via PLMN

SNPN 5GS deployments are based on the 5G system architecture depicted in clause 4.2.3 of TS 23.501 [2], the architecture for 5GC with untrusted non-3GPP access for access to SNPN services via a PLMN (and vice versa).

Charged Party: An End User (UE) who has a subscription with the PLMN and SNPN which allows access of the SNPN service via PLMN.

Charging Party:

-	SNPN operator who charges the End user for data connectivity in SNPN.

The potential charging requirements and key issues for this UC are: REQ-CH_SNPN_DC-02 and Key Issue #1b.

#### 5.1.1.3	Use case #1c: End users access the PLMN service via SNPN

SNPN 5GS deployments are based on the 5G system architecture depicted in clause 4.2.3 of TS 23.501 [2], the architecture for access to PLMN services via SNPN is depicted in Figure 4.2.1-3. To access PLMN services, a UE in SNPN access mode establishes the data connectivity to an N3IWF in the PLMN.

Charged Party: A End User (UE) who has a subscription with the SNPN which allows access of the PLMN service.

Charging Party:

-	SNPN operator who charges the End user for data connectivity via SNPN.

The potential charging requirements for this UC are: REQ-CH_SNPN_DC-02 and Key Issue #1b.

#### 5.1.2	Potential charging requirements

REQ-CH_SNPN_DC-01: The SNPN should support collecting charging information of data connectivity charging in SNPN per UE for SNPN services.

REQ-CH_SNPN_DC-02: The SNPN should support collecting charging information of data connectivity charging via SNPN per UE for PLMN services.

5.1.3	Key issues

The following key issues are identified:

-	Key Issue #1a: the charging information collection and reporting in SNPN for data connectivity per UE directly.

-	Key Issue #1b: the charging information collection and reporting in SNPN for data connectivity per UE indirectly via the access network.

5.1.4	Possible solutions

#### 5.1.4.1	Solution #1: Data connectivity in SNPN Charging per UE directly

##### 5.1.4.1.1	General

This solution #1 which relying on 5G data connectivity converged charging architecture defined in TS 32.255 [12], with the extension of SNPN charging per UE, addresses the Key Issue #1a.

##### 5.1.4.1.2	Reference architecture

Based on figure 4.2.1-3, the CHF in the SNPN is added. The UE is charged by CHF in the SNPN for 5G data connectivity in SNPN.

Figure 5.1.4.1.2-1: Data connectivity in SNPN Charging per UE directly in SBI representation

##### 5.1.4.1.3	Message flows

The message flows are the same with the 5G data connectivity per PDU session charging specified in the TS 32.255 [12].

SMF in the SNPN reports the Charging Data Request contains SNPN ID (PLMN ID and NID identifying an SNPN) to the CHF in the SNPN.

#### 5.1.4.2	Solution #2: Data connectivity in SNPN Charging per UE (access the SNPN via PLMN)

##### 5.1.4.2.1	General

This solution #2 which relying on 5G data connectivity converged charging architecture defined in TS 32.255 [12], with Charging UE by SNPN, addresses the Key Issue #1b for indirectly access SNPN.

##### 5.1.4.2.2	Reference architecture

Based on figure 4.2.1-2 the CHF in the SNPN is added. The UE is charged by CHF in the SNPN for data connectivity via PLMN.

Figure 5.1.4.2.1-1 Data connectivity of SNPN Charging (access the SNPN via PLMN)

##### 5.1.4.2.3	Message flows

For access the SNPN services, the message flows for the PDU session establishment, PDU session modification, PDU session release of 5G data connectivity charging would be the same as in clauses 5.2.2.13.2.1, 5.2.2.13.3.1 and 5.2.2.13.4.1 of TS 32.255 [12].

The SMF in SNPN reports the Charging Data Request contains access type ("Non-3GPP access") to the CHF belongs to the same SNPN, using PLMN ID and NID to identify the SNPN.

5.1.4.3	Solution #3: Data connectivity in SNPN Charging per UE (access the PLMN via SNPN)

##### 5.1.4.3.1	General

This solution #3 which relying on 5G data connectivity converged charging architecture defined in TS 32.255 [12], with the extension of SNPN charging per UE for accessing the PLMN service, addresses the Key Issue #1b.

##### 5.1.4.3.2	Reference architecture

Based on figure 4.2.1-3, the CHF in the SNPN is added. The UE is charged by CHF in the SNPN for 5G data connectivity.to support of accessing the PLMN service

Figure 5.1.4.3.2-1 Data connectivity in SPNP Charging (access the PLMN via SNPN)

##### 5.1.4.3.3	Message flows

For access the PLMN services via SNPN, the message flows for the PDU session establishment, PDU session modification, PDU session release of 5G data connectivity charging would be the same as in clauses 5.2.2.13.2.1, 5.2.2.13.3.1 and 5.2.2.13.4.1 of TS 32.255 [12].

The SMF in the SNPN reports the Charging Data Request contains RAT type specifics to SNPN with the SNPN ID and the N3IWF address to the CHF in the SNPN.

Next changes

## 5.3	Topic 3: Converged charging for data connectivity in PLMN

### 5.3.1	Use cases

#### 5.3.1.1	Use case #3a: End users access the PLMN service via SNPN

SNPN 5GS deployments are based on the 5G system architecture depicted in clause 4.2.3 of TS 23.501 [2], the architecture for access to PLMN services via SNPN is depicted in Figure 4.2.1-3. To access PLMN services, a UE in SNPN access mode establishes the data connectivity to an N3IWF in the PLMN.

Charged Party: The End User(UE) who has a subscription with the SNPN which allows access of the PLMN service.

Charging Party:

-

PLMN who charges the 5G data connectivity for the End user.

-	PLMN who charges the 5G data connectivity for the SNPN.

The potential charging requirements for this UC are: REQ-CH_PLMN_DC-01, REQ-CH_PLMN_DC-02 and Key Issue #3a, Key Issue #3b.

#### 5.3.1.2	Use case #3b: End users access the SNPN service via PLMN

SNPN 5GS deployments are based on the 5G system architecture depicted in clause 4.2.3 of TS 23.501 [2], the architecture for 5GC with untrusted non-3GPP access for access to SNPN services via a PLMN.

Charged Party: An End User (UE) who has a subscription with the PLMN which allows access of the SNPN service.

Charging Party:

-	PLMN who charges the 5G data connectivity for the End user.

-	PLMN who charges the 5G data connectivity for the SNPN.

The potential charging requirements and key issues for this UC are: REQ-CH_PLMN_DC-01 and REQ-CH_PLMN_DC-02 and Key Issue #3a and key issue #3b.

### 5.3.2	Potential charging requirements

REQ-CH_PLMN_DC-01: The 5G system should support collecting charging information based on the 5G data connectivity usage per UE.

REQ-CH_PLMN_DC-02: The 5G system should support collecting charging information based on the 5G data connectivity usage per SNPN.

### 5.3.3	Key issues

The following key issues are identified:

-	Key Issue #3a: the charging information collection and reporting in 5G system for 5G data connectivity per UE.

-	Key Issue #3b: the charging information collection and reporting in 5G system for 5G data connectivity for PLMN per SNPN.

### 5.3.4	Possible solutions

#### 5.3.4.1	Solution #1: Data connectivity in PLMN Charging per UE (access the PLMN via SNPN)

##### 5.3.4.1.1	General

This solution #1 which relying on 5G data connectivity converged charging architecture defined in TS 32.255 [12], with the extension of PLMN Charging via SNPN access per UE, addresses the Key Issue #3a.

##### 5.3.4.1.2	Reference architecture

Based on figure 4.2.1-3, the CHF in the PLMN is added. The UE is charged by CHF in the PLMN for 5G data connectivity of PLMN services.

Figure 5.3.4.1.2-1 Data connectivity in PLMN Charging per UE (access the PLMN via SNPN)

##### 5.3.4.1.3	Message flows

For access the PLMN services, the message flows for the PDU session establishment, PDU session modification, PDU session release of 5G data connectivity charging would be the same as in clauses 5.2.2.13.2.1, 5.2.2.13.3.1 and 5.2.2.13.4.1 of TS 32.255 [12].

The SMF in the PLMN reports the Charging Data Request contains RAT type specifics to SNPN

#### 5.3.4.2	Solution #2: Data connectivity in PLMN Charging per SNPN (access the PLMN via SNPN)

##### 5.3.4.2.1	General

This solution #2 which relying on 5G data connectivity converged charging architecture defined in TS 32.255 [12], with the extension of PLMN Charging via SNPN access per SNPN, addresses the Key Issue #3b.

##### 5.3.4.2.2	Reference architecture

The architecture figure is the same with figure 5.3.4.1.2-1. The SNPN is charged by CHF in the PLMN for 5G data connectivity of PLMN services.

##### 5.3.4.2.3	Message flows

The CHF in PLMN charges the volume usage per SNPN based on the accumulation of usage for UEs belonging to the same SNPN.

In the solution, the SMF reporting the charging Data request contains the SNPN ID (PLMN ID And NID identifying the SNPN). If the SNPN ID is not available, the solution 2 does not work.

#### 5.3.4.3	Solution #3: Data connectivity in PLMN Charging per SNPN (access the SNPN via PLMN)

##### 5.3.4.3.1	General

This solution #3 which relying on 5G data connectivity converged charging architecture defined in TS 32.255 [12], with the charging for the usage in the PLMN per SNPN by PLMN, addresses the Key Issue #3b.

##### 5.3.4.3.2	Reference architecture

Based on figure 4.2.1-2, the CHF in the PLMN is added. The SNPN is charged by CHF in the PLMN for data connectivity.

Figure 5.3.4.3.2-1 Data connectivity in PLMN Charging per SNPN (access the SNPN via PLMN)

##### 5.3.4.3.3	Message flows

For access the SNPN services, the message flows for the PDU session establishment, PDU session modification, PDU session release of 5G data connectivity charging would be the same as in clauses 5.2.2.13.2.1, 5.2.2.13.3.1 and 5.2.2.13.4.1 of TS 32.255 [12].

The NWu (for NPN) between the UE and the N3IWF in the SNPN for establishing a secure tunnel between UE and the N3IWF over the PLMN. The SMF in PLMN can distinguish the usage packet transferred to N3IWF in SNPN, and sends the volume to the CHF in the PLMN belongs to same N3IWF address.

The CHF in PLMN charges the volume usage per SNPN based on the accumulation of usage reporting per UE, may with the same N3IWF address.

NOTE: the solution requires the underlay network PLMN can provide indication of the PLMN, which depend on the 5GS core system.

End of changes