3GPP TSG-SA5 Meeting #146Bis-e	S5-231105rev2

Electronic meeting, 16 - 19 January 2023

- Source:	Ericsson

- Title:	Clarifying multilevel attribute properties

- Document for:	Endorsement

Agenda Item:	6.1.1

# 1	Decision/action requested

Endorsement

# 2	References

- 3GPP TS 32.156 Telecommunication management; Fixed Mobile Convergence (FMC) Model repertoire

# 3	Rationale

Potentially impacts all NRM modifying WIDs, SIDs. Specifically related to "WID on enhance SBMA specifications", "WID on SBMA enabler enhancements".

TS 32.156 needs clarification about attribute type and attribute properties for structured datatypes.

For attributes with structured datatypes attribute properties can be defined on multiple leves. Once for the attribute itself but also for the individual attribute fields (subparts). There was confusion about how attribute properties declared on different levels affect each other. E.g., if the whole attribute is declared readOnly, but a subpart is declared readWrite, is it possible to modify that subpart?

It was not even described that attributes may have both a simple and a structured datatype.

# 4	Detailed proposal

It is proposed to enhance 32.156 by describing

- attributes may have both a simple and a structured datatypes

- how attribute properties declared on different levels interact

- defining terminology for attributes, subparts and individual values as copied over from 28.532 clause 11.1.1.11.1

Note: The meaning of the properties isNullable and passedById is not well defined, so they are not considered in this document.