3GPP TSG-SA5 Meeting #146-bis-e 	S5-231107rev1

emeeting 16 -19 January 2023

Source:	China Unicom, Huawei, CATT, China Telecom, ZTE, CMCC, AsiaInfo

Title:	New Rel-18 WID on Management Aspects of URLLC

Document for:	Approval

Agenda Item:	6.2.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Management Aspects of URLLC

## Acronym: URLLC_Mgt

## Unique identifier:

Potential target Release: Rel-18.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No | X | X |  |  | X |
| Don’t know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |
| 950028 | Study on Management Aspects of URLLC |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940055 | Study on 5G Timing Resiliency and TSC&URLLC enhancements | SA2 study item |
| 970024 | Stage 2 of Timing Resiliency and URLLC enhancements | SA2 work item |

Dependency on non-3GPP (draft) specification: none.

## 3	Justification

5G system is able to provide data transfer capability with strict performance requirements, e.g. very low latency and high reliability requirements, to fulfil the demands of vertical industry. The performance requirements of URLLC are specified in TS 22.261, which is different from eMBB.

Existing 3GPP specifications dealing with management of URLLC in RAN network have some limitations which are proposed in the FS_URLLC_mgt study item. Several issues have been identified in the study especially on the aspects of configuration management and performance evaluation/assurance in RAN. Once the corresponding use case and solutions proposed is accepted, it will be recommended to normative work in the WI. This may include: defining NG-RAN KPIs or measurements for assurance of URLLC typical performance, defining NG-RAN KPI or measurements for evaluation of resource load in coexisting scenario, describing enhancements on configuration management of reliability when the requirements for uplink and downlink are asymmetry, describing enhancements on management of URLLC-related parameters configuration in RAN, etc.

Besides, the 5G end-to-end Qos monitor has configuration requirements for 5G NF. E.g., UPF ; When the PDU sessions of UE are established for various URLLC, the N3 underlying link between RAN and UPF corresponding to them should be different, which means the interface may need to be enhanced.

Other WGs, such as SA2 and RAN2, have specified features/enhancements for URLLC, some of which may need the support of SA5. The ongoing WI, Timing Resiliency and URLLC enhancements, is conducted by SA2 and part of the objectives are about the enhancements for URLLC. It is mentioned in the WID that there are potential impacts to be covered by SA5.

The target of this WI is to specify the management enhancement related to URLLC.

## 4	Objective

The objective of this work item is listed as follows:

To enhance the management of URLLC-related NG RAN by specifying the following aspects:

- Specify requirements and solutions for enhancement to 3GPP NRM supporting:

  - Configuration management of reliability in RAN using network slice to implement the service when the requirements for UL and DL are asymmetric

  - Configuration management of URLLC-related parameters in service or slice profiles

- Specify requirements and solutions for enhancement to performance management:

  - Defining NG-RAN KPIs or measurements for assurance of URLLC typical performance

  - Defining NG-RAN KPIs or measurements for evaluation of resource load in coexisting scenario

  - Specifying the appropriate granular of existing performance measurements or KPIs used for URLLC

- Specify other potential enhancements on management aspects of URLLC led by other WGs

To enhance the management of URLLC-related by specifying other aspects:

- Specify the interfaces for enhancement to support various URLLC requirements

- Specify End-to-end QoS monitor configuration requirements for 5G NF

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 28.541 | Add new and/or enhance existing IOCs/attributes for URLLC | Dec 2023(SA#102) |  |
| TS 28.552 | Add new and/or enhance existing measurements for URLLC | Dec 2023(SA#102) |  |
| TS 28.554 | Add new and/or enhance existing KPIs for URLLC | Dec 2023(SA#102) |  |
| TS 28.533 | Add new and/or enhance interfaces | Dec 2023(SA#102) |  |

## 6	Work item Rapporteur(s)

Wang Zhaoning, China Unicom, wangzn18@chinaunicom.cn

## 7	Work item leadership

SA WG5

## 8	Aspects that involve other WGs

Co-ordination with SA2, RAN2 if needed.

## 9	Supporting Individual Members

Supporting IM name

China Unicom

Huawei

CATT

China Telecom

ZTE

CMCC

AsiaInfo