3GPP TSG-SA5 Meeting #148e 	S5-233280

Electronic meeting, Online, 17 -25 April 2023

- Source:	Huawei, Deutsche Telekom, China Mobile,Ericsson

- Title:	Add solution for modelling ServiceProfile attributes as intent expectation for network slice service delivering and assurance

- Document for:	Approval

Agenda Item:	6.7.4.3

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP TR 28.836: "3GPP TR 28.836 Study on intent-driven management for network slicing".

[2]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

# 3	Rationale

This contribution proposes to add intent driven approach for network slice delivering and assurance to support following two use cases in TR 28.836.

-	Use case of Intent driven management to deliver a network slice instance

-	Use case of Intent driven management to express expectation of network slice service assurance

# 4	Detailed proposal

1st Change

### 6.2.2	Modelling service profile attributes as intent expectation for network slice

#### 6.2.2.1	ExpectationObject

Following are the specific allowed values when implemented the IntentExpectation for Network Slice Expectation.

Table 6.2.2.1.2.1-1

| Attribute | Allowed Values |
| --- | --- |
| ObjectType (CM) | NetworkSlice |
| objectInstance (CM) | DN of the NetworkSlice |

Following attributes in ServiceProfile can be modelled as ObjectContexts, these attributes represent the functional requirements for the network slice.

| Attributes in ServiceProfile | Object Context |
| --- | --- |
| coverageArea | contextAttribute: "coverageArea" contextCondition: "IS_WITHIN_RANGE" contextValueRange: coverageArea defined in clause 6.4.1 of 3GPP TS 28.541 [2]. |
| radioSpectrum | contextAttribute: "radioSpectrum" contextCondition: "IS_WITHIN_RANGE" contextValueRange: radioSpectrum defined in clause 6.3.31 of 3GPP TS 28.541 [2]. |

#### 6.2.2.2	ExpectationTargets

Following attributes in ServiceProfile can be modelled as ExpectationTargets, these attributes represent the capacity and performance requirements for the network slice.

| Attributes in ServiceProfile | Expectation Target |
| --- | --- |
| dLLatency | targetName: "dLLatency" targetCondition: "IS_LESS_THAN " targetValueRange: integer |
| uLLatency | targetName: "uLLatency" targetCondition: "IS_LESS_THAN" targetValueRange: integer |
| dLThptPerUE | targetName: "dLThptPerUE" targetCondition: "IS_WITHIN_RANGE" targetValueRange: A pair of integers to represent the guaranteed data rate and the maximum data rate. |
| uLThptPerUE | targetName: "uLThptPerUE" targetCondition: "IS_WITHIN_RANGE" targetValueRange: A pair of integers to represent the guaranteed data rate and the maximum data rate. |

Editor’s Note: Whether any of the remaining attributes (e.g. maxNumberofUEs, activityFactor, maxNumberofPDUSessions) in ServiceProfile can be modelled as IntentExpectation is FFS

#### 6.2.2.3	ExpectationContext

Following attributes in ServiceProfile can be modelled as ExpectationContext, these attributes used to represent additional requirements for network slice (e.g. operational requirements)

| Attributes in ServiceProfile | ExpectationContext |
| --- | --- |

Editor’s Note: Whether to reuse the Service Support Expectation in TS 28.312 or define a new Expectation for network slice service delivering and assurance is FFS.

End of changes