3GPP TSG-SA5 Meeting #149	S5-234147

# Berlin, Germany,  22 - 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0424 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to the NWDAF service provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MANWDAF |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently PM cannot support the measurements related to NWDAF service provisioning. The TR 28.864 proposed performance measurements of the NWDAF on providing different type of services, including the analytics service, ML model provisioning service and data management service, etc. It is necessary to add the related measurements in TS 28.552 based on the recommendations of TR 28.864. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added measurements related to NWDAF service provisioning based on the recommendations in TR 28.864. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of NWDAF on service provisioning cannot be evaluated efficiently. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

3.3	Measurement family

The measurement names defined in the present document are all beginning with a prefix containing the measurement family name. This family name identifies all measurements which relate to a given functionality and it may be used for measurement administration.

The list of families currently used in the present document is as follows:

-	DRB (measurements related to Data Radio Bearer).

-	RRC (measurements related to Radio Resource Control).

-	UECNTX (measurements related to UE Context).

-	RRU (measurements related to Radio Resource Utilization).

-	RM (measurements related to Registration Management).

-	SM (measurements related to Session Management).

-	GTP (measurements related to GTP Management).

-	IP (measurements related to IP Management).

-	PA (measurements related to Policy Association).

-	MM (measurements related to Mobility Management).

-	VR (measurements related to Virtualized Resource).

-	CARR (measurements related to Carrier).

-	QF (measurements related to QoS Flow).

-	AT (measurements related to Application Triggering).

-	SMS (measurements related to Short Message Service).

-	PEE (measurements related to Power, Energy and Environment).

-	NFS (measurements related to NF service).

-	PFD (measurements related to Packet Flow Description).

-	RACH (measurements related to Random Access Channel).

-	MR (measurements related to Measurement Report).

-	L1M (measurements related to Layer 1 Measurement).

-	NSS (measurements related to Network Slice Selection).

-	PAG (measurements related to Paging).

-	NIDD (measurements related to Non-IP Data Delivery).

-	EPP (measurements related to external parameter provisioning).

-	TI (measurements related to traffic influence).

-	CE (measurements related to Connection Establishment).

-	SPP (measurements related to Service Parameter Provisioning).

-	BDTP (measurements related to Background Data Transfer Policy).

-	DM (measurements related to Data Management).

-	AFQ (measurements related to AF session with QoS).

-	UCM (measurements related to UE radio Capability Management).

-	PAU (measurements related to Policy Authorization).

-	EEX (measurements related to Event Exposure).

-	SDM (measurements related to subscriber data management).

-	PPV (measurements related to parameter provisioning).

-	DIS (measurements related to discovery).

-	Location Management (measurements related to Location Management).

-	SP (measurement related to service provisioning).

-	DAS (measurement related to Data Analtics Service).

1st Modified Section

5.X	Performance measurements for NWDAF

5.X.C	Measurements related to the NWDAF service provisioning

5.X.C.1	NWDAF service provisioning subscription

5.X.C.1.1	Number of service subscription requests

a)	This measurement provides the number of service subscription requests received by the NWDAF.

b)	CC

c)	On receipt by the NWDAF of the subscription request to NWDAF service (See clause 7 in TS 23.288 [2]), each received subscription request is added to the relevant counter. The measurement can be split into subcounters per service type, subcounters per Analytics ID and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DAS.SerSubReqReceived, and

DAS.SerSubReqReceived.ServiceType, where ServiceType identifies the target type of service, and

DAS.SerSubReqReceived.AnalyticsID, where AnalyticsID identifies the target type of analytics, and

DAS.SerSubReqReceived.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for performance assurance.

5.X.C.1.2	Number of successful service subscription requests

a)	This measurement provides the number of service subscription requests received and accepted by the NWDAF.

b)	CC

c)	On receipt by the NWDAF of the subscription request to NWDAF service (See clause 7 in TS 23.288 [2]), each accepted subscription request is added to the relevant counter. The measurement can be split into subcounters per service type, subcounters per Analytics ID and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DAS.SerSubReqAccepted, and

DAS.SerSubReqAccepted.ServiceType, where ServiceType identifies the target type of service, and

DAS.SerSubReqAccepted.AnalyticsID, where AnalyticsID identifies the target type of analytics, and

DAS.SerSubReqAccepted.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for performance assurance.

5.X.C.1.3	Number of failed service subscription requests

a)	This measurement provides the number of service subscription requests received but rejected by the NWDAF.

b)	CC

c)	On receipt by the NWDAF of the subscription request to NWDAF service (See clause 7 in TS 23.288 [2]), each rejected subscription request is added to the relevant counter. The measurement can be split into subcounters per service type, subcounters per Analytics ID and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DAS.SerSubReqRejected, and

DAS.SerSubReqRejected.ServiceType where ServiceType identifies the target type of service, and

DAS.SerSubReqRejected.AnalyticsID, where AnalyticsID identifies the target type of analytics, and

DAS.SerSubReqRejected.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for performance assurance.

5.X.C.2	NWDAF service provisioning request

5.X.C.2.1	Number of service requests

a)	This measurement provides the number of service requests received by the NWDAF.

b)	CC

c)	On receipt by the NWDAF of the request to NWDAF service (See clause 7 in TS 23.288 [2]), each received request is added to the relevant counter. The measurement can be split into subcounters per service type, subcounters per Analytics ID and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DAS.SerReqReceived, and

DAS.SerReqReceived.ServiceType, where ServiceType identifies the target type of service, and

DAS.SerReqReceived.AnalyticsID, where AnalyticsID identifies the target type of analytics, and

DAS.SerReqReceived.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for performance assurance.

5.X.C.2.2	Number of successful service requests

a)	This measurement provides the number of service requests received and accepted by the NWDAF.

b)	CC

c)	On receipt by the NWDAF of the request to NWDAF service (See clause 7 in TS 23.288 [2]), each accepted request is added to the relevant counter. The measurement can be split into subcounters per service type, subcounters per Analytics ID and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DAS.SerReqAccepted, and

DAS.SerReqAccepted.ServiceType, where ServiceType identifies the target type of service, and

DAS.SerReqAccepted.AnalyticsID, where AnalyticsID identifies the target type of analytics, and

DAS.SerReqAccepted.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for performance assurance.

5.X.C.2.3	Number of failed service requests

a)	This measurement provides the number of service provisioning subscription requests received but rejected by the NWDAF.

b)	CC

c)	On receipt by the NWDAF of the request to NWDAF service (See clause 7 in TS 23.288 [2]), each rejected request is added to the relevant counter. The measurement can be split into subcounters per specific service type, subcounters per Analytics ID and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DAS.SerReqRejected, and

DAS.SerReqRejected.ServiceType, where ServiceType identifies the target type of service, and

DAS.SerReqRejected.AnalyticsID, where AnalyticsID identifies the target type of analytics, and

DAS.SerReqRejected.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for performance assurance.

5.X.C.3	Successful NWDAF service provisioning

5.X.C.3.1	Number of service notifications generated by the NWDAF

a)	This measurement provides the number of service subscription requests notified by the NWDAF.

b)	CC

c)	On transmission by the NWDAF of the notification corresponding to the service subscription request (See clause 7 in TS 23.288 [2]), each transmitted notification is added to the relevant counter. The measurement can be split into subcounters per specific service type, subcounters per Analytics ID and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DAS.SerNotificationGenerated, and

DAS.SerNotificationGenerated.ServiceType, where ServiceType identifies the target type of service, and

DAS.SerNotificationGenerated.AnalyticsID, where AnalyticsID identifies the target type of analytics, and

DAS.SerNotificationGenerated.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for performance assurance.

5.X.C.3.2	Number of service responses generated by the NWDAF

a)	This measurement provides the number of service requests responsed by the NWDAF.

b)	CC

c)	On transmission or generation by the NWDAF of the response corresponding to the service request (See clause 7 in TS 23.288 [2]), each transmitted or generated response is added to the relevant counter. The measurement can be split into subcounters per specific service type, subcounters per Analytics ID and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DAS.SerResponseGenerated, and

DAS.SerResponseGenerated.ServiceType, where ServiceType identifies the target type of service, and

DAS.SerResponseGenerated.AnalyticsID, where AnalyticsID identifies the target type of analytics, and

DAS.SerResponseGenerated.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for performance assurance.

End of modification