3GPP TSG-SA5 Meeting #151	S5-236428

# Xiamen, China, 9-13 October 2023

# Source:	Huawei, CMCC

# Title:	pCR 28.830 Add solution of fault impacts analysis

# Document for:	Approval

Agenda Item:	6.7.4.2

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-220153: "New SID on Fault Supervision Evolution"

[2]	TR 28.830 "Fault supervision evolution"

3	Rationale

This tdoc discusses the solution relate to fault impact analysis in FSEV study [1, 2].

WoP#2 of the FS_FSEV study item is as follows: “2. How fault supervision evolution supports 5G use cases, such as 5G SLS deterioration, risk prediction”. The issue of fault impact analysis is related to this WoP.

In TR 28.830 [2], some descriptions in 5.X.1 are more like solutions and it is proposed to move them to the solution part.

4	Detailed proposal

1st Change

5.X	Key Issue #X: Potential enhancements for fault related analysis

5.X.1	Description

Editor’s note: This clause provides a description of the key issue.

Editor’s note: This clause provides a description of the key issue.

If a potential fault/failure is predicted and reported to the consumer, the consumer would like to know the consequence. More analysis information on 3GPP system is helpful for the consumer to perform more proper actions, e.g., performance degradation analysis and predictions, KPI anomaly analysis and predictions, etc.

For example, the impacts on 3GPP system may not be perceived significantly in densely populated urban areas if there are overlapping coverage when a few sites encounter faults. However, service outage may occur due to faults in a site if there are few overlapping coverage in suburban areas. If this kind of information can be provided, the different handlings may be performed by the consumer.

5.X.2	Potential solutions

5.X.2.a	Potential solution #3: Fault impact analysis

5.X.2.a.1	Introduction

The output information of fault impact analysis are provided.

5.X.2.a.2	Description

Based on the topology relationship of the concerned managed objects, the scope and degree of fault impacts are evaluated and provided. It may contain the following aspects:

-	scope and severity level of the impacted managed objects: the service types and coverage areas which are affected by the fault, e.g., the VoNR, URLLC service types, number of affected PDU sessions, and the coverage areas etc;

-	impacted managed objects: the managed objects which are affected by the fault, e.g. network slice, network slice subnet, network elements, network functions, number of affected gNBs and cells etc;

-	Rootcause types, name and descriptions of different types of rootcauses, etc.

-	Recommended actions for recovery;

5.X.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work.

End of change