3GPP TSG-SA5 Meeting #152 	S5-237711

Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0191 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct issues for End-to-end Network Resource Expectation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use of term “End-to-end Network Resource Expectation” cannot clearly show the meaning of this expectation. The referred clause for ObjectContexts of core network is missing. Duplicate clause number of 6.2.2.1.3. Some typos exist in the description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the name of expectation into “End-to-end Network Optimization Expectation”. Add the reference of clause for 5GC in 6.2.2.1.4.2. Correct the clause number. Fix some typos. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Several issues exist in the published TS 28.312. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1.3, 6.2.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.2.2.1.3	End-to-end Network Optimization Expectation

6.2.2.1.3.1	Definition

End-to-end Network Optimization Expectation is an IntentExpectation which can be used to represent MnS consumer's expectations for the optimization of network resources across multiple network domains, i.e., MnS consumer expresses its intent containing an intent expectation with targets on the whole network comprising RAN and Core Network.

The End-to-end Network Optimization Expectation is defined utilizing the constructs of the generic IntentExpectation <<dataType>> with set of allowed values and concrete dataTypes specified. It enables optimization among desired targets for end-to-end network optimization based on a list of expectation targets that are provided in an ordered list where those at the top of the list are the most important.

Following are the specific allowed values when implemented the IntentExpectation for End-to-end Network Optimization Expectation.

Table 6.2.2.1.3.1-1

| Attribute | Allowed Values |
| --- | --- |
| objectType (CM) | Subnetwork |
| objectInstance (CM) | DN of the Subnetwork |

NOTE:	Following are the qualifier description for attribute "objectType" and "objectInstance":

-	In case of the intent expectation is not for a specific subnetwork instance or/and MnS consumer have no knowledge of the DN of this subnetwork instance, the attribute "objectType" needs to be specified with the value as "subnetwork".

-	In case of the intent expectation is for a specific Subnetwork instance and MnS consumer have the knowledge of the DN of this Subnetwork instance, the attribute "objectInstance" needs to be specified.

6.2.2.1.3.2	ObjectContexts

Based on the common structure of ObjectContext, the concrete ObjectContexts for end-to-end Network Optimization Expectation is the combination of ObjectContext defined for the RAN in clause 6.2.2.1.1.2 and for the 5GC in clause 6.2.2.1.4.2.

6.2.2.1.3.3	ExpectationTargets

Following provides the concrete ExpectationTargets for end-to-end Network Optimization Expectation based on the common structure of ExpectationTarget. The attribute properties defined for these ExpectationTargets should be same with the properties defined for ExpectationTargets in clause 6.2.1.3.

Editor's Note:	Which KPIs defined in TS 28.554 can be used is FFS.

2nd change

6.2.2.1.4	5GC Network Expectation

6.2.2.1.4.1	Definition

5GC Network Expectation is an IntentExpectation which can be used to represent MnS consumer's expectations for 5GC network (5GC SubNetwork) delivering

The 5GC Network Expectation is defined by utilizing the construct of the generic IntentExpectation <<dataType>> with set of allowed values and concrete dataTypes specified.

Following are the specific allowed values when implemented the IntentExpectation for 5GC Network Expectation.

Table 6.2.2.1.3.1-1

| Attribute Name | Allowed Values |
| --- | --- |
| objectType (CM) | 5GC SubNetwork |
| objectInstance (CM) | DN of the 5GC SubNetwork |

NOTE:	Following are the qualifier description for attribute "objectType" and "objectInstance":

-	In case of the intent expectation is not for a specific 5GC SubNetwork instance or/and MnS consumer have no knowledge of the DN of this 5GC SubNetwork instance, the attribute "objectType" needs to be specified.

-	In case of the intent expectation is for a specific 5GC SubNetwork instance and MnS consumer have the knowledge of the DN of this 5GC SubNetwork instance, the attribute "objectInstance" needs to specified.

6.2.2.1.4.2	ObjectContexts

Following provides the concrete ObjectContexts for 5GC Network Expectation based on the common structure of ObjectContext. The properties of the attributes in the following table should be same with properties of ObjectContexts defined in clause 6.2.1.3.

Table 6.2.2.1.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nfTypeContext | O | T | T | F | F |
| nfInstanceLocationContext | O | T | T | F | F |
| pLMNContext | O | T | T | F | F |
| taiContext | O | T | T | F | F |

6.2.2.1.4.3	ExpectationTargets

Following provides the concrete ExpectationTargets for 5GC Network Expectation based on the common structure of ExpectationTarget. The properties of the attributes in the following table should be same with properties of ExpectationTargets defined in clause 6.2.1.3.

Table 6.2.2.1.3.3-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| maxNumberofPDUsessionsTarget | O | T | T | F | F |
| maxNumberofRegisteredsubscribersTarget | O | T | T | F | F |

Editor's note: other ExpectationTargets of 5GC network is FFS.

End of change