# 3GPP TSG SA WG5 Meeting #152                 	           S5-237866

# Chicago, US, 13-17 November 2023

# Source:	Huawei

# Title:	Define the structure of TSN Charging Information

# Document for:	Approval

Agenda Item:	7.4.6

1	Decision/action requested

Agree the structure of TSN Charging Information in TS 32.282.

2	References

[1] 		3GPP TS 32.282: "Charging management; Time-Sensitive Networking (TSN) charging".

3	Rationale

Define the TSN charging specific parameters, i.e. the TSN charging information.

4	Detailed proposal

Propose to incorporate the following change into the TS 32.282 [1].

First change

6.2	Time sensitive networking charging specific parameters

6.2.1	Definition of time sensitive networking charging information

6.2.1.1	General

The Charging Information parameter used for time sensitive networking charging is provided in the following clauses.

6.2.1.2	Definition of TSN Charging Information

Specific charging information for TSN charging is provided within the TSN Charging Information.

The detailed structure of the TSN Charging Information can be found in table 6.2.1.2.1.

Table 6.2.1.2-1: Structure of TSN Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Service name | OC | This parameter holds the name of the service invoked. For 5GS bridge configuration and management charging, the service name is the “5GS bridge configuration and reporting”. For TSC and TS service enabler charging, the service name is the NF services provided by TSCTSF defined in Table 5.2.2.3.1-1 TS 23.502 [12]. |
| Service operation | OC | This parameter holds the service operation name, defined in Table 5.2.2.3.1-1 TS 23.502 [12], applicable for request from AF to TSCTSF, or from TSCTSF to AF. |
| DNN | OC | This field holds data network name the AF belongs to. |
| S-NSSAI | OC | This field holds network slice information the AF belongs to. |
| Target UE Identifiers | OC | This field holds one or a list of individual UE(s), or a group of UE represented by Internal Group Identifier(s) that the AF request is targeted for, defined in TS 23.501 [11]. |
| 5GS Bridge Information | OC | This parameter holds the information for TSN 5GS bridge, defined in clause 6.2.1.3. |
| QoS and TSC Assistance Information | OC | This parameter holds the information for TSN QoS and TSC Assistance, defined in clause 6.2.1.4. |
| Time Synchronization Information | OC | This parameter holds the information for Time Synchronization, defined in clause 6.2.1.5. |
| Time Synchronization Status Information | OC | This parameter holds the information for Time Synchronization Status, defined in clause 6.2.1.6. |

End of change